Skill 10 - Lab 0 - Configuration Static NAT

CCNA beginner lab: static one-to-one NAT with a simulated ISP and outside global targets

Figure 1. Network topology for Cafe LAN, Static NAT, and simulated ISP outside global targets.

# Field Briefing

Coffee House Beta has two inside LAN hosts that use private IPv4 addresses. The cafe router, Cafe-RT, must translate each inside private address to one dedicated public address using static NAT. The ISP router is simulated with two loopback addresses, 1.1.1.1 and 8.8.8.8, which act as outside global Internet targets. Students will configure the switch, both routers, static NAT, routing, and verification commands from beginning to end.

Beginner note: In NAT vocabulary, 1.1.1.1 and 8.8.8.8 are outside global addresses. The public NAT addresses 216.0.5.20 and 216.0.5.21 are inside global addresses because they represent the inside PCs to the outside network.

# Lab Objectives

Build and label the Packet Tracer topology.

Configure the Cafe-LAN-SW switch management IP and access ports.

Configure Cafe-RT LAN and WAN interfaces.

Configure ISP-RT as a simulated ISP with loopbacks 1.1.1.1 and 8.8.8.8.

Configure one-to-one static NAT for two inside hosts.

Configure routing so return traffic reaches the NAT inside global addresses.

Verify NAT translations using ping and show commands.

Explain inside local, inside global, outside local, and outside global addresses.

# Access Credentials

| Device access | Value |
| --- | --- |
| Console / VTY username | cisco |
| Console / VTY password | cisco |
| Privileged EXEC secret | CrC0ffee! |
| Recommended save command | copy running-config startup-config |

# Task 0 - Create the Network Topology

Objective: Add devices, connect cables, and rename each device before configuration.

| Device | Suggested Packet Tracer device | Hostname / label | Purpose |
| --- | --- | --- | --- |
| PC | PC-PT | PC2 | Inside LAN host 1 |
| PC | PC-PT | PC1 | Inside LAN host 2 |
| Switch | 2960 switch | Cafe-LAN-SW | Layer 2 LAN switch |
| Router | 1941 / 2911 / ISR router | Cafe-RT | Cafe default gateway and NAT router |
| Router | 1941 / 2911 / ISR router | ISP-RT | Simulated ISP router with outside global loopbacks |

| Cable | From | To | Interface notes |
| --- | --- | --- | --- |
| Copper straight-through | PC2 Fa0 | Cafe-LAN-SW Fa0/3 | PC2 inside host |
| Copper straight-through | PC1 Fa0 | Cafe-LAN-SW Fa0/2 | PC1 inside host |
| Copper straight-through | Cafe-LAN-SW Fa0/1 | Cafe-RT G0/0 | LAN gateway link |
| Copper crossover or auto cable | Cafe-RT G0/1 | ISP-RT G0/0 | WAN / ISP link |

Beginner note: If your router shows Ethernet0/0 instead of GigabitEthernet0/0, use the Ethernet interface names shown by show ip interface brief. The IP addresses and NAT logic stay the same.

# Addressing and NAT Plan

| Device | Interface | IPv4 address | Mask | Purpose |
| --- | --- | --- | --- | --- |
| PC2 | Fa0 | 192.168.1.50 | 255.255.255.0 | Inside host 1 |
| PC1 | Fa0 | 192.168.1.51 | 255.255.255.0 | Inside host 2 |
| Cafe-LAN-SW | VLAN 1 | 192.168.1.2 | 255.255.255.0 | Switch management IP |
| Cafe-RT | G0/0 | 192.168.1.1 | 255.255.255.0 | Default gateway / NAT inside |
| Cafe-RT | G0/1 | 216.0.5.2 | 255.255.255.252 | WAN / NAT outside |
| ISP-RT | G0/0 | 216.0.5.1 | 255.255.255.252 | ISP gateway |
| ISP-RT | Loopback0 | 1.1.1.1 | 255.255.255.255 | Simulated Internet outside global target |
| ISP-RT | Loopback1 | 8.8.8.8 | 255.255.255.255 | Simulated Internet outside global target |

| Inside host | Inside local address | Inside global address | Explanation |
| --- | --- | --- | --- |
| PC2 | 192.168.1.50 | 216.0.5.20 | PC2 is represented to the outside as 216.0.5.20. |
| PC1 | 192.168.1.51 | 216.0.5.21 | PC1 is represented to the outside as 216.0.5.21. |

Beginner note: Do not assign 216.0.5.20 or 216.0.5.21 directly to a physical interface. They are static NAT inside global addresses. ISP-RT must have routes that point these public host addresses back to Cafe-RT at 216.0.5.2.

# Static NAT Concepts Used in This Lab

| NAT term | Meaning in this lab | Example |
| --- | --- | --- |
| Inside local | The real private IP address on the cafe LAN. | PC2 = 192.168.1.50; PC1 = 192.168.1.51 |
| Inside global | The public IP address that represents an inside host to the outside network. | 216.0.5.20 and 216.0.5.21 |
| Outside global | The real outside address of the simulated Internet target. | 1.1.1.1 and 8.8.8.8 |
| Outside local | How the outside host appears to the inside network. In this simple lab, it stays the same as outside global. | 1.1.1.1 and 8.8.8.8 |
| ip nat inside | Marks the LAN-facing router interface as the NAT inside zone. | Cafe-RT G0/0 |
| ip nat outside | Marks the ISP-facing router interface as the NAT outside zone. | Cafe-RT G0/1 |

# Task 1 - Configure PC IP Addresses

Objective: Give each PC a private IP address and the Cafe-RT LAN interface as the default gateway.

| PC | IP address | Subnet mask | Default gateway |
| --- | --- | --- | --- |
| PC2 | 192.168.1.50 | 255.255.255.0 | 192.168.1.1 |
| PC1 | 192.168.1.51 | 255.255.255.0 | 192.168.1.1 |

Packet Tracer steps: Desktop tab > IP Configuration > Static > enter the values above.

Optional PC command prompt checks

ipconfig

ping 192.168.1.1

# Task 2 - Configure Cafe-LAN-SW

Objective: Configure a basic Layer 2 switch setup, management SVI, default gateway, and access ports for the cafe LAN.

Cafe-LAN-SW beginner configuration

enable

configure terminal

hostname Cafe-LAN-SW

no ip domain-lookup

service password-encryption

username cisco

enable secret CrC0ffee!

!

line console 0

password cisco

login

logging synchronous

exit

line vty 0 4

password cisco

login

exit

!

interface vlan 1

description Management SVI for Cafe LAN

ip address 192.168.1.2 255.255.255.0

no shutdown

exit

ip default-gateway 192.168.1.1

!

interface fastethernet0/1

description To Cafe-RT G0/0

switchport mode access

switchport access vlan 1

no shutdown

exit

interface fastethernet0/2

description To PC1 Fa0

switchport mode access

switchport access vlan 1

spanning-tree portfast

no shutdown

exit

interface fastethernet0/3

description To PC2 Fa0

switchport mode access

switchport access vlan 1

spanning-tree portfast

no shutdown

exit

end

copy running-config startup-config

Cafe-LAN-SW verification

show ip interface brief

show interfaces status

show running-config interface vlan 1

show running-config | include default-gateway

ping 192.168.1.1

Beginner note: The switch management IP is only for managing the switch. PCs do not use 192.168.1.2 as their gateway. PCs use Cafe-RT at 192.168.1.1 as the default gateway.

# Task 3 - Configure Cafe-RT Interfaces and Default Route

Objective: Configure the LAN gateway, WAN link, and default route toward ISP-RT.

Cafe-RT base routing configuration

enable

configure terminal

hostname Cafe-RT

no ip domain-lookup

service password-encryption

enable secret CrC0ffee!

!

line console 0

password cisco

login

logging synchronous

exit

line vty 0 4

password cisco

login

exit

!

interface gigabitethernet0/0

description To Cafe-LAN-SW Fa0/1 - Cafe LAN

ip address 192.168.1.1 255.255.255.0

no shutdown

exit

interface gigabitethernet0/1

description To ISP-RT G0/0 - WAN outside

ip address 216.0.5.2 255.255.255.252

no shutdown

exit

!

ip route 0.0.0.0 0.0.0.0 216.0.5.1

end

copy running-config startup-config

Cafe-RT baseline verification before NAT

show ip interface brief

show ip route

ping 192.168.1.2

ping 192.168.1.50

ping 192.168.1.51

ping 216.0.5.1

# Task 4 - Configure Static NAT on Cafe-RT

Objective: Mark the NAT inside and outside interfaces, then map each inside private host to a dedicated public inside global address.

Cafe-RT static NAT configuration

enable

configure terminal

interface gigabitethernet0/0

ip nat inside

exit

interface gigabitethernet0/1

ip nat outside

exit

!

ip nat inside source static 192.168.1.50 216.0.5.20

ip nat inside source static 192.168.1.51 216.0.5.21

end

copy running-config startup-config

Cafe-RT NAT verification commands

show running-config | include ip nat

show running-config interface gigabitethernet0/0

show running-config interface gigabitethernet0/1

show ip nat translations

show ip nat statistics

# Task 5 - Configure ISP-RT as the Simulated ISP

Objective: Configure the ISP-facing WAN interface, create two loopbacks as outside global targets, and add return routes for the static NAT public addresses.

ISP-RT simulated ISP configuration

enable

configure terminal

hostname ISP-RT

no ip domain-lookup

service password-encryption

enable secret CrC0ffee!

!

line console 0

password cisco

login

logging synchronous

exit

line vty 0 4

password cisco

login

exit

!

interface gigabitethernet0/0

description To Cafe-RT G0/1

ip address 216.0.5.1 255.255.255.252

no shutdown

exit

!

interface loopback0

description Simulated Internet target - outside global 1

ip address 1.1.1.1 255.255.255.255

exit

interface loopback1

description Simulated Internet target - outside global 2

ip address 8.8.8.8 255.255.255.255

exit

!

ip route 216.0.5.20 255.255.255.255 216.0.5.2

ip route 216.0.5.21 255.255.255.255 216.0.5.2

end

copy running-config startup-config

Beginner note: The host routes on ISP-RT are required because 216.0.5.20 and 216.0.5.21 are not part of the 216.0.5.0/30 WAN link. ISP-RT must know to send those addresses to Cafe-RT at 216.0.5.2.

ISP-RT verification

show ip interface brief

show ip route

ping 216.0.5.2

ping 1.1.1.1

ping 8.8.8.8

show ip route 216.0.5.20

show ip route 216.0.5.21

# Interface Name Adjustment

Some Packet Tracer routers or lab images use Ethernet0/0 and Ethernet0/1 instead of GigabitEthernet0/0 and GigabitEthernet0/1. Use the same IP addresses, but replace the interface names as needed.

If Cafe-RT uses Ethernet interfaces

interface ethernet0/0

description To Cafe-LAN-SW Fa0/1 - Cafe LAN

ip address 192.168.1.1 255.255.255.0

ip nat inside

no shutdown

exit

interface ethernet0/1

description To ISP-RT E0/0 - WAN outside

ip address 216.0.5.2 255.255.255.252

ip nat outside

no shutdown

exit

If ISP-RT uses Ethernet interfaces

interface ethernet0/0

description To Cafe-RT E0/1

ip address 216.0.5.1 255.255.255.252

no shutdown

exit

# Task 6 - Test Static NAT from the Inside LAN

Objective: Generate inside-to-outside traffic from both PCs so the NAT table can be verified.

From PC2 command prompt

ipconfig

ping 192.168.1.1

ping 216.0.5.1

ping 1.1.1.1

ping 8.8.8.8

From PC1 command prompt

ipconfig

ping 192.168.1.1

ping 216.0.5.1

ping 1.1.1.1

ping 8.8.8.8

On Cafe-RT after the pings

show ip nat translations

show ip nat statistics

show running-config | include ip nat

| Expected item | What you should see |
| --- | --- |
| Static NAT entry for PC2 | Inside global 216.0.5.20 mapped to inside local 192.168.1.50 |
| Static NAT entry for PC1 | Inside global 216.0.5.21 mapped to inside local 192.168.1.51 |
| Outside global target 1 | 1.1.1.1 should reply to PC pings |
| Outside global target 2 | 8.8.8.8 should reply to PC pings |

Sample show ip nat translations output

Pro  Inside global      Inside local       Outside local      Outside global

---  216.0.5.20        192.168.1.50       ---                ---

---  216.0.5.21        192.168.1.51       ---                ---

icmp 216.0.5.20:1      192.168.1.50:1     1.1.1.1:1          1.1.1.1:1

icmp 216.0.5.21:1      192.168.1.51:1     8.8.8.8:1          8.8.8.8:1

# Task 7 - Test Static NAT from the ISP Side

Objective: Prove that the public NAT addresses route back through Cafe-RT to the private PCs.

From ISP-RT

ping 216.0.5.20

ping 216.0.5.21

traceroute 216.0.5.20

traceroute 216.0.5.21

show ip route 216.0.5.20

show ip route 216.0.5.21

Beginner note: If the first ping fails but later pings succeed, that may be normal because devices are resolving ARP. If all pings fail, use the troubleshooting checklist below.

# Task 8 - Troubleshooting Checklist

If PC2 or PC1 cannot ping 192.168.1.1, fix the local LAN, IP address, subnet mask, gateway, or switch port first.

If Cafe-RT cannot ping 216.0.5.1, fix the WAN link before troubleshooting NAT.

If Cafe-RT can ping 1.1.1.1 and 8.8.8.8 but PCs cannot, check ip nat inside and ip nat outside placement.

If ISP-RT cannot ping 216.0.5.20 or 216.0.5.21, verify the ISP host routes point to 216.0.5.2.

If show ip nat translations only shows the two static entries, generate traffic from PC2 and PC1 by pinging 1.1.1.1 and 8.8.8.8.

Do not put the public NAT addresses directly on the PCs. PCs keep the private addresses 192.168.1.50 and 192.168.1.51.

Make sure PC firewalls are not simulated or blocking ICMP. Packet Tracer PCs normally respond to ping by default.

Use show ip interface brief often. It quickly shows wrong IP addresses and down interfaces.

# Useful Verification Commands

| Command | Why you use it |
| --- | --- |
| show ip interface brief | Checks interface IP addresses and up/up status. |
| show ip route | Confirms connected routes, default routes, and ISP host routes. |
| show running-config ¦ include ip nat | Shows NAT statements quickly. |
| show running-config interface g0/0 | Verifies ip nat inside on the Cafe LAN interface. |
| show running-config interface g0/1 | Verifies ip nat outside on the Cafe WAN interface. |
| show ip nat translations | Displays static and active NAT translations. |
| show ip nat statistics | Shows inside/outside interfaces and translation counters. |
| clear ip nat translation * | Clears dynamic translations. Static entries remain configured. |
| show mac address-table | Useful on the switch to confirm PCs and router are learned on access ports. |

# Student Review Questions

Which Cafe-RT interface is NAT inside? Why?

Which Cafe-RT interface is NAT outside? Why?

What is the inside local address for PC2?

What is the inside global address for PC2?

Why are 1.1.1.1 and 8.8.8.8 called outside global addresses in this lab?

Why does ISP-RT need host routes to 216.0.5.20 and 216.0.5.21?

Why should 216.0.5.20 and 216.0.5.21 not be configured on the PCs?

# Final Completion Check

[ ] PC2 is 192.168.1.50/24 with gateway 192.168.1.1.

[ ] PC1 is 192.168.1.51/24 with gateway 192.168.1.1.

[ ] Cafe-LAN-SW has management IP 192.168.1.2/24 and default gateway 192.168.1.1.

[ ] Cafe-RT G0/0 is 192.168.1.1/24 and configured as ip nat inside.

[ ] Cafe-RT G0/1 is 216.0.5.2/30 and configured as ip nat outside.

[ ] Cafe-RT has static NAT entries for 192.168.1.50 and 192.168.1.51.

[ ] Cafe-RT has a default route to 216.0.5.1.

[ ] ISP-RT G0/0 is 216.0.5.1/30.

[ ] ISP-RT Loopback0 is 1.1.1.1/32.

[ ] ISP-RT Loopback1 is 8.8.8.8/32.

[ ] ISP-RT has return routes to 216.0.5.20 and 216.0.5.21 via 216.0.5.2.

[ ] PC2 and PC1 can ping 1.1.1.1 and 8.8.8.8.

[ ] show ip nat translations displays the expected static translations.

# Lab Completion Key and Full Answer Configurations

Use this section after completing the lab to check your work. Students should try the tasks first before reading the full answer configurations.

## Answer Summary

| Task | Correct result |
| --- | --- |
| Task 0 | Topology is built with PC2, PC1, Cafe-LAN-SW, Cafe-RT, and ISP-RT. |
| Task 1 | PC2 and PC1 use 192.168.1.1 as their default gateway. |
| Task 2 | Cafe-LAN-SW has VLAN 1 management IP 192.168.1.2/24 and default gateway 192.168.1.1. |
| Task 3 | Cafe-RT has LAN 192.168.1.1/24, WAN 216.0.5.2/30, and default route to 216.0.5.1. |
| Task 4 | Cafe-RT maps 192.168.1.50 to 216.0.5.20 and 192.168.1.51 to 216.0.5.21. |
| Task 5 | ISP-RT has loopbacks 1.1.1.1/32 and 8.8.8.8/32, plus routes to the NAT public addresses. |
| Task 6 | PCs can ping the ISP loopbacks and Cafe-RT shows NAT translations. |
| Task 7 | ISP-RT can ping 216.0.5.20 and 216.0.5.21. |

Complete Cafe-LAN-SW answer configuration

hostname Cafe-LAN-SW

!

no ip domain-lookup

service password-encryption

enable secret CrC0ffee!

!

line console 0

password cisco

login

logging synchronous

!

line vty 0 4

password cisco

login

!

interface vlan 1

description Management SVI for Cafe LAN

ip address 192.168.1.2 255.255.255.0

no shutdown

!

ip default-gateway 192.168.1.1

!

interface fastethernet0/1

description To Cafe-RT G0/0

switchport mode access

switchport access vlan 1

no shutdown

!

interface fastethernet0/2

description To PC1 Fa0

switchport mode access

switchport access vlan 1

spanning-tree portfast

no shutdown

!

interface fastethernet0/3

description To PC2 Fa0

switchport mode access

switchport access vlan 1

spanning-tree portfast

no shutdown

Complete Cafe-RT answer configuration

hostname Cafe-RT

!

no ip domain-lookup

service password-encryption

enable secret CrC0ffee!

!

interface gigabitethernet0/0

description To Cafe-LAN-SW Fa0/1 - Cafe LAN

ip address 192.168.1.1 255.255.255.0

ip nat inside

no shutdown

!

interface gigabitethernet0/1

description To ISP-RT G0/0 - WAN outside

ip address 216.0.5.2 255.255.255.252

ip nat outside

no shutdown

!

ip nat inside source static 192.168.1.50 216.0.5.20

ip nat inside source static 192.168.1.51 216.0.5.21

ip route 0.0.0.0 0.0.0.0 216.0.5.1

Complete ISP-RT answer configuration

hostname ISP-RT

!

no ip domain-lookup

service password-encryption

enable secret CrC0ffee!

!

interface gigabitethernet0/0

description To Cafe-RT G0/1

ip address 216.0.5.1 255.255.255.252

no shutdown

!

interface loopback0

description Simulated Internet target - outside global 1

ip address 1.1.1.1 255.255.255.255

!

interface loopback1

description Simulated Internet target - outside global 2

ip address 8.8.8.8 255.255.255.255

!

ip route 216.0.5.20 255.255.255.255 216.0.5.2

ip route 216.0.5.21 255.255.255.255 216.0.5.2

## Answer Key for Review Questions

Which Cafe-RT interface is NAT inside? G0/0 is NAT inside because it faces the private Cafe LAN, 192.168.1.0/24.

Which Cafe-RT interface is NAT outside? G0/1 is NAT outside because it faces ISP-RT and the outside network.

What is the inside local address for PC2? 192.168.1.50.

What is the inside global address for PC2? 216.0.5.20.

Why are 1.1.1.1 and 8.8.8.8 outside global addresses? They are real addresses of outside simulated Internet targets from the point of view of the NAT router.

Why does ISP-RT need host routes to 216.0.5.20 and 216.0.5.21? Those addresses are not connected to ISP-RT. The routes tell ISP-RT to send traffic for them to Cafe-RT at 216.0.5.2.

Why not configure 216.0.5.20 and 216.0.5.21 on the PCs? The PCs are inside private hosts. Static NAT on Cafe-RT represents those private hosts with public inside global addresses.