CCNA LAB MANUAL

Applying New Castle Rysen Subnet Strategy

From /26 subnet math to live addressing, DHCP, and NAT

| Student: ____________________ | Date: ____________________ | Lab Time: __________ |
| --- | --- | --- |

Figure 1. District Shop #1 target topology after applying the 10.0.18.0/26 subnet strategy.

# Mission Briefing

The subnet plan is complete, but a subnet is only useful after it is applied to live devices. In this lab, you will take the District Shop #1 network from a paper design to a functioning Packet Tracer deployment. You will calculate the 10.0.18.0/26 range, assign predictable infrastructure addresses, correct DHCP exclusions, update NAT, and prove that client devices can reach the simulated Internet.

Real-world habit: Before entering commands, redraw the network. Mark every interface, subnet, gateway, static address, DHCP range, and service. A simple hand sketch often reveals addressing mistakes before they become troubleshooting problems.

## Learning Objectives

Configure local console and VTY authentication with a username secret, enable secret, and encrypted password display.

Convert /26 prefix notation to the dotted-decimal mask 255.255.255.192.

Calculate the network address, usable host range, broadcast address, block size, and wildcard mask.

Apply a predictable infrastructure, DHCP, and reserved-static addressing policy.

Configure router and switch management addresses inside 10.0.18.0/26.

Configure DHCP so clients receive addresses from 10.0.18.11 through 10.0.18.50.

Update the NAT access list so it matches the new /26 network.

Verify DHCP bindings, NAT translations, routing, and end-to-end connectivity.

## Equipment and Starting Files

The supplied Packet Tracer file for Skill 11, Lesson 08.5.

Cafe01-RT1 and the Internet router.

Cafe01-SW01 and Cafe01-SW02 (Catalyst 2960 switches).

PCs, laptops, wireless access points, server, and optional printer shown in the topology.

The uploaded running configurations for reference and audit.

# Part 0 - Configure Access Credentials

| Access Context | Username | Password |
| --- | --- | --- |
| Router and switch console / VTY login | cisco | cisco |
| Privileged EXEC access (enable) | - | CrC0ffee! |

Login sequence: At the router console or VTY prompt, enter username cisco and password cisco. Then enter enable and use CrC0ffee! for privileged EXEC access. Credentials are case-sensitive.

Interface-name note: The supplied files show Cafe01-RT1 G0/0 as the LAN interface and G0/1 as the WAN interface. CDP shows the router LAN connected to Cafe01-SW01 Fa0/3. Confirm your actual Packet Tracer port labels before configuring.

## Step 1: Enter Privileged EXEC and Global Configuration Modes

Perform the credential configuration on Cafe01-RT1 (or Cafe01-RT01), Internet, Cafe01-SW01, and Cafe01-SW02. Replace Device with the hostname shown at the prompt.

Device> enable Device# configure terminal

## Step 2: Create the Local User and Privileged EXEC Secret

Create the local account used by console and VTY login. Configure a separate enable secret for privileged EXEC mode, then enable password obfuscation for any remaining plain-text passwords.

Device(config)# username cisco secret cisco Device(config)# enable secret CrC0ffee! Device(config)# service password-encryption

Security note: The username secret and enable secret commands store one-way hashes. The service password-encryption command only obscures other plain-text passwords in the configuration; it is not a substitute for strong secrets or SSH in a production network.

## Step 3: Require the Local Account on the Console and VTY Lines

The login local command tells IOS to authenticate with the local username database. Logging synchronous keeps system messages from interrupting commands typed at the console.

Device(config)# line console 0 Device(config-line)# login local Device(config-line)# logging synchronous Device(config-line)# exit Device(config)# line vty 0 15 Device(config-line)# login local Device(config-line)# exit

Platform note: If an IOS image rejects line vty 0 15, configure line vty 0 4 and line vty 5 15 separately, applying login local to both ranges.

## Step 4: Save and Verify the Credential Configuration

Device(config)# end Device# copy running-config startup-config Device# show running-config ¦ include username¦enable secret¦service password-encryption Device# show running-config ¦ section line con Device# show running-config ¦ section line vty

## Step 5: Test the Login Sequence

1. Exit the current session or close and reopen the device CLI.
2. At User Access Verification, enter username cisco and password cisco.
3. At the user EXEC prompt, enter enable.
4. Enter CrC0ffee! and confirm that the prompt changes from > to #.
5. Repeat the test on each router and switch. Credentials are case-sensitive.

| Device | Console / VTY Login | Enable Secret | Student Check |
| --- | --- | --- | --- |
| Cafe01-RT1 / Cafe01-RT01 | cisco / cisco | CrC0ffee! | [ ] |
| Internet | cisco / cisco | CrC0ffee! | [ ] |
| Cafe01-SW01 | cisco / cisco | CrC0ffee! | [ ] |
| Cafe01-SW02 | cisco / cisco | CrC0ffee! | [ ] |

# Common Useful Show Commands

Use these commands throughout the lab to establish a baseline, verify each configuration stage, and isolate the first point of failure. The Cafe01 router may appear as Cafe01-RT1 or Cafe01-RT01 depending on the Packet Tracer file.

## Cafe01-RT1 / Cafe01-RT01 - Interfaces and Routing

! On Cafe01-RT01 show ip interface brief show ip route show running-config ¦ include ip route show interfaces description show cdp neighbors

## Cafe01-RT1 / Cafe01-RT01 - DHCP and NAT

show running-config ¦ section dhcp show ip dhcp pool show ip dhcp binding show access-lists 10 show ip nat statistics show ip nat translations

## Cafe01-SW01 and Cafe01-SW02

show ip interface brief show interfaces status show vlan brief show mac address-table show cdp neighbors show running-config ¦ section interface Vlan1 show running-config ¦ include ip default-gateway

## Internet Router

show ip interface brief show ip route show running-config ¦ include ip route show interfaces description show cdp neighbors

## All Cisco Devices and End Hosts

| Location | Command | Purpose |
| --- | --- | --- |
| Any Cisco device | show running-config | Display the active configuration. |
| Any Cisco device | show startup-config | Confirm the saved configuration. |
| Any Cisco device | show version | Identify the model, IOS image, and uptime. |
| PC / laptop | ipconfig | Verify the IP address, mask, gateway, and DNS. |
| PC / laptop | ping <destination> | Test reachability to one destination. |
| PC / laptop | tracert <destination> | Identify the routed path and the first failing hop. |

# Part 1 - Read the /26 Before You Configure It

## Step 1: Convert the Prefix to a Mask

A /26 prefix contains 26 network bits and 6 host bits. In the final octet, the first two bits are network bits: 11000000. Binary 11000000 equals decimal 192, so the subnet mask is 255.255.255.192.

| Item | Value | How to Derive It |
| --- | --- | --- |
| Prefix length | /26 | 26 network bits; 6 host bits |
| Subnet mask | 255.255.255.192 | Binary last octet 11000000 |
| Block size | 64 | 256 - 192 |
| Total addresses | 64 | 2^6 host-bit combinations |
| Usable hosts | 62 | 64 minus network and broadcast |
| Wildcard mask | 0.0.0.63 | Inverse of 255.255.255.192 |

## Step 2: Identify the Subnet Boundaries

A block size of 64 produces subnet boundaries at .0, .64, .128, and .192. The selected subnet begins at 10.0.18.0 and ends immediately before the next boundary, 10.0.18.64.

| Address Type | Address | Valid Host? |
| --- | --- | --- |
| Network address | 10.0.18.0 | No - identifies the subnet |
| First usable host | 10.0.18.1 | Yes |
| Last usable host | 10.0.18.62 | Yes |
| Broadcast address | 10.0.18.63 | No - reaches all hosts in the subnet |
| Next subnet | 10.0.18.64/26 | Not part of this LAN |

CCNA checkpoint: Do not assign 10.0.18.0 or 10.0.18.63 to a device. The first is the network address and the second is the broadcast address.

## Student Subnet Worksheet

| Question | Student Answer |
| --- | --- |
| What is the dotted-decimal mask for /26? | ____________________________ |
| What is the block size? | ____________________________ |
| How many total addresses are present? | ____________________________ |
| How many usable host addresses are present? | ____________________________ |
| What is the wildcard mask? | ____________________________ |

# Part 2 - Build a Predictable Addressing Policy

Use address patterns so that documentation, troubleshooting, DHCP exclusions, and future expansion remain predictable. This lab follows the class strategy below.

| Range | Purpose | Examples |
| --- | --- | --- |
| 10.0.18.1-10.0.18.10 | Infrastructure and management | Router, switches, WAPs, printer, server |
| 10.0.18.11-10.0.18.50 | DHCP clients | PCs, laptops, tablets, cameras, IoT clients |
| 10.0.18.51-10.0.18.62 | Reserved static / specialty | Future servers, monitoring, special devices |

## Target Addressing Plan

| Device | Interface | IPv4 Address | Mask | Gateway / Purpose |
| --- | --- | --- | --- | --- |
| Cafe01-RT1 | G0/0 | 10.0.18.1 | 255.255.255.192 | LAN gateway / NAT inside |
| Cafe01-SW01 | VLAN 1 | 10.0.18.2 | 255.255.255.192 | Gateway 10.0.18.1 |
| Cafe01-SW02 | VLAN 1 | 10.0.18.3 | 255.255.255.192 | Gateway 10.0.18.1 |
| WAP1 | Management | 10.0.18.4 | 255.255.255.192 | Gateway 10.0.18.1 |
| Printer | NIC | 10.0.18.5 | 255.255.255.192 | Gateway 10.0.18.1 |
| Server | Fa0 | 10.0.18.6 | 255.255.255.192 | Gateway 10.0.18.1 |
| WAP2 | Management | 10.0.18.7 | 255.255.255.192 | Gateway 10.0.18.1 |
| PCs / Laptops | Fa0 / Wireless0 | DHCP: .11-.50 | 255.255.255.192 | Gateway 10.0.18.1 |
| Cafe01-RT1 | G0/1 | 216.0.5.2 | 255.255.255.252 | WAN / NAT outside |
| Internet | G0/0 | 216.0.5.1 | 255.255.255.252 | WAN peer |
| Internet | Loopback0 | 1.1.1.1 | 255.255.255.255 | Simulated Internet target |
| Internet | Loopback1 | 8.8.8.8 | 255.255.255.255 | Simulated Internet target |

Design logic: Using .1 for the gateway, .2 and .3 for switches, and low addresses for infrastructure creates a pattern that is easy to remember and document. The device roles matter more than the exact numbers, but consistency matters.

# Part 3 - Audit the Supplied Packet Tracer Network

## Step 1: Draw the Network Again

1.  Open the supplied Packet Tracer file.

2.  On paper, draw Cafe01-RT1, Internet, Cafe01-SW01, Cafe01-SW02, the WAPs, server, PCs, laptops, and printer if present.

3.  Label each physical connection and write the target IP address beside each infrastructure device.

4.  Write 10.0.18.0/26 around the entire inside LAN and 216.0.5.0/30 on the router-to-Internet link.

## Step 2: Record the Existing State

! On Cafe01-RT1 / Cafe01-RT01 show ip interface brief show ip route show running-config ¦ include ip route show running-config ¦ section dhcp show running-config ¦ include ip nat¦access-list 10 show cdp neighbors ! On Cafe01-SW01 and Cafe01-SW02 show ip interface brief show interfaces status show vlan brief show running-config ¦ section interface Vlan1 show running-config ¦ include ip default-gateway show cdp neighbors ! On Internet show ip interface brief show ip route show running-config ¦ include ip route

## Step 3: Compare the Current State with the Target

| Item to Audit | Target Result | Student Finding |
| --- | --- | --- |
| Router LAN interface | 10.0.18.1/26 and ip nat inside | ________________________ |
| SW01 management SVI | 10.0.18.2/26 | ________________________ |
| SW02 management SVI | 10.0.18.3/26 | ________________________ |
| DHCP usable range | 10.0.18.11-10.0.18.50 | ________________________ |
| NAT ACL | permit 10.0.18.0 0.0.0.63 | ________________________ |
| WAN default route | 0.0.0.0/0 via 216.0.5.1 | ________________________ |

Important audit finding: The uploaded router configuration excludes .1-.19 and .50-.62, which creates a different DHCP range. This lab replaces those exclusions so the DHCP range exactly matches the lecture strategy: .11 through .50.

# Part 4 - Configure Cafe01-RT1 for the New Subnet

## Step 1: Configure the LAN and WAN Interfaces

Cafe01-RT1> enable Cafe01-RT1# configure terminal Cafe01-RT1(config)# no ip domain-lookup Cafe01-RT1(config)# interface GigabitEthernet0/0 Cafe01-RT1(config-if)# description DISTRICT_SHOP_1_LAN_10.0.18.0_26 Cafe01-RT1(config-if)# ip address 10.0.18.1 255.255.255.192 Cafe01-RT1(config-if)# ip nat inside Cafe01-RT1(config-if)# no shutdown Cafe01-RT1(config-if)# exit Cafe01-RT1(config)# interface GigabitEthernet0/1 Cafe01-RT1(config-if)# description WAN_TO_INTERNET Cafe01-RT1(config-if)# ip address 216.0.5.2 255.255.255.252 Cafe01-RT1(config-if)# ip nat outside Cafe01-RT1(config-if)# no shutdown Cafe01-RT1(config-if)# exit Cafe01-RT1(config)# ip route 0.0.0.0 0.0.0.0 216.0.5.1 Cafe01-RT1(config)# end Cafe01-RT1# copy running-config startup-config

## Step 2: Remove Stale Addressing

Inspect unused interfaces, old loopbacks, old IP routes, and old DHCP pools. Remove only entries that belong to the previous design. Do not remove the active Internet loopbacks on the Internet router.

show running-config show ip interface brief show ip route ! Example cleanup pattern - use only when the old command actually exists: configure terminal interface <old-interface> no ip address shutdown exit no ip route <old-network> <old-mask> <old-next-hop> end

Why cleanup matters: Changing the new address without removing stale configuration can leave overlapping routes, old NAT matches, incorrect descriptions, or unused interfaces that complicate troubleshooting.

## Step 3: Verify the Router Interfaces

show ip interface brief show ip route show running-config ¦ include ip route ping 216.0.5.1 ping 1.1.1.1 ping 8.8.8.8

Expected: G0/0 is 10.0.18.1/26 and up/up; G0/1 is 216.0.5.2/30 and up/up; the default route points to 216.0.5.1.

# Part 5 - Configure Switch Management Addresses

The Catalyst 2960 switches remain Layer 2 devices. Their VLAN 1 SVIs are management addresses only; they do not route client traffic. The ip default-gateway command tells each switch how to reach off-subnet management stations.

## Step 1: Configure Cafe01-SW01

Cafe01-SW01> enable Cafe01-SW01# configure terminal Cafe01-SW01(config)# hostname Cafe01-SW01 Cafe01-SW01(config)# interface Vlan1 Cafe01-SW01(config-if)# description MANAGEMENT_SVI Cafe01-SW01(config-if)# ip address 10.0.18.2 255.255.255.192 Cafe01-SW01(config-if)# no shutdown Cafe01-SW01(config-if)# exit Cafe01-SW01(config)# ip default-gateway 10.0.18.1 Cafe01-SW01(config)# end Cafe01-SW01# copy running-config startup-config

## Step 2: Configure Cafe01-SW02

Cafe01-SW02> enable Cafe01-SW02# configure terminal Cafe01-SW02(config)# hostname Cafe01-SW02 Cafe01-SW02(config)# interface Vlan1 Cafe01-SW02(config-if)# description MANAGEMENT_SVI Cafe01-SW02(config-if)# ip address 10.0.18.3 255.255.255.192 Cafe01-SW02(config-if)# no shutdown Cafe01-SW02(config-if)# exit Cafe01-SW02(config)# ip default-gateway 10.0.18.1 Cafe01-SW02(config)# end Cafe01-SW02# copy running-config startup-config

## Step 3: Verify Switch Management

show ip interface brief show interfaces status show running-config ¦ section interface Vlan1 ping 10.0.18.1 ! Verify the other switch ping 10.0.18.2 ping 10.0.18.3

SVI state: A Layer 2 switch SVI may remain down/down until VLAN 1 has at least one active physical port. Check cabling and show interfaces status before assuming the IP configuration is wrong.

# Part 6 - Configure DHCP to Match the Address Policy

## Step 1: Replace the Existing Exclusions

The lecture reserves .1-.10 for infrastructure, assigns .11-.50 through DHCP, and reserves .51-.62 for specialty static addresses. Remove the older exclusions before adding the new ranges.

Cafe01-RT1# configure terminal ! Remove the ranges found in the supplied router configuration. Cafe01-RT1(config)# no ip dhcp excluded-address 10.0.18.1 10.0.18.19 Cafe01-RT1(config)# no ip dhcp excluded-address 10.0.18.50 10.0.18.62 ! Apply the class addressing policy. Cafe01-RT1(config)# ip dhcp excluded-address 10.0.18.1 10.0.18.10 Cafe01-RT1(config)# ip dhcp excluded-address 10.0.18.51 10.0.18.62

## Step 2: Rebuild the DHCP Pool

Cafe01-RT1(config)# no ip dhcp pool DISTRICT_SHOP_1 Cafe01-RT1(config)# ip dhcp pool DISTRICT_SHOP_1 Cafe01-RT1(dhcp-config)# network 10.0.18.0 255.255.255.192 Cafe01-RT1(dhcp-config)# default-router 10.0.18.1 Cafe01-RT1(dhcp-config)# dns-server 1.1.1.1 Cafe01-RT1(dhcp-config)# exit Cafe01-RT1(config)# end Cafe01-RT1# copy running-config startup-config

## Step 3: Clear Old Bindings and Request New Leases

Cafe01-RT1# clear ip dhcp binding * Cafe01-RT1# show ip dhcp pool Cafe01-RT1# show ip dhcp binding

1.  On each PC or laptop, open Desktop > IP Configuration.

2.  Select Static briefly, then select DHCP to force a new request.

3.  Open Command Prompt and run ipconfig.

4.  Confirm the address is between 10.0.18.11 and 10.0.18.50, with mask 255.255.255.192 and gateway 10.0.18.1.

Lease-order note: The first clients commonly receive .11 and .12, but the exact order depends on previous bindings and device requests. The grading requirement is that every lease falls inside .11-.50.

# Part 7 - Update NAT for 10.0.18.0/26

## Step 1: Understand the Wildcard Mask

Cisco access lists use a wildcard mask. The inverse of 255.255.255.192 is 0.0.0.63. Therefore, ACL 10 must permit 10.0.18.0 0.0.0.63 so every usable address in the /26 can be translated.

| Subnet Mask Octet | Wildcard Octet | Calculation |
| --- | --- | --- |
| 255 | 0 | 255 - 255 |
| 255 | 0 | 255 - 255 |
| 255 | 0 | 255 - 255 |
| 192 | 63 | 255 - 192 |

## Step 2: Rebuild the Dynamic NAT Match

Cafe01-RT1# configure terminal ! Remove the old match and NAT statement if present. Cafe01-RT1(config)# no ip nat inside source list 10 pool CAFE_DYN_POOL Cafe01-RT1(config)# no access-list 10 ! Match the new /26 and restore dynamic NAT. Cafe01-RT1(config)# access-list 10 permit 10.0.18.0 0.0.0.63 Cafe01-RT1(config)# ip nat pool CAFE_DYN_POOL 216.0.5.20 216.0.5.21 netmask 255.255.255.0 Cafe01-RT1(config)# ip nat inside source list 10 pool CAFE_DYN_POOL Cafe01-RT1(config)# end Cafe01-RT1# clear ip nat translation * Cafe01-RT1# copy running-config startup-config

## Step 3: Verify the Internet Router Return Paths

Internet> enable Internet# configure terminal Internet(config)# ip route 216.0.5.20 255.255.255.255 216.0.5.2 Internet(config)# ip route 216.0.5.21 255.255.255.255 216.0.5.2 Internet(config)# end Internet# copy running-config startup-config Internet# show ip route 216.0.5.20 Internet# show ip route 216.0.5.21

Dynamic NAT capacity: This pool contains only two public addresses, so only two inside devices can hold active one-to-one translations at the same time. If a third client cannot reach the Internet, inspect the NAT table before blaming subnetting.

# Part 8 - Configure Infrastructure and Client Devices

## Step 1: Assign Static Infrastructure Addresses

| Device | Address | Mask | Default Gateway | Configuration Method |
| --- | --- | --- | --- | --- |
| WAP1 | 10.0.18.4 | 255.255.255.192 | 10.0.18.1 | Config tab / management interface |
| Printer | 10.0.18.5 | 255.255.255.192 | 10.0.18.1 | Config tab or Desktop IP Configuration |
| Server | 10.0.18.6 | 255.255.255.192 | 10.0.18.1 | Desktop > IP Configuration |
| WAP2 | 10.0.18.7 | 255.255.255.192 | 10.0.18.1 | Config tab / management interface |

If the Packet Tracer topology does not include a printer or second WAP, keep the address reserved. Do not reuse it for DHCP clients.

## Step 2: Configure PCs and Laptops for DHCP

1.  Open the PC or laptop.

2.  Select Desktop > IP Configuration.

3.  Choose DHCP.

4.  Wait for the address, mask, gateway, and DNS fields to populate.

5.  Use ipconfig to verify the lease.

## Step 3: Validate Every Address Against the /26

| Validation Question | Pass Condition |
| --- | --- |
| Is the mask correct? | 255.255.255.192 |
| Is the address inside the usable range? | 10.0.18.1 through 10.0.18.62 |
| Is it a duplicate? | Every static address is unique |
| Does a client use the DHCP range? | 10.0.18.11 through 10.0.18.50 |
| Is the gateway correct? | 10.0.18.1 |
| Is DNS present? | 1.1.1.1 |

Common mistake: A 10.0.18.x address with a 255.255.255.0 mask is not equivalent to the /26 design. The wrong mask changes the host's understanding of which addresses are local.

# Part 9 - Verify the Deployment

## Test 1: Verify a DHCP Client

C:\> ipconfig C:\> ping 10.0.18.1 C:\> ping 10.0.18.2 C:\> ping 10.0.18.3 C:\> ping 10.0.18.4 C:\> ping 10.0.18.6

Expected: the client has an address from .11-.50 and can reach the gateway and infrastructure devices.

## Test 2: Verify WAN and Internet Reachability

C:\> ping 216.0.5.2 C:\> ping 216.0.5.1 C:\> ping 1.1.1.1 C:\> ping 8.8.8.8 C:\> tracert 8.8.8.8

Expected traceroute: the first hop is 10.0.18.1, followed by the Internet router path to 8.8.8.8. Packet Tracer may display only the routed hops that respond.

## Test 3: Verify Services on Cafe01-RT1

show ip interface brief show ip route show running-config ¦ include ip route show ip dhcp pool show ip dhcp binding show access-lists 10 show ip nat statistics show ip nat translations

## Test 4: Verify Switch Management

! On Cafe01-SW01 ping 10.0.18.1 ping 10.0.18.3 ! On Cafe01-SW02 ping 10.0.18.1 ping 10.0.18.2

First-ping behavior: The first echo may time out while ARP resolves a MAC address. Repeat the ping before deciding that the configuration failed.

# Part 10 - Read the Evidence, Not Just the Green Links

## DHCP Evidence

| Command | What the Student Should Confirm |
| --- | --- |
| show ip dhcp pool | Network is 10.0.18.0/26 and addresses are available |
| show ip dhcp binding | Client IPs are between .11 and .50 and mapped to client identifiers |
| ipconfig | Client mask is /26, gateway is .1, and DNS is 1.1.1.1 |

## NAT Evidence

| Command | What the Student Should Confirm |
| --- | --- |
| show access-lists 10 | Matches 10.0.18.0 with wildcard 0.0.0.63 |
| show ip nat statistics | G0/0 is inside; G0/1 is outside; CAFE_DYN_POOL is active |
| show ip nat translations | Inside local 10.0.18.x is temporarily mapped to .20 or .21 |

## Routing Evidence

| Device | Expected Route / Connected Network |
| --- | --- |
| Cafe01-RT1 | C 10.0.18.0/26 via G0/0 |
| Cafe01-RT1 | C 216.0.5.0/30 via G0/1 |
| Cafe01-RT1 | S* 0.0.0.0/0 via 216.0.5.1 |
| Internet | Host routes to 216.0.5.20 and 216.0.5.21 via 216.0.5.2 |
| Internet | Connected loopbacks 1.1.1.1/32 and 8.8.8.8/32 |

## Operational Proof Table

| Test | Result | Student Initials |
| --- | --- | --- |
| PC receives a .11-.50 DHCP lease | Pass / Fail | ________ |
| PC pings 10.0.18.1 | Pass / Fail | ________ |
| PC pings switch management addresses | Pass / Fail | ________ |
| PC pings 8.8.8.8 | Pass / Fail | ________ |
| NAT translation appears | Pass / Fail | ________ |
| Local console login and enable secret tested | Pass / Fail | ________ |

# Part 11 - Troubleshooting Guide

| Symptom | Likely Cause | Corrective Action |
| --- | --- | --- |
| Client receives 169.254.x.x | DHCP request failed or path to router is down | Verify switch ports, G0/0, DHCP pool, and exclusions; renew the lease. |
| Client receives .20-.49 instead of .11-.50 | Old exclusion ranges still exist | Remove .1-.19 and .50-.62 exclusions; configure .1-.10 and .51-.62. |
| Client has 10.0.18.x but /24 mask | Old static mask remains | Set DHCP or change the mask to 255.255.255.192. |
| Client cannot ping 10.0.18.1 | Wrong address, mask, gateway, cabling, or router LAN interface | Check ipconfig, G0/0 up/up, and switch port status. |
| Router can ping 8.8.8.8 but client cannot | NAT ACL or inside/outside roles are wrong | Verify ACL 10 wildcard, ip nat inside on G0/0, and ip nat outside on G0/1. |
| Two clients work; a third fails Internet tests | Two-address dynamic NAT pool is exhausted | Clear translations, wait for timeout, enlarge the pool, or use PAT in a later lab. |
| Switch management SVI is down/down | No active port in VLAN 1 | Connect/enable a VLAN 1 port and verify show interfaces status. |
| Switch cannot ping outside LAN | Missing or wrong ip default-gateway | Configure ip default-gateway 10.0.18.1. |
| NAT table remains empty | Traffic did not match ACL or no outside traffic generated | Ping 1.1.1.1/8.8.8.8 and verify access-list 10 counters. |
| First ping loses one packet | ARP resolution | Repeat the ping; one initial timeout can be normal. |
| Console or VTY rejects cisco / cisco | Local user is missing, the secret is incorrect, or the line does not use login local. | Verify username cisco secret cisco and inspect the console and VTY sections of the running configuration. |

## Structured Troubleshooting Order

1.  Confirm the client IP address, /26 mask, and default gateway.

2.  Ping the local gateway 10.0.18.1.

3.  Ping the router WAN interface 216.0.5.2.

4.  Ping the Internet router 216.0.5.1.

5.  Ping 1.1.1.1 or 8.8.8.8.

6.  Inspect DHCP bindings, routing, the NAT ACL, and NAT translations at the first point of failure.

# Lab Completion Checklist

| Completion Item | Complete |
| --- | --- |
| The network was redrawn and all interfaces were labeled. | [ ] |
| 10.0.18.0/26 was calculated correctly. | [ ] |
| Cafe01-RT1 G0/0 is 10.0.18.1/26 and marked ip nat inside. | [ ] |
| Cafe01-RT1 G0/1 is 216.0.5.2/30 and marked ip nat outside. | [ ] |
| Cafe01-SW01 VLAN 1 is 10.0.18.2/26. | [ ] |
| Cafe01-SW02 VLAN 1 is 10.0.18.3/26. | [ ] |
| Infrastructure addresses .1-.10 are excluded from DHCP. | [ ] |
| Addresses .51-.62 are excluded from DHCP. | [ ] |
| DHCP clients receive addresses from .11-.50. | [ ] |
| ACL 10 matches 10.0.18.0 0.0.0.63. | [ ] |
| Dynamic NAT translations appear after Internet pings. | [ ] |
| A client can ping 8.8.8.8. | [ ] |
| All router and switch configurations are saved. | [ ] |
| Local username, enable secret, service password-encryption, console login local, and VTY login local are configured and tested. | [ ] |

# Student Knowledge Check

1. What dotted-decimal mask corresponds to /26?

________________________________________________________________________________

2. How is the block size calculated from 255.255.255.192?

________________________________________________________________________________

3. What are the network, first usable, last usable, and broadcast addresses for 10.0.18.0/26?

________________________________________________________________________________

4. Why is 10.0.18.63 not assignable to a host?

________________________________________________________________________________

5. How many usable host addresses are available in a /26?

________________________________________________________________________________

# Student Knowledge Check - Continued

6. Why does the DHCP server exclude 10.0.18.1 through 10.0.18.10?

________________________________________________________________________________

7. How many addresses are in the lecture DHCP range 10.0.18.11 through 10.0.18.50?

________________________________________________________________________________

8. Why does the NAT access list use 0.0.0.63 instead of 255.255.255.192?

________________________________________________________________________________

9. Why can a two-address dynamic NAT pool support only two simultaneous inside translations?

________________________________________________________________________________

10. What problem can occur if a device keeps a /24 mask after the network changes to /26?

________________________________________________________________________________

11. Why is redrawing the topology useful even when a formal diagram already exists?

________________________________________________________________________________

12. Why is login local required on the console and VTY lines after creating a local username?

________________________________________________________________________________

13. Which show commands verify the username, enable secret, console settings, and VTY settings?

________________________________________________________________________________

# Student Reflection

Describe one addressing, DHCP, NAT, or routing problem you encountered. Identify the command or test that revealed the problem, explain the correction, and describe how the final result proved that the subnet strategy was successfully operationalized.

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________