CCNA LAB MANUAL

Skill 12 - Lesson 06 Lab

The Purpose of the Native VLAN

Castle Rysen - Native VLAN Alignment Exercise

| Student: ____________________ | Date: ____________________ | Lab Time: __________ |
| --- | --- | --- |

Figure 1. Starting topology: Cafe-SW1 Fa0/1 connected to Cafe-SW2 Fa0/1.

Lab focus: Observe how an IEEE 802.1Q trunk handles untagged traffic, intentionally create a native VLAN mismatch, and then correct the mismatch by configuring native VLAN 99 on both ends.

# Lab Overview

## Mission Briefing

The virtualization cluster that keeps Castle Rysen's coffee roasters humming has entered the bunker lab. Jeremy wants you to trace how untagged management frames survive a trunked network. Your drill is to discover the native VLAN's role, move untagged trunk traffic into VLAN 99, observe a deliberate mismatch, and restore a consistent trunk configuration.

## Purpose

Audit a nontrunking switch-to-switch link, convert it to a static IEEE 802.1Q trunk, assign VLAN 99 as the native VLAN, and verify that both trunk endpoints agree.

## Training Objectives

Audit the Cafe-SW1-to-Cafe-SW2 uplink and document its initial switchport state.

Confirm VLANs 10 and 20 already exist before changing the uplink.

Create VLAN 99 and name it MGMT-NATIVE on both switches.

Force the uplink into static 802.1Q trunking.

Assign VLAN 99 as the native VLAN and explain how untagged frames are handled.

Recognize a CDP native VLAN mismatch and correct it.

Verify that VLANs 1, 10, 20, and 99 are active and forwarding across the trunk.

## Native VLAN Concept

Core idea: On an IEEE 802.1Q trunk, frames in the native VLAN are normally transmitted without an 802.1Q tag. An untagged frame received on a trunk is placed into the local native VLAN. Both ends of the trunk must use the same native VLAN to avoid traffic being classified into different VLANs.

| Term | Meaning in this lab |
| --- | --- |
| Access port | Carries traffic for one VLAN and sends/receives frames without VLAN tags. |
| 802.1Q trunk | Carries traffic for multiple VLANs over one physical link. |
| Native VLAN | The VLAN used for untagged frames on the 802.1Q trunk. |
| Native VLAN mismatch | The two trunk endpoints assign untagged traffic to different VLANs. |
| VLAN 99 | The dedicated native VLAN selected for this exercise. |

Important distinction: This lab names VLAN 99 MGMT-NATIVE and uses it as the trunk native VLAN. That does not automatically configure a switch management IP address. A management SVI would require a separate interface Vlan99 configuration, which is outside this exercise.

## Equipment and Starting Conditions

Two Cisco switches named Cafe-SW1 and Cafe-SW2.

A physical link between Fa0/1 on Cafe-SW1 and Fa0/1 on Cafe-SW2.

VLAN 10 and VLAN 20 already exist on both switches.

The Fa0/1 link is physically connected but is not trunking at the beginning of the lab.

Cisco Discovery Protocol (CDP) is enabled so a native VLAN mismatch can be reported.

Interface naming: The topology uses FastEthernet0/1 (Fa0/1). The supplied instructions use Ethernet0/1 (Et0/1). Use the interface name shown on your switch. Replace every Fa0/1 command with Ethernet0/1 if your platform uses Et0/1.

Catalyst 2960 note: A Catalyst 2960 supports only 802.1Q trunk encapsulation. If switchport trunk encapsulation dot1q returns an invalid-input message, omit that command and continue with switchport mode trunk.

# Topology and VLAN Plan

| Local Device | Local Port | Remote Device | Remote Port | Starting State |
| --- | --- | --- | --- | --- |
| Cafe-SW1 | Fa0/1 | Cafe-SW2 | Fa0/1 | Connected; dynamic auto; operationally access VLAN 1 |

| VLAN | Name | Purpose | Expected Starting Presence |
| --- | --- | --- | --- |
| 1 | default | Initial access/native VLAN | Present by default |
| 10 | VLAN0010 | Existing access VLAN | Present before the lab |
| 20 | VLAN0020 | Existing access VLAN | Present before the lab |
| 99 | MGMT-NATIVE | New native VLAN for untagged trunk frames | Created during Tasks 1 and 2 |

| Checkpoint | Expected Result | Student Initials |
| --- | --- | --- |
| Physical link | Fa0/1 to Fa0/1 is connected/up |  |
| Initial trunk summary | Blank; no active trunks |  |
| Initial operational mode | Static access |  |
| Initial access VLAN | VLAN 1 |  |
| Existing VLANs | VLAN 10 and VLAN 20 are active |  |

Before continuing: Do not configure VLAN 99 or trunk mode until you have recorded the baseline. The temporary mismatch created later is easier to understand when the initial condition is known.

# Task 0 - Profile the Untagged Lifeline

Goal. Document how the Fa0/1 link currently handles frames before trunking is enabled.

Open the Cafe-SW1 console and enter privileged EXEC mode.

Display the trunk summary. A blank result confirms that no interface is currently trunking.

Inspect the detailed switchport state for Fa0/1.

Confirm that the administrative mode is dynamic auto, the operational mode is static access, and the access VLAN is VLAN 1.

Confirm VLANs 10 and 20 already exist.

Review the log for any existing VLAN or native VLAN warnings.

Cafe-SW1> enable Cafe-SW1# show interfaces trunk Cafe-SW1# show interfaces FastEthernet0/1 switchport ¦ include Administrative Mode¦Operational Mode¦Access Mode¦Native VLAN¦Trunking VLANs Enabled Cafe-SW1# show vlan brief Cafe-SW1# show logging ¦ include VLAN¦Native

Command variation: Some IOS images accept show interface trunk (singular), while others use show interfaces trunk (plural). Use the form supported by your switch.

## Sample Starting State

Cafe-SW1# show interfaces trunk Cafe-SW1# Administrative Mode: dynamic auto Operational Mode: static access Access Mode VLAN: 1 (default) Trunking VLANs Enabled: ALL 10 VLAN0010 active Fa0/2 20 VLAN0020 active Fa0/3

Interpretation: Two interfaces in dynamic auto do not negotiate a trunk with each other. The physically connected link therefore remains an access link in VLAN 1 until you force trunk mode.

| Observation | Recorded Value |
| --- | --- |
| Is Fa0/1 listed by show interfaces trunk? |  |
| Administrative Mode |  |
| Operational Mode |  |
| Access Mode VLAN |  |
| Native VLAN shown in switchport details |  |
| VLAN 10 present? |  |
| VLAN 20 present? |  |
| Existing mismatch messages? |  |

# Task 1 - Anchor Management Traffic on Cafe-SW1

Goal. Create VLAN 99, force Fa0/1 into 802.1Q trunking, and assign VLAN 99 as the native VLAN on Cafe-SW1.

Create VLAN 99 and name it MGMT-NATIVE.

Enter interface configuration mode for Fa0/1.

Select 802.1Q trunk encapsulation when the platform supports the command.

Force the interface into static trunk mode.

Set VLAN 99 as the native VLAN.

Verify the trunk and review the log for the expected temporary mismatch warning.

Cafe-SW1> enable Cafe-SW1# configure terminal Cafe-SW1(config)# vlan 99 Cafe-SW1(config-vlan)# name MGMT-NATIVE Cafe-SW1(config-vlan)# exit Cafe-SW1(config)# interface FastEthernet0/1 Cafe-SW1(config-if)# switchport trunk encapsulation dot1q Cafe-SW1(config-if)# switchport mode trunk Cafe-SW1(config-if)# switchport trunk native vlan 99 Cafe-SW1(config-if)# end Cafe-SW1# show interfaces trunk Cafe-SW1# show logging ¦ include Native¦mismatch

Expected temporary condition: Cafe-SW1 now uses native VLAN 99, but Cafe-SW2 still uses VLAN 1. CDP should detect this disagreement and report a native VLAN mismatch.

## Expected Syslog Message

%CDP-4-NATIVE_VLAN_MISMATCH: Native VLAN mismatch discovered on FastEthernet0/1 (99), with Cafe-SW2 FastEthernet0/1 (1).

Why this matters: An untagged frame arriving on Cafe-SW1 would be placed in VLAN 99, while the same type of frame arriving on Cafe-SW2 would be placed in VLAN 1. The two ends would disagree about the frame's VLAN membership.

| Cafe-SW1 Check | Expected Result | Student Initials |
| --- | --- | --- |
| VLAN 99 | Active and named MGMT-NATIVE |  |
| Fa0/1 mode | Trunking |  |
| Encapsulation | 802.1Q |  |
| Native VLAN | 99 |  |
| Temporary mismatch log | Present while Cafe-SW2 remains on native VLAN 1 |  |

# Task 2 - Stabilize the Native VLAN Across the Uplink

Goal. Configure Cafe-SW2 to match Cafe-SW1 so both ends use VLAN 99 as the native VLAN.

Open the Cafe-SW2 console and enter privileged EXEC mode.

Create VLAN 99 and use the same MGMT-NATIVE name.

Configure Fa0/1 as a static 802.1Q trunk.

Assign VLAN 99 as the native VLAN.

Verify the trunk summary and the active VLAN lists.

Review the log. Historical mismatch messages may remain, but no new mismatch messages should be generated after both sides match.

Cafe-SW2> enable Cafe-SW2# configure terminal Cafe-SW2(config)# vlan 99 Cafe-SW2(config-vlan)# name MGMT-NATIVE Cafe-SW2(config-vlan)# exit Cafe-SW2(config)# interface FastEthernet0/1 Cafe-SW2(config-if)# switchport trunk encapsulation dot1q Cafe-SW2(config-if)# switchport mode trunk Cafe-SW2(config-if)# switchport trunk native vlan 99 Cafe-SW2(config-if)# end Cafe-SW2# show interfaces trunk Cafe-SW2# show logging ¦ include Native¦mismatch

Save the configuration: After verification succeeds, save both switches with copy running-config startup-config.

Cafe-SW1# copy running-config startup-config Cafe-SW2# copy running-config startup-config

## Expected Trunk Alignment

Port Mode Encapsulation Status Native vlan Fa0/1 on 802.1q trunking 99 Port Vlans allowed and active in management domain Fa0/1 1,10,20,99 Port Vlans in spanning tree forwarding state and not pruned Fa0/1 1,10,20,99

Final log condition: The earlier mismatch entry may still appear in the buffered log. Confirm that no new %CDP-4-NATIVE_VLAN_MISMATCH messages appear after Cafe-SW2 is corrected.

# Final Verification

## Step 1: Verify Cafe-SW1

Cafe-SW1# show interfaces trunk Cafe-SW1# show interfaces FastEthernet0/1 switchport Cafe-SW1# show vlan brief Cafe-SW1# show logging ¦ include Native¦mismatch Cafe-SW1# show running-config interface FastEthernet0/1

## Step 2: Verify Cafe-SW2

Cafe-SW2# show interfaces trunk Cafe-SW2# show interfaces FastEthernet0/1 switchport Cafe-SW2# show vlan brief Cafe-SW2# show logging ¦ include Native¦mismatch Cafe-SW2# show running-config interface FastEthernet0/1

## Expected Results

| Verification Item | Cafe-SW1 | Cafe-SW2 |
| --- | --- | --- |
| Fa0/1 operational mode | Trunk | Trunk |
| Trunk encapsulation | 802.1Q | 802.1Q |
| Native VLAN | 99 | 99 |
| VLAN 99 name | MGMT-NATIVE | MGMT-NATIVE |
| Allowed and active VLANs | 1,10,20,99 | 1,10,20,99 |
| New mismatch alerts | None after alignment | None after alignment |
| Configuration saved | Yes | Yes |

Completion standard: The lab is complete when both switches list Fa0/1 as an 802.1Q trunk with native VLAN 99 and no new native VLAN mismatch messages are generated.

## Packet-Flow Interpretation

| Frame Arriving on Trunk | How the Receiving Switch Handles It |
| --- | --- |
| Tagged VLAN 10 frame | Places the frame into VLAN 10. |
| Tagged VLAN 20 frame | Places the frame into VLAN 20. |
| Tagged VLAN 99 frame | Normally uncommon for the native VLAN unless native VLAN tagging is explicitly enabled. |
| Untagged frame | Places the frame into native VLAN 99. |

# Troubleshooting Guide

| Symptom | Likely Cause | Corrective Action |
| --- | --- | --- |
| show interfaces trunk is blank after configuration | The interface is not in trunk mode, the cable is disconnected, or the wrong interface was configured. | Check cabling, interface status, and switchport mode trunk on the actual uplink. |
| switchport trunk encapsulation dot1q is rejected | The switch supports only 802.1Q and does not expose an encapsulation-selection command. | Omit the command and continue with switchport mode trunk. |
| Native VLAN mismatch continues | One side still uses VLAN 1 or a different native VLAN. | Run show interfaces trunk on both switches and set switchport trunk native vlan 99 on both ends. |
| VLAN 99 is missing from the active list | VLAN 99 was not created on one switch. | Create vlan 99 and name it MGMT-NATIVE locally on the affected switch. |
| VLAN 10 or VLAN 20 is not active on the trunk | The VLAN is missing locally or is not allowed on the trunk. | Verify show vlan brief and inspect the trunk allowed-VLAN configuration. |
| Old mismatch message remains in the log | Buffered logs preserve historical messages. | Wait and confirm that no new mismatch messages appear; optionally clear logging if permitted. |
| Fa0/1 remains static access | Only one side was changed or trunk mode was not applied. | Configure switchport mode trunk on both endpoints. |
| Wrong port configured | The device uses Ethernet0/1 instead of FastEthernet0/1, or vice versa. | Use show interfaces status or show cdp neighbors to identify the connected uplink. |

Security reminder: A native VLAN mismatch is not merely cosmetic. It can cause untagged traffic to cross VLAN boundaries unexpectedly. Consistent trunk configuration is required on both ends.

## Useful Diagnostic Commands

show interfaces status show interfaces trunk show interfaces FastEthernet0/1 switchport show vlan brief show cdp neighbors show logging ¦ include Native¦mismatch show running-config interface FastEthernet0/1

# Lab Completion Checklist

| Completion Item | Complete |
| --- | --- |
| The Fa0/1-to-Fa0/1 cable is connected and both interfaces are up. | [ ] |
| The initial trunk summary was recorded before configuration. | [ ] |
| VLANs 10 and 20 were confirmed before the change. | [ ] |
| VLAN 99 MGMT-NATIVE exists on Cafe-SW1. | [ ] |
| VLAN 99 MGMT-NATIVE exists on Cafe-SW2. | [ ] |
| Cafe-SW1 Fa0/1 is a static 802.1Q trunk. | [ ] |
| Cafe-SW2 Fa0/1 is a static 802.1Q trunk. | [ ] |
| Cafe-SW1 Fa0/1 uses native VLAN 99. | [ ] |
| Cafe-SW2 Fa0/1 uses native VLAN 99. | [ ] |
| VLANs 1, 10, 20, and 99 are active and forwarding on the trunk. | [ ] |
| The temporary mismatch was observed and explained. | [ ] |
| No new mismatch alerts appear after both sides are aligned. | [ ] |
| Both switch configurations were saved. | [ ] |

# Student Knowledge Check

1. What is the purpose of the native VLAN on an IEEE 802.1Q trunk?

____________________________________________________________________________________________________________

____________________________________________________________________________________________________________

2. How does a switch classify an untagged frame received on a trunk?

____________________________________________________________________________________________________________

____________________________________________________________________________________________________________

3. Why did show interfaces trunk return a blank result at the beginning of the lab?

____________________________________________________________________________________________________________

____________________________________________________________________________________________________________

4. Why must both trunk endpoints use the same native VLAN?

____________________________________________________________________________________________________________

____________________________________________________________________________________________________________

5. What caused the temporary CDP native VLAN mismatch warning?

____________________________________________________________________________________________________________

____________________________________________________________________________________________________________

6. Why was VLAN 99 created on both switches?

____________________________________________________________________________________________________________

____________________________________________________________________________________________________________

7. What is the difference between an access VLAN and a native VLAN?

____________________________________________________________________________________________________________

____________________________________________________________________________________________________________

8. Does naming VLAN 99 MGMT-NATIVE automatically give the switch a management IP address? Explain.

____________________________________________________________________________________________________________

____________________________________________________________________________________________________________

9. Why might an old mismatch message remain in show logging after the problem is corrected?

____________________________________________________________________________________________________________

____________________________________________________________________________________________________________

10. Which commands prove that Fa0/1 is trunking and using native VLAN 99?

____________________________________________________________________________________________________________

____________________________________________________________________________________________________________

# Student Reflection

Describe what happened when Cafe-SW1 used native VLAN 99 while Cafe-SW2 still used native VLAN 1. Identify the commands or log messages that revealed the mismatch, and explain why matching the native VLAN on both ends corrected the problem.

___________________________________________________________________________________________________________________

___________________________________________________________________________________________________________________

___________________________________________________________________________________________________________________

___________________________________________________________________________________________________________________

___________________________________________________________________________________________________________________

___________________________________________________________________________________________________________________

___________________________________________________________________________________________________________________

___________________________________________________________________________________________________________________

___________________________________________________________________________________________________________________

___________________________________________________________________________________________________________________

___________________________________________________________________________________________________________________

___________________________________________________________________________________________________________________

___________________________________________________________________________________________________________________

___________________________________________________________________________________________________________________

___________________________________________________________________________________________________________________