Skill 10 - Lab - Configuring NAT Overload

CCNA beginner practice lab: NAT Overload/PAT on Cafe-RT1

Figure 1. NAT overload topology using Cafe-LAN-SW, Cafe-RT1, and ISP-RT.

Terminology note: NAT Overload uses the Cisco IOS keyword overload. In this lab, you will configure NAT Overload, also called PAT, so multiple inside devices can share public addresses by using port numbers.

# Field Briefing

Coffee House Beta has two wired PCs and a management switch in the inside LAN 192.168.1.0/24. The simulated ISP provides two public NAT pool addresses, 216.0.5.20 and 216.0.5.21. In the previous Dynamic NAT lab, only two inside devices could use the two public pool addresses at the same time. In this lab, you will change the NAT rule on Cafe-RT1 so the same public pool can be overloaded with port numbers.

The focus is Cafe-RT1. You will intentionally disable the outside interface, observe that connectivity fails, configure NAT Overload using access list 1, restore the interface, and validate that inside hosts can reach the simulated Internet targets 1.1.1.1 and 8.8.8.8.

# Lab Objectives

Use the provided topology with PC1, PC2, Cafe-LAN-SW, Cafe-RT1, and ISP-RT.

Configure baseline IP addressing, default routes, and simulated ISP loopbacks.

Identify the NAT inside and NAT outside interfaces on Cafe-RT1.

Shut down the Cafe-RT1 outside interface and observe the effect on connectivity.

Remove previous Dynamic NAT or Static NAT commands from Cafe-RT1.

Configure NAT Overload using access-list 1 and the Cisco IOS overload keyword.

Restore the outside interface and validate NAT translations, NAT statistics, ACL matches, and pings.

Explain the difference between Dynamic NAT and NAT Overload/PAT.

# Access Credentials

| Device access | Value |
| --- | --- |
| Console / VTY username | cisco |
| Console / VTY password | cisco |
| Privileged EXEC secret | CrC0ffee! |

# Addressing Plan

| Device | Interface | Address | Mask | Purpose |
| --- | --- | --- | --- | --- |
| PC2 | Fa0 | 192.168.1.50 | 255.255.255.0 | Inside host |
| PC1 | Fa0 | 192.168.1.51 | 255.255.255.0 | Inside host |
| Cafe-LAN-SW | VLAN 1 | 192.168.1.2 | 255.255.255.0 | Switch management / inside local |
| Cafe-RT1 | G0/1 | 192.168.1.1 | 255.255.255.0 | LAN gateway / NAT inside |
| Cafe-RT1 | G0/0 | 216.0.5.2 | 255.255.255.252 | WAN toward ISP / NAT outside |
| ISP-RT | G0/0 | 216.0.5.1 | 255.255.255.252 | ISP gateway toward Cafe-RT1 |
| ISP-RT | Loopback0 | 1.1.1.1 | 255.255.255.255 | Simulated Internet target |
| ISP-RT | Loopback1 | 8.8.8.8 | 255.255.255.255 | Simulated Internet target |

# NAT Overload Plan

| NAT item | Configuration value | Meaning |
| --- | --- | --- |
| Inside local network | 192.168.1.0/24 | Private inside addresses matched by ACL 1. |
| ACL number | 1 | Standard ACL used to identify inside addresses allowed to be translated. |
| Inside global pool | 216.0.5.20 - 216.0.5.21 | Public addresses used by NAT Overload in this pool-based PAT lab. |
| Pool name | CAFE_POOL | Name used in the ip nat pool command. |
| NAT rule | ip nat inside source list 1 pool CAFE_POOL overload | Maps ACL 1 to the pool and allows port translation. |
| Outside targets | 1.1.1.1 and 8.8.8.8 | Loopbacks on ISP-RT used to test Internet reachability. |

CCNA note: This lab uses pool-based NAT Overload so it still matches the public addresses in the uploaded diagram. A common alternative is interface-based PAT: ip nat inside source list 1 interface gigabitethernet0/0 overload. That alternative would use Cafe-RT1 G0/0 address 216.0.5.2 as the inside global address.

# NAT Vocabulary Review

| Term | Meaning in this lab |
| --- | --- |
| Inside local | The real private address on the inside LAN, such as 192.168.1.50 or 192.168.1.51. |
| Inside global | The public address used to represent the inside host, such as 216.0.5.20 or 216.0.5.21. |
| Outside local | How the outside host appears to the inside network. In this lab it normally stays the same as the outside global address. |
| Outside global | The real outside address, such as 1.1.1.1 or 8.8.8.8. |
| NAT Overload / PAT | Many private inside hosts can share public addresses by using port numbers. |

# Task 1 - Build the Network Topology

Objective: Build the topology shown in Figure 1 before configuring NAT Overload.

Connect PC2 Fa0 to Cafe-LAN-SW Fa0/3.

Connect PC1 Fa0 to Cafe-LAN-SW Fa0/2.

Connect Cafe-LAN-SW Fa0/1 to Cafe-RT1 G0/1.

Connect Cafe-RT1 G0/0 to ISP-RT G0/0.

Use the interface labels shown by your Packet Tracer device. If your router model uses different port numbers, follow your actual port labels.

# Task 2 - Configure PC Addresses

Objective: Configure the two inside hosts with private IPv4 addresses and the correct default gateway.

| PC | IP address | Subnet mask | Default gateway |
| --- | --- | --- | --- |
| PC2 | 192.168.1.50 | 255.255.255.0 | 192.168.1.1 |
| PC1 | 192.168.1.51 | 255.255.255.0 | 192.168.1.1 |

Beginner note: In Packet Tracer, open the PC, go to Desktop > IP Configuration, and enter the IP address, subnet mask, and default gateway from the table.

# Task 3 - Configure Cafe-LAN-SW

Objective: Configure the switch hostname, local login, management SVI, default gateway, and access ports.

Cafe-LAN-SW baseline configuration

enable
configure terminal
hostname Cafe-LAN-SW

username cisco secret cisco
enable secret CrC0ffee!
service password-encryption

line console 0
 login local
 logging synchronous
 exit
line vty 0 15
 login local
 exit

interface vlan 1
 description Management SVI
 ip address 192.168.1.2 255.255.255.0
 no shutdown
 exit

ip default-gateway 192.168.1.1

interface fastethernet0/1
 description To Cafe-RT1 G0/1
 switchport mode access
 no shutdown
 exit
interface fastethernet0/2
 description To PC1 Fa0 - 192.168.1.51
 switchport mode access
 spanning-tree portfast
 no shutdown
 exit
interface fastethernet0/3
 description To PC2 Fa0 - 192.168.1.50
 switchport mode access
 spanning-tree portfast
 no shutdown
 exit

end
copy running-config startup-config

Cafe-LAN-SW verification commands

show ip interface brief
show running-config interface vlan 1
show interfaces description
ping 192.168.1.1

# Task 4 - Configure ISP-RT as the Simulated Internet

Objective: Configure ISP-RT with the WAN address, simulated Internet loopbacks, and return routes to the NAT public pool.

ISP-RT baseline configuration

enable
configure terminal
hostname ISP-RT

username cisco secret cisco
enable secret CrC0ffee!
service password-encryption

line console 0
 login local
 logging synchronous
 exit
line vty 0 4
 login local
 exit

interface gigabitethernet0/0
 description To Cafe-RT1 G0/0
 ip address 216.0.5.1 255.255.255.252
 no shutdown
 exit

interface loopback0
 description Simulated Internet target 1
 ip address 1.1.1.1 255.255.255.255
 exit

interface loopback1
 description Simulated Internet target 2
 ip address 8.8.8.8 255.255.255.255
 exit

ip route 216.0.5.20 255.255.255.255 216.0.5.2
ip route 216.0.5.21 255.255.255.255 216.0.5.2

end
copy running-config startup-config

ISP-RT verification commands

show ip interface brief
show ip route
ping 216.0.5.2
show ip route 216.0.5.20
show ip route 216.0.5.21

# Task 5 - Configure Cafe-RT1 Baseline Routing

Objective: Configure Cafe-RT1 LAN and WAN interfaces, mark NAT zones, and add the default route toward ISP-RT. Do not add the overload rule yet.

Cafe-RT1 baseline configuration

enable
configure terminal
hostname Cafe-RT1

username cisco secret cisco
enable secret CrC0ffee!
service password-encryption

line console 0
 login local
 logging synchronous
 exit
line vty 0 4
 login local
 exit

interface gigabitethernet0/1
 description Default gateway for LAN network 192.168.1.0/24
 ip address 192.168.1.1 255.255.255.0
 ip nat inside
 no shutdown
 exit

interface gigabitethernet0/0
 description To ISP-RT - WAN outside
 ip address 216.0.5.2 255.255.255.252
 ip nat outside
 no shutdown
 exit

ip route 0.0.0.0 0.0.0.0 216.0.5.1

end
copy running-config startup-config

Cafe-RT1 baseline verification commands

show ip interface brief
show ip route
show running-config interface gigabitethernet0/0
show running-config interface gigabitethernet0/1
ping 216.0.5.1

# Task 6 - Disable Cafe-RT1 Outside Interface and Observe Failure

Objective: Practice controlled troubleshooting. Shut down the outside interface before applying the NAT Overload rule. This makes the failure visible and helps you prove that connectivity is restored later.

Disable the outside interface on Cafe-RT1

enable
configure terminal
interface gigabitethernet0/0
 description To ISP-RT - WAN outside - temporarily disabled for NAT Overload lab
 shutdown
 exit
end

Validate that the outside link is down

show ip interface brief
ping 216.0.5.1

Expected failure: Expected result: Cafe-RT1 G0/0 should show administratively down/down, and ping 216.0.5.1 should fail. PC1 and PC2 should also fail to ping 1.1.1.1 and 8.8.8.8 while the outside interface is shut down.

# Task 7 - Remove Previous NAT Practice Configuration

Objective: Clean up old Dynamic NAT or Static NAT commands so the final configuration clearly shows NAT Overload using ACL 1.

Cleanup commands on Cafe-RT1

enable
configure terminal

! Remove old Static NAT from the static NAT lab if present.
no ip nat inside source static 192.168.1.50 216.0.5.20
no ip nat inside source static 192.168.1.51 216.0.5.21

! Remove old Dynamic NAT from the dynamic NAT lab if present.
no ip nat inside source list 10 pool CAFE_POOL overload
no ip nat inside source list 10 pool CAFE_POOL
no access-list 10
no ip nat pool CAFE_POOL 216.0.5.20 216.0.5.21 netmask 255.255.255.0

end
clear ip nat translation *

Beginner note: If a no command reports that the line does not exist, that is OK. The purpose is to remove old NAT statements before adding the new overload rule.

# Task 8 - Configure NAT Overload on Cafe-RT1

Objective: Use access-list 1 to identify inside addresses, create the public pool, and apply the NAT Overload rule. The overload keyword is the key difference from pure Dynamic NAT.

Cafe-RT1 NAT Overload configuration

enable
configure terminal

! Confirm NAT zone placement.
interface gigabitethernet0/1
 ip nat inside
 exit
interface gigabitethernet0/0
 ip nat outside
 exit

! ACL 1 matches the inside LAN.
access-list 1 permit 192.168.1.0 0.0.0.255

! Public pool assigned by the simulated ISP.
ip nat pool CAFE_POOL 216.0.5.20 216.0.5.21 netmask 255.255.255.0

! NAT Overload / PAT rule.
ip nat inside source list 1 pool CAFE_POOL overload

end
copy running-config startup-config

Command note: Use the Cisco IOS keyword overload at the end of the NAT command. This changes the rule from pure dynamic NAT into PAT/NAT Overload.

# Task 9 - Restore Cafe-RT1 Outside Connectivity

Objective: Bring the WAN interface back up and verify that Cafe-RT1 can reach ISP-RT again.

Restore Cafe-RT1 outside interface

enable
configure terminal
interface gigabitethernet0/0
 no shutdown
 exit
end

Cafe-RT1 link validation

show ip interface brief
ping 216.0.5.1
ping 1.1.1.1
ping 8.8.8.8

Beginner note: If ping 216.0.5.1 fails after no shutdown, check the cable between Cafe-RT1 G0/0 and ISP-RT G0/0. Both sides must be up/up before NAT can be tested.

# Task 10 - Validate NAT Overload from Inside Hosts

Objective: Generate traffic from inside devices and prove that Cafe-RT1 creates PAT translations.

From PC2 command prompt

ipconfig
ping 192.168.1.1
ping 216.0.5.1
ping 1.1.1.1
ping 8.8.8.8

From PC1 command prompt

ipconfig
ping 192.168.1.1
ping 216.0.5.1
ping 1.1.1.1
ping 8.8.8.8

On Cafe-RT1 after the pings

show ip nat translations
show ip nat statistics
show access-lists 1
show running-config | include ip nat|access-list

| Expected observation | What it means |
| --- | --- |
| PC1 and PC2 can ping 1.1.1.1 and 8.8.8.8. | Routing and NAT Overload are working from inside to outside. |
| show ip nat translations shows port numbers after the inside global address. | PAT is using ports to allow multiple flows through the public pool. |
| show access-lists 1 shows match counts increasing. | Inside LAN traffic is matching ACL 1. |
| show ip nat statistics shows hits increasing. | Cafe-RT1 is translating traffic. |

Example show ip nat translations output

Pro  Inside global        Inside local         Outside local       Outside global
icmp 216.0.5.20:100     192.168.1.50:100    1.1.1.1:100       1.1.1.1:100
icmp 216.0.5.20:101     192.168.1.51:101    8.8.8.8:101       8.8.8.8:101
icmp 216.0.5.21:102     192.168.1.2:102     1.1.1.1:102       1.1.1.1:102

# Task 11 - Prove NAT Overload Is Different from Dynamic NAT

Objective: Compare the result to the previous Dynamic NAT lab. With pure Dynamic NAT, two public addresses can support only two active inside translations. With NAT Overload, many sessions can share public addresses by adding port numbers.

Reset the NAT table on Cafe-RT1

clear ip nat translation *
show ip nat translations

From PC1 command prompt

ping 1.1.1.1 -t

From PC2 command prompt

ping 8.8.8.8 -t

On Cafe-RT1 while pings are running

show ip nat translations
show ip nat statistics
show access-lists 1

Beginner note: In Packet Tracer PC command prompt, stop a continuous ping with Ctrl+C.

# Task 12 - Optional Interface-Based Overload Method

Objective: See the alternate CCNA command that overloads directly on the outside interface IP address instead of using a pool. Do not configure this section unless your instructor asks for it.

Optional interface-based PAT command

configure terminal
no ip nat inside source list 1 pool CAFE_POOL overload
ip nat inside source list 1 interface gigabitethernet0/0 overload
end
clear ip nat translation *

Optional note: If you use this optional method, inside devices will translate to 216.0.5.2, which is Cafe-RT1 G0/0. ISP-RT does not need return routes to 216.0.5.20 or 216.0.5.21 for this method.

# Troubleshooting Checklist

If PC1 or PC2 cannot ping 192.168.1.1, fix the LAN first. Check PC IP, default gateway, switch port, and Cafe-RT1 G0/1.

If Cafe-RT1 cannot ping 216.0.5.1, fix the WAN link before troubleshooting NAT. Check Cafe-RT1 G0/0 and ISP-RT G0/0 are up/up.

If G0/0 shows administratively down, enter interface g0/0 and use no shutdown.

If show ip nat translations is empty, generate traffic from an inside PC to 1.1.1.1 or 8.8.8.8.

If show access-lists 1 shows no matches, verify ACL 1 permits 192.168.1.0 0.0.0.255.

If show ip nat statistics shows no inside or outside interfaces, verify ip nat inside on G0/1 and ip nat outside on G0/0.

If the NAT command fails, verify access-list 1 and CAFE_POOL, then re-enter the overload command.

If NAT still fails after cleanup, run clear ip nat translation * and test again from PC1 or PC2.

# Useful Verification Commands

| Command | Why you use it |
| --- | --- |
| show ip interface brief | Checks IP addresses and up/up status. |
| show ip route | Confirms connected routes and the default route. |
| show running-config ¦ include ip nat¦access-list | Shows NAT statements and ACL quickly. |
| show running-config interface g0/0 | Verifies Cafe-RT1 WAN interface and ip nat outside. |
| show running-config interface g0/1 | Verifies Cafe-RT1 LAN interface and ip nat inside. |
| show ip nat translations | Displays current NAT translations and port numbers. |
| show ip nat statistics | Shows inside/outside interfaces, hits, misses, and pool use. |
| show access-lists 1 | Shows ACL match counters for inside LAN traffic. |
| clear ip nat translation * | Clears existing NAT entries so you can retest. |

# Lab Completion Key

Use this section after completing the lab to check your work.

Cafe-LAN-SW VLAN 1 is 192.168.1.2/24 with default gateway 192.168.1.1.

PC2 is 192.168.1.50/24 and PC1 is 192.168.1.51/24. Both use gateway 192.168.1.1.

Cafe-RT1 G0/1 is 192.168.1.1/24 and is marked ip nat inside.

Cafe-RT1 G0/0 is 216.0.5.2/30 and is marked ip nat outside.

ISP-RT G0/0 is 216.0.5.1/30 with loopbacks 1.1.1.1/32 and 8.8.8.8/32.

Cafe-RT1 has a default route to 216.0.5.1.

ISP-RT has host routes for 216.0.5.20 and 216.0.5.21 via 216.0.5.2.

Cafe-RT1 uses access-list 1 and ip nat inside source list 1 pool CAFE_POOL overload.

PC1 and PC2 can ping 1.1.1.1 and 8.8.8.8.

show ip nat translations displays port-based translations.

# Complete Cafe-RT1 Answer Configuration

hostname Cafe-RT1
!
username cisco secret cisco
enable secret CrC0ffee!
service password-encryption
!
line console 0
 login local
 logging synchronous
!
line vty 0 4
 login local
!
interface gigabitethernet0/1
 description Default gateway for LAN network 192.168.1.0/24
 ip address 192.168.1.1 255.255.255.0
 ip nat inside
 no shutdown
!
interface gigabitethernet0/0
 description To ISP-RT - WAN outside
 ip address 216.0.5.2 255.255.255.252
 ip nat outside
 no shutdown
!
access-list 1 permit 192.168.1.0 0.0.0.255
ip nat pool CAFE_POOL 216.0.5.20 216.0.5.21 netmask 255.255.255.0
ip nat inside source list 1 pool CAFE_POOL overload
!
ip route 0.0.0.0 0.0.0.0 216.0.5.1

# Student Understanding Questions

1. What Cisco IOS keyword enables PAT/NAT Overload?

2. Which Cafe-RT1 interface is NAT inside, and why?

3. Which Cafe-RT1 interface is NAT outside, and why?

4. What does access-list 1 permit 192.168.1.0 0.0.0.255 select?

5. What is the main difference between Dynamic NAT and NAT Overload/PAT?

6. Why did connectivity fail when G0/0 was shut down?

7. Which command shows whether ACL 1 is matching packets?

8. Which command shows the current NAT translations?

9. Why does a NAT overload translation show port numbers?

10. If PC1 can ping 192.168.1.1 but not 1.1.1.1, what are two things to check?

# Answer Key

1. The correct keyword is overload.

2. G0/1 is NAT inside because it faces the private LAN 192.168.1.0/24.

3. G0/0 is NAT outside because it faces ISP-RT and the outside network 216.0.5.0/30.

4. It selects the inside LAN addresses that are allowed to be translated.

5. Dynamic NAT uses one temporary public address per inside host. NAT Overload/PAT allows multiple sessions to share public addresses by using port numbers.

6. The outside interface is the path to ISP-RT. If it is shut down, no outside traffic can leave Cafe-RT1.

7. show access-lists 1

8. show ip nat translations

9. PAT uses ports to identify different sessions that share the same public address.

10. Check Cafe-RT1 G0/0 is up/up and ip nat outside, check the default route to 216.0.5.1, check ACL 1, and check the NAT overload rule.