Skill 10 - Lesson 04 - Configuring Static NAT

Beginner CCNA Packet Tracer Lab: Static NAT on Cafe01-RT01

Figure 1. Static NAT topology with Cafe01-RT01 connected to ISP-RT and Fallout-RT01 as an inside routed network.

# 1. Field briefing

Coffee House Beta has a Coffee Shop LAN and a Fallout Shelter LAN behind Cafe01-RT01. Cafe01-RT01 is the only router that connects to the simulated Internet, so Cafe01-RT01 is the only router that performs NAT. Fallout-RT01 remains an inside LAN router only. In this lab, you will create static one-to-one NAT mappings on Cafe01-RT01 so the simulated ISP can reach selected inside servers by using public NAT addresses.

Note: Static NAT creates a permanent one-to-one mapping. The public address is not configured directly on the inside server. It is configured as a NAT inside-global address on Cafe01-RT01.

# 2. Learning objectives

Identify NAT inside and NAT outside interfaces on Cafe01-RT01.

Keep Fallout-RT01 as an inside router with routing only; do not configure NAT on Fallout-RT01.

Configure ISP-RT with loopbacks 1.1.1.1 and 8.8.8.8 as simulated Internet targets.

Configure static NAT on Cafe01-RT01 for one Coffee Shop server and one Fallout Shelter server.

Verify static routes, NAT translations, inside-to-outside pings, and ISP-to-inside public NAT tests.

Practice removing static NAT entries and static routes safely.

# 3. Addressing plan

| Device | Interface | IP address | Mask | Gateway | Role |
| --- | --- | --- | --- | --- | --- |
| PC1 | Fa0 | 192.168.1.51 | 255.255.255.0 | 192.168.1.1 | Coffee Shop PC |
| Laptop1 | Wireless0 | 192.168.1.16 | 255.255.255.0 | 192.168.1.1 | Coffee Shop wireless client |
| Server | Fa0 | 192.168.1.5 | 255.255.255.0 | 192.168.1.1 | Coffee Shop server |
| Laptop2 | Wireless0 | 192.168.1.101 | 255.255.255.0 | 192.168.1.1 | Coffee Shop wireless client |
| Cafe01-SW01 | VLAN 1 | 192.168.1.11 | 255.255.255.0 | 192.168.1.1 | Switch management |
| Cafe01-RT01 | G0/0 | 192.168.1.1 | 255.255.255.0 | - | Coffee Shop gateway / NAT inside |
| Cafe01-RT01 | G0/1 | 216.0.5.2 | 255.255.255.252 | - | WAN to ISP-RT / NAT outside |
| Cafe01-RT01 | G0/3/0 | 10.0.0.1 | 255.255.255.252 | - | WAN to Fallout-RT01 / NAT inside |
| ISP-RT | G0/0 | 216.0.5.1 | 255.255.255.252 | - | ISP gateway |
| ISP-RT | Loopback0 | 1.1.1.1 | 255.255.255.255 | - | Simulated Internet target |
| ISP-RT | Loopback1 | 8.8.8.8 | 255.255.255.255 | - | Simulated Internet target |
| Fallout-RT01 | G0/2/0 | 10.0.0.2 | 255.255.255.252 | - | WAN to Cafe01-RT01 |
| Fallout-RT01 | G0/0 | 192.168.3.1 | 255.255.255.0 | - | Fallout gateway |
| Fallout-SW01 | VLAN 1 | 192.168.3.2 | 255.255.255.0 | 192.168.3.1 | Switch management |
| Server02 | Fa0 | 192.168.3.5 | 255.255.255.0 | 192.168.3.1 | Fallout server |
| PC02 | Fa0 | 192.168.3.51 | 255.255.255.0 | 192.168.3.1 | Fallout PC |

# 4. Static NAT mapping plan

| Inside device | Inside local address | Inside global address | Purpose |
| --- | --- | --- | --- |
| Coffee Shop Server | 192.168.1.5 | 216.0.5.20 | ISP users reach the Coffee Shop server by pinging 216.0.5.20. |
| Fallout Server02 | 192.168.3.5 | 216.0.5.21 | ISP users reach the Fallout server by pinging 216.0.5.21. |

Important: 216.0.5.20 and 216.0.5.21 are not placed on physical interfaces. They are static NAT inside-global addresses. ISP-RT uses static routes to send those public addresses back to Cafe01-RT01 at 216.0.5.2.

# 5. Static NAT terms in this lab

| Term | Meaning |
| --- | --- |
| Inside local | The real private address of an inside device, such as 192.168.1.5 or 192.168.3.5. |
| Inside global | The public NAT address that represents an inside device to the Internet, such as 216.0.5.20 or 216.0.5.21. |
| Outside global | The real outside address, such as ISP loopback 1.1.1.1 or 8.8.8.8. |
| NAT inside interface | An interface facing an inside network. On Cafe01-RT01, G0/0 and G0/3/0 are NAT inside. |
| NAT outside interface | The interface facing the ISP. On Cafe01-RT01, G0/1 is NAT outside. |

# 6. Task 1 - Validate the baseline before NAT

Before adding NAT, make sure basic routing works. NAT cannot fix a broken LAN, wrong default gateway, or down WAN interface.

Baseline verification commands

PC1> ipconfig PC1> ping 192.168.1.1 PC1> ping 10.0.0.1 Server> ipconfig Server> ping 192.168.1.1 Server02> ipconfig Server02> ping 192.168.3.1 Cafe01-RT01# show ip interface brief Cafe01-RT01# ping 10.0.0.2 Cafe01-RT01# ping 216.0.5.1 Fallout-RT01# show ip interface brief Fallout-RT01# ping 10.0.0.1

Note: If PC1 can ping 10.0.0.1 but cannot ping 10.0.0.2, check Fallout-RT01 and make sure it has a route back to 192.168.1.0/24. For this lab, Fallout-RT01 should usually use a default route to 10.0.0.1.

# 7. Task 2 - Configure Cafe01-RT01 baseline routing

Cafe01-RT01 baseline configuration

enable configure terminal hostname Cafe01-RT01 no ip domain-lookup username cisco secret cisco enable secret CrC0ffee! service password-encryption line console 0 login local logging synchronous exit line vty 0 4 login local exit interface gigabitethernet0/0 description Coffee Shop LAN - NAT inside ip address 192.168.1.1 255.255.255.0 no shutdown exit interface gigabitethernet0/1 description To ISP-RT - NAT outside ip address 216.0.5.2 255.255.255.252 no shutdown exit interface gigabitethernet0/3/0 description WAN to Fallout-RT01 - NAT inside ip address 10.0.0.1 255.255.255.252 no shutdown exit ! Route to the inside Fallout LAN ip route 192.168.3.0 255.255.255.0 10.0.0.2 ! Default route toward the ISP ip route 0.0.0.0 0.0.0.0 216.0.5.1 end copy running-config startup-config

# 8. Task 3 - Configure Fallout-RT01 as an inside LAN router only

Note: Do not configure NAT on Fallout-RT01. It remains an inside router. Its job is to route the 192.168.3.0/24 Fallout LAN toward Cafe01-RT01.

Fallout-RT01 inside routing configuration

enable configure terminal hostname Fallout-RT01 no ip domain-lookup username cisco secret cisco enable secret CrC0ffee! service password-encryption line console 0 login local logging synchronous exit line vty 0 4 login local exit interface gigabitethernet0/2/0 description WAN to Cafe01-RT01 ip address 10.0.0.2 255.255.255.252 no shutdown exit interface gigabitethernet0/0 description Fallout Shelter LAN ip address 192.168.3.1 255.255.255.0 no shutdown exit ! Send non-local traffic toward Cafe01-RT01 ip route 0.0.0.0 0.0.0.0 10.0.0.1 end copy running-config startup-config

# 9. Task 4 - Configure ISP-RT

Use this ISP-RT configuration for the simulated Internet router. It creates loopbacks for 1.1.1.1 and 8.8.8.8 and installs return routes for the static NAT public addresses.

ISP-RT configuration

enable configure terminal hostname ISP-RT username cisco secret cisco enable secret CrC0ffee! service password-encryption line console 0 login local logging synchronous exit line vty 0 4 login local exit interface gigabitethernet0/0 description To Cafe-RT G0/0 ip address 216.0.5.1 255.255.255.252 no shutdown exit interface loopback0 description Simulated Internet target 1 ip address 1.1.1.1 255.255.255.255 exit interface loopback1 description Simulated Internet target 2 ip address 8.8.8.8 255.255.255.255 exit ip route 216.0.5.20 255.255.255.255 216.0.5.2 ip route 216.0.5.21 255.255.255.255 216.0.5.2 end copy running-config startup-config

# 10. Task 5 - Configure Static NAT on Cafe01-RT01

This is the main lesson. Cafe01-RT01 has two NAT inside interfaces because it has two inside networks behind it: the Coffee Shop LAN on G0/0 and the Fallout routed network on G0/3/0. The ISP-facing interface G0/1 is NAT outside.

Static NAT configuration on Cafe01-RT01

enable configure terminal ! Mark the Coffee Shop LAN as NAT inside interface gigabitethernet0/0 ip nat inside exit ! Mark the Fallout routed network as NAT inside interface gigabitethernet0/3/0 ip nat inside exit ! Mark the ISP-facing interface as NAT outside interface gigabitethernet0/1 ip nat outside exit ! Static one-to-one NAT mappings ip nat inside source static 192.168.1.5 216.0.5.20 ip nat inside source static 192.168.3.5 216.0.5.21 end copy running-config startup-config

Beginner reminder: Only Server 192.168.1.5 and Server02 192.168.3.5 have static NAT in this lab. PC1, Laptop1, Laptop2, and PC02 will not be translated to the Internet unless you add more static NAT entries or configure PAT/dynamic NAT later.

# 11. Task 6 - Verify static NAT from inside devices

Inside-to-outside NAT verification

Server> ping 1.1.1.1 Server> ping 8.8.8.8 Server02> ping 1.1.1.1 Server02> ping 8.8.8.8 Cafe01-RT01# show ip nat translations Cafe01-RT01# show ip nat statistics Cafe01-RT01# show running-config ¦ include ip nat Cafe01-RT01# show running-config interface gigabitethernet0/0 Cafe01-RT01# show running-config interface gigabitethernet0/1 Cafe01-RT01# show running-config interface gigabitethernet0/3/0

| Expected field | Expected value |
| --- | --- |
| Static translation 1 | Inside global 216.0.5.20 maps to inside local 192.168.1.5 |
| Static translation 2 | Inside global 216.0.5.21 maps to inside local 192.168.3.5 |
| Inside interfaces | GigabitEthernet0/0 and GigabitEthernet0/3/0 |
| Outside interface | GigabitEthernet0/1 |

# 12. Task 7 - Verify static NAT from the ISP side

ISP-side verification

ISP-RT# ping 216.0.5.20 ISP-RT# ping 216.0.5.21 ISP-RT# traceroute 216.0.5.20 ISP-RT# traceroute 216.0.5.21 ISP-RT# show ip route 216.0.5.20 ISP-RT# show ip route 216.0.5.21

Expected result: ISP-RT should have host routes for 216.0.5.20/32 and 216.0.5.21/32 pointing to Cafe01-RT01 at 216.0.5.2. If the pings fail, verify the ISP static routes, Cafe01-RT01 NAT statements, and the inside server default gateways.

# 13. Task 8 - Remove Static NAT or static routes when needed

## Remove only the Static NAT entries

Remove the static NAT mappings

enable configure terminal no ip nat inside source static 192.168.1.5 216.0.5.20 no ip nat inside source static 192.168.3.5 216.0.5.21 end show ip nat translations show running-config ¦ include ip nat inside source static

## Remove NAT role statements from interfaces

Remove NAT inside/outside interface roles

enable configure terminal interface gigabitethernet0/0 no ip nat inside exit interface gigabitethernet0/3/0 no ip nat inside exit interface gigabitethernet0/1 no ip nat outside exit end show ip nat statistics

## Remove static routes

Remove static routes safely

! On Cafe01-RT01, remove the route to Fallout LAN only if the lab asks for it configure terminal no ip route 192.168.3.0 255.255.255.0 10.0.0.2 no ip route 0.0.0.0 0.0.0.0 216.0.5.1 end ! On ISP-RT, remove the NAT return routes if you are tearing down the NAT lab configure terminal no ip route 216.0.5.20 255.255.255.255 216.0.5.2 no ip route 216.0.5.21 255.255.255.255 216.0.5.2 end

Note: Do not confuse static NAT with static routes. Static NAT changes addresses. Static routes tell routers where to send traffic.

# 14. Troubleshooting checklist

If Server cannot ping 192.168.1.1, fix the Coffee Shop LAN before troubleshooting NAT.

If Server02 cannot ping 192.168.3.1, fix the Fallout LAN before troubleshooting NAT.

If Cafe01-RT01 cannot ping 216.0.5.1, fix the ISP link before troubleshooting NAT.

If Server02 cannot ping 1.1.1.1, check Fallout-RT01 default route to 10.0.0.1 and Cafe01-RT01 route to 192.168.3.0/24.

If ISP-RT cannot ping 216.0.5.20 or 216.0.5.21, check ISP return routes to 216.0.5.2.

If show ip nat translations does not show static entries, check the static NAT commands on Cafe01-RT01.

If NAT statistics show the wrong inside/outside interfaces, fix ip nat inside and ip nat outside placement.

# 15. Complete answer configuration - Cafe01-RT01

Cafe01-RT01 answer configuration

hostname Cafe01-RT01 ! interface gigabitethernet0/0 description Coffee Shop LAN - NAT inside ip address 192.168.1.1 255.255.255.0 ip nat inside no shutdown ! interface gigabitethernet0/1 description To ISP-RT - NAT outside ip address 216.0.5.2 255.255.255.252 ip nat outside no shutdown ! interface gigabitethernet0/3/0 description WAN to Fallout-RT01 - NAT inside ip address 10.0.0.1 255.255.255.252 ip nat inside no shutdown ! ip nat inside source static 192.168.1.5 216.0.5.20 ip nat inside source static 192.168.3.5 216.0.5.21 ip route 192.168.3.0 255.255.255.0 10.0.0.2 ip route 0.0.0.0 0.0.0.0 216.0.5.1

# 16. Complete answer configuration - Fallout-RT01

Fallout-RT01 answer configuration

hostname Fallout-RT01 ! interface gigabitethernet0/2/0 description WAN to Cafe01-RT01 ip address 10.0.0.2 255.255.255.252 no shutdown ! interface gigabitethernet0/0 description Fallout Shelter LAN ip address 192.168.3.1 255.255.255.0 no shutdown ! ! Fallout-RT01 remains an inside router. No NAT is configured here. ip route 0.0.0.0 0.0.0.0 10.0.0.1

# 17. Complete answer configuration - ISP-RT

ISP-RT answer configuration

hostname ISP-RT ! interface gigabitethernet0/0 description To Cafe-RT G0/0 ip address 216.0.5.1 255.255.255.252 no shutdown ! interface loopback0 description Simulated Internet target 1 ip address 1.1.1.1 255.255.255.255 ! interface loopback1 description Simulated Internet target 2 ip address 8.8.8.8 255.255.255.255 ! ip route 216.0.5.20 255.255.255.255 216.0.5.2 ip route 216.0.5.21 255.255.255.255 216.0.5.2

# 18. Final completion checklist

☐ Cafe01-RT01 G0/0 is 192.168.1.1/24 and marked ip nat inside.

☐ Cafe01-RT01 G0/3/0 is 10.0.0.1/30 and marked ip nat inside.

☐ Cafe01-RT01 G0/1 is 216.0.5.2/30 and marked ip nat outside.

☐ Cafe01-RT01 has a route to 192.168.3.0/24 through 10.0.0.2.

☐ Cafe01-RT01 has a default route through 216.0.5.1.

☐ Fallout-RT01 has a default route through 10.0.0.1 and no NAT configuration.

☐ ISP-RT has loopbacks 1.1.1.1 and 8.8.8.8.

☐ ISP-RT has host routes to 216.0.5.20 and 216.0.5.21 via 216.0.5.2.

☐ Cafe01-RT01 static NAT maps 192.168.1.5 to 216.0.5.20.

☐ Cafe01-RT01 static NAT maps 192.168.3.5 to 216.0.5.21.

☐ ISP-RT can ping 216.0.5.20 and 216.0.5.21.

# 19. Knowledge check questions

1. Which router performs NAT in this topology?

2. Why is Fallout-RT01 not configured for NAT?

3. Which Cafe01-RT01 interfaces are NAT inside?

4. Which Cafe01-RT01 interface is NAT outside?

5. What is the inside local address for the Coffee Shop server?

6. What is the inside global address for the Coffee Shop server?

7. Why does ISP-RT need routes to 216.0.5.20 and 216.0.5.21?

8. What command removes the static NAT mapping for Server02?

9. What is the difference between removing static NAT and removing a static route?

10. Why might PC1 fail to ping 1.1.1.1 even though Server can ping 1.1.1.1?

# 20. Knowledge check answer key

1. Cafe01-RT01 performs NAT because it is the router connected to the ISP / Internet.

2. Fallout-RT01 stays inside the private network. It routes 192.168.3.0/24 traffic to Cafe01-RT01 but does not translate addresses.

3. G0/0 and G0/3/0 are NAT inside on Cafe01-RT01.

4. G0/1 is NAT outside on Cafe01-RT01.

5. 192.168.1.5 is the inside local address for the Coffee Shop server.

6. 216.0.5.20 is the inside global address for the Coffee Shop server.

7. Because 216.0.5.20 and 216.0.5.21 are not directly connected to ISP-RT. ISP-RT must send those public NAT addresses to Cafe01-RT01 at 216.0.5.2.

8. no ip nat inside source static 192.168.3.5 216.0.5.21

9. Removing static NAT removes an address translation. Removing a static route removes a path in the routing table.

10. Only Server and Server02 have static NAT in this lab. PC1 has no NAT mapping unless you add one or configure PAT/dynamic NAT.