Skill 10 - Lab 1 - Configuring Dynamic NAT

CCNA beginner practice lab: dynamic NAT between a Cafe LAN and a simulated ISP

Figure 1. Dynamic NAT topology using Cafe-LAN-SW, Cafe-RT, and ISP-RT.

# Field Briefing

Coffee House Beta has a small inside LAN using private addresses in 192.168.1.0/24. The cafe has been assigned a small public NAT pool, 216.0.5.20 through 216.0.5.21, by the simulated ISP. Your mission is to configure dynamic NAT on Cafe-RT so inside devices can borrow a public address from the pool when they initiate traffic to the simulated Internet targets 1.1.1.1 and 8.8.8.8.

Beginner note: This lab uses pure dynamic NAT, not PAT/NAT overload. Because the public pool has only two addresses, only two inside devices can have active translations at the same time. This is intentional so you can see how a NAT pool can run out.

# Lab Objectives

Build the Packet Tracer topology shown in Figure 1.

Configure usernames, passwords, and device hostnames on Cafe-LAN-SW, Cafe-RT, and ISP-RT.

Configure Cafe-LAN-SW for management access with VLAN 1 address 192.168.1.2/24.

Configure Cafe-RT LAN and WAN interfaces and identify the correct NAT inside and outside interfaces.

Configure ISP-RT as a simulated ISP with loopback targets 1.1.1.1 and 8.8.8.8.

Configure dynamic NAT using ACL 10 and a public pool of 216.0.5.20 to 216.0.5.21.

Verify routing, NAT translations, and NAT pool behavior with show and ping commands.

Explain inside local, inside global, outside local, and outside global addresses.

# Access Credentials

| Device access | Value |
| --- | --- |
| Console / VTY username | cisco |
| Console / VTY password | cisco |
| Privileged EXEC secret | CrC0ffee! |

# Addressing Plan

| Device | Interface | Address | Mask | Default gateway / Purpose |
| --- | --- | --- | --- | --- |
| PC2 | Fa0 | 192.168.1.50 | 255.255.255.0 | Gateway 192.168.1.1 / Inside host |
| PC1 | Fa0 | 192.168.1.51 | 255.255.255.0 | Gateway 192.168.1.1 / Inside host |
| Cafe-LAN-SW | VLAN 1 | 192.168.1.2 | 255.255.255.0 | Management SVI / Inside local |
| Cafe-RT | G0/1 | 192.168.1.1 | 255.255.255.0 | LAN default gateway / NAT inside |
| Cafe-RT | G0/0 | 216.0.5.2 | 255.255.255.252 | WAN toward ISP / NAT outside |
| ISP-RT | G0/0 | 216.0.5.1 | 255.255.255.252 | ISP gateway toward Cafe-RT |
| ISP-RT | Loopback0 | 1.1.1.1 | 255.255.255.255 | Simulated Internet target |
| ISP-RT | Loopback1 | 8.8.8.8 | 255.255.255.255 | Simulated Internet target |

# Dynamic NAT Plan

| NAT item | Configuration value | Meaning |
| --- | --- | --- |
| Inside local addresses | 192.168.1.0/24 | Private inside LAN addresses permitted by ACL 10. |
| Inside global pool | 216.0.5.20 - 216.0.5.21 | Public addresses borrowed temporarily by inside devices. |
| Pool name | CAFE_POOL | Name used in the ip nat pool command. |
| NAT type | Dynamic NAT | Temporary one-to-one mapping. No overload is used in this lab. |
| Outside global targets | 1.1.1.1 and 8.8.8.8 | Simulated Internet addresses on ISP-RT loopbacks. |

Beginner note: 216.0.5.20 and 216.0.5.21 are not configured directly on Cafe-RT interfaces. They are NAT pool addresses. ISP-RT must have routes pointing these addresses back to Cafe-RT at 216.0.5.2.

# Dynamic NAT Concepts

| Term | Meaning in this lab |
| --- | --- |
| Inside local | The real private IP on the inside LAN, such as 192.168.1.50, 192.168.1.51, or switch management 192.168.1.2. |
| Inside global | The public IP borrowed from the NAT pool, such as 216.0.5.20 or 216.0.5.21. |
| Outside local | How the outside host appears to the inside network. In this simple lab, it usually stays the same as the outside global address. |
| Outside global | The real outside address, such as 1.1.1.1 or 8.8.8.8. |
| Dynamic NAT | A temporary one-to-one translation from a private inside address to an available public pool address. |
| PAT / overload | Many-to-one NAT using ports. It is not used here because this lab is focused on pure dynamic NAT. |

# Task 1 - Build the Network Topology

Objective: Create the physical topology before configuring IP addresses and NAT.

Add two PCs, one switch named Cafe-LAN-SW, one router named Cafe-RT, and one router named ISP-RT.

Connect PC2 Fa0 to Cafe-LAN-SW Fa0/3.

Connect PC1 Fa0 to Cafe-LAN-SW Fa0/2.

Connect Cafe-LAN-SW Fa0/1 to Cafe-RT G0/1.

Connect Cafe-RT G0/0 to ISP-RT G0/0.

Use the interface names shown in your Packet Tracer file. If your router model uses a different interface number, follow your actual port labels.

# Task 2 - Configure PC Addresses

Objective: Give the inside hosts IP addresses and the correct default gateway.

| PC | IP address | Subnet mask | Default gateway |
| --- | --- | --- | --- |
| PC2 | 192.168.1.50 | 255.255.255.0 | 192.168.1.1 |
| PC1 | 192.168.1.51 | 255.255.255.0 | 192.168.1.1 |

Beginner note: In Packet Tracer, open the PC, go to Desktop > IP Configuration, and enter the values from the table.

# Task 3 - Configure Cafe-LAN-SW

Objective: Configure the switch hostname, local login, management IP address, default gateway, and port descriptions.

Cafe-LAN-SW baseline configuration

enable configure terminal hostname Cafe-LAN-SW username cisco secret cisco enable secret CrC0ffee! service password-encryption line console 0 login local logging synchronous exit line vty 0 15 login local exit interface vlan 1 description Management SVI ip address 192.168.1.2 255.255.255.0 no shutdown exit ip default-gateway 192.168.1.1 interface fastethernet0/1 description To Cafe-RT G0/1 switchport mode access no shutdown exit interface fastethernet0/2 description To PC1 Fa0 - 192.168.1.51 switchport mode access spanning-tree portfast no shutdown exit interface fastethernet0/3 description To PC2 Fa0 - 192.168.1.50 switchport mode access spanning-tree portfast no shutdown exit end copy running-config startup-config

Cafe-LAN-SW verification commands

show ip interface brief show running-config ¦ section line show running-config interface vlan 1 show interfaces description ping 192.168.1.1

# Task 4 - Configure Cafe-RT Interfaces and Default Route

Objective: Configure the inside LAN gateway, the outside WAN link, and the default route toward ISP-RT.

Cafe-RT baseline routing configuration

enable configure terminal hostname Cafe-RT username cisco secret cisco enable secret CrC0ffee! service password-encryption line console 0 login local logging synchronous exit line vty 0 4 login local exit interface gigabitethernet0/1 description Default gateway for LAN network 192.168.1.0/24 ip address 192.168.1.1 255.255.255.0 ip nat inside no shutdown exit interface gigabitethernet0/0 description To ISP-RT - WAN outside ip address 216.0.5.2 255.255.255.252 ip nat outside no shutdown exit ip route 0.0.0.0 0.0.0.0 216.0.5.1 end copy running-config startup-config

Beginner note: The default route means: send unknown Internet traffic to ISP-RT at 216.0.5.1. Cafe-RT should be 216.0.5.2. ISP-RT should be 216.0.5.1.

Cafe-RT baseline verification commands

show ip interface brief show ip route show running-config interface gigabitethernet0/0 show running-config interface gigabitethernet0/1 ping 216.0.5.1

# Task 5 - Configure ISP-RT as the Simulated ISP

Objective: Configure the ISP router, simulated Internet loopbacks, and return routes to the NAT pool.

ISP-RT configuration

enable configure terminal hostname ISP-RT username cisco secret cisco enable secret CrC0ffee! service password-encryption line console 0 login local logging synchronous exit line vty 0 4 login local exit interface gigabitethernet0/0 description To Cafe-RT G0/0 ip address 216.0.5.1 255.255.255.252 no shutdown exit interface loopback0 description Simulated Internet target 1 ip address 1.1.1.1 255.255.255.255 exit interface loopback1 description Simulated Internet target 2 ip address 8.8.8.8 255.255.255.255 exit ip route 216.0.5.20 255.255.255.255 216.0.5.2 ip route 216.0.5.21 255.255.255.255 216.0.5.2 end copy running-config startup-config

ISP-RT verification commands

show ip interface brief show ip route ping 216.0.5.2 show ip route 216.0.5.20 show ip route 216.0.5.21

# Task 6 - Configure Dynamic NAT on Cafe-RT

Objective: Remove any existing Static NAT from the previous lab, then configure an ACL, NAT pool, and pure Dynamic NAT rule on Cafe-RT.

Dynamic NAT cleanup and configuration on Cafe-RT

enable configure terminal ! Step 6A - Remove old Static NAT from the previous lab. ! These commands remove the one-to-one static NAT mappings. no ip nat inside source static 192.168.1.50 216.0.5.20 no ip nat inside source static 192.168.1.51 216.0.5.21 ! Optional cleanup if you practiced NAT before. no ip nat inside source list 10 pool CAFE_POOL overload no ip nat inside source list 10 pool CAFE_POOL no access-list 10 ! Step 6B - Configure pure Dynamic NAT. access-list 10 permit 192.168.1.0 0.0.0.255 ip nat pool CAFE_POOL 216.0.5.20 216.0.5.21 netmask 255.255.255.0 ip nat inside source list 10 pool CAFE_POOL end clear ip nat translation * copy running-config startup-config

Beginner note: If you are starting from a blank router, the 'no ip nat inside source static' commands may not remove anything. That is OK. The goal is to make sure old Static NAT entries are gone before Dynamic NAT is added. Do not add the word overload in this lab; overload changes the lab into PAT/NAT overload instead of pure Dynamic NAT.

Check the dynamic NAT configuration

show running-config ¦ include ip nat¦access-list show ip nat statistics show ip nat translations

# Task 7 - Test Dynamic NAT from Inside Hosts

Objective: Generate inside-to-outside traffic and verify that Cafe-RT creates dynamic NAT translations.

From PC2 command prompt

ipconfig ping 192.168.1.1 ping 216.0.5.1 ping 1.1.1.1 ping 8.8.8.8

From PC1 command prompt

ipconfig ping 192.168.1.1 ping 216.0.5.1 ping 1.1.1.1 ping 8.8.8.8

On Cafe-RT after the pings

show ip nat translations show ip nat statistics show access-lists 10

| Expected observation | What it means |
| --- | --- |
| PC1 and PC2 can ping 1.1.1.1 and 8.8.8.8. | Routing and NAT are working from inside to outside. |
| show ip nat translations shows 192.168.1.50 mapped to either 216.0.5.20 or 216.0.5.21. | PC2 borrowed one public address from the dynamic NAT pool. |
| show ip nat translations shows 192.168.1.51 mapped to the other public address. | PC1 borrowed the second public address from the dynamic NAT pool. |
| The exact inside global address assigned to each PC can vary. | Dynamic NAT chooses an available pool address. It is not permanently assigned like static NAT. |

Example show ip nat translations output

Pro Inside global Inside local Outside local Outside global --- 216.0.5.20 192.168.1.50 --- --- --- 216.0.5.21 192.168.1.51 --- --- icmp 216.0.5.20:1 192.168.1.50:1 1.1.1.1:1 1.1.1.1:1 icmp 216.0.5.21:2 192.168.1.51:2 8.8.8.8:2 8.8.8.8:2

# Task 8 - Observe NAT Pool Limitation

Objective: See what happens when more inside devices need NAT than the pool can support.

1.  Keep PC1 and PC2 translations active by pinging 1.1.1.1 or 8.8.8.8.

2.  From Cafe-LAN-SW, try ping 1.1.1.1 using the switch management address 192.168.1.2.

3.  If both NAT pool addresses are already being used, the switch ping may fail or may not create a new translation.

4.  On Cafe-RT, use show ip nat translations to confirm only two inside global addresses are available.

5.  Clear the NAT translations, then test the switch first.

Clear dynamic translations on Cafe-RT

clear ip nat translation * show ip nat translations

From Cafe-LAN-SW after clearing translations

ping 1.1.1.1 ping 8.8.8.8

Beginner note: This is why many small networks use PAT/NAT overload. Dynamic NAT needs one public address for each active inside translation. PAT allows many inside hosts to share one public address by using port numbers.

# Task 9 - Optional Outside-Side Test

Objective: Understand that dynamic NAT is not the same as static NAT for inbound access.

From ISP-RT before any inside traffic

ping 216.0.5.20 ping 216.0.5.21

Beginner note: Dynamic NAT does not create permanent inbound reachability. The ISP side usually cannot initiate a connection to an inside device until an inside device first creates a dynamic translation. For permanent inbound reachability to a server, use static NAT instead.

# Troubleshooting Checklist

If PC1 or PC2 cannot ping 192.168.1.1, fix the LAN addressing and cabling first.

If Cafe-RT cannot ping 216.0.5.1, fix the WAN link between Cafe-RT and ISP-RT.

If you see "%Invalid next hop address (it's this router)", you probably assigned 216.0.5.1 to Cafe-RT. Cafe-RT should use 216.0.5.2, and ISP-RT should use 216.0.5.1.

If Cafe-RT shows 216.0.5.22/30 on the WAN, change it to 216.0.5.2/30 for this lab. The 216.0.5.20 and 216.0.5.21 addresses are reserved for the NAT pool.

If Cafe-RT can ping 1.1.1.1 but the PCs cannot, check ip nat inside, ip nat outside, ACL 10, and the NAT pool command.

If show ip nat translations is empty, generate traffic from an inside device to 1.1.1.1 or 8.8.8.8.

If the third inside device cannot reach the outside, remember that the NAT pool only has two public addresses. Clear translations or add more pool addresses.

Do not configure 216.0.5.20 or 216.0.5.21 directly on a PC or router interface. They are NAT pool addresses.

# Useful Verification Commands

| Command | Why you use it |
| --- | --- |
| show ip interface brief | Checks IP addresses and up/up status. |
| show ip route | Confirms connected routes, static routes, and default route. |
| show running-config ¦ include ip nat¦access-list | Shows NAT statements and ACL 10 quickly. |
| show running-config interface g0/0 | Verifies Cafe-RT WAN interface and ip nat outside. |
| show running-config interface g0/1 | Verifies Cafe-RT LAN interface and ip nat inside. |
| show ip nat translations | Displays current dynamic translations. |
| show ip nat statistics | Shows inside/outside interfaces, pool use, and translation counts. |
| show access-lists 10 | Shows ACL matches for inside LAN traffic. |
| clear ip nat translation * | Clears dynamic translations so you can retest. |

# Lab Completion Key

Use this section after completing the lab to check your work.

## Answer for Task 1 - Topology

PC2 connects to Cafe-LAN-SW Fa0/3, PC1 connects to Fa0/2, Cafe-LAN-SW Fa0/1 connects to Cafe-RT G0/1, and Cafe-RT G0/0 connects to ISP-RT G0/0.

## Answer for Task 2 - PC Addressing

PC2 should be 192.168.1.50/24 with gateway 192.168.1.1. PC1 should be 192.168.1.51/24 with gateway 192.168.1.1.

## Answer for Task 3 - Switch

Cafe-LAN-SW should have VLAN 1 set to 192.168.1.2/24 and default gateway 192.168.1.1. Console and VTY lines should use login local with username cisco secret cisco.

## Answer for Task 4 - Cafe-RT Routing

Cafe-RT G0/1 should be 192.168.1.1/24 with ip nat inside. Cafe-RT G0/0 should be 216.0.5.2/30 with ip nat outside. Cafe-RT should have default route ip route 0.0.0.0 0.0.0.0 216.0.5.1.

## Answer for Task 5 - ISP-RT

ISP-RT G0/0 should be 216.0.5.1/30. Loopback0 should be 1.1.1.1/32. Loopback1 should be 8.8.8.8/32. ISP-RT should route 216.0.5.20/32 and 216.0.5.21/32 back to 216.0.5.2.

## Answer for Task 6 - Dynamic NAT

Cafe-RT should first remove the old static NAT mappings from the previous Static NAT lab. Then Cafe-RT should have ACL 10 permitting 192.168.1.0/24, NAT pool CAFE_POOL from 216.0.5.20 through 216.0.5.21, and ip nat inside source list 10 pool CAFE_POOL. The final configuration should not include static NAT mappings or overload.

# Complete Cafe-LAN-SW Answer Configuration

hostname Cafe-LAN-SW ! username cisco secret cisco enable secret CrC0ffee! service password-encryption ! line console 0 login local logging synchronous ! line vty 0 15 login local ! interface vlan 1 description Management SVI ip address 192.168.1.2 255.255.255.0 no shutdown ! ip default-gateway 192.168.1.1 ! interface fastethernet0/1 description To Cafe-RT G0/1 switchport mode access no shutdown ! interface fastethernet0/2 description To PC1 Fa0 - 192.168.1.51 switchport mode access spanning-tree portfast no shutdown ! interface fastethernet0/3 description To PC2 Fa0 - 192.168.1.50 switchport mode access spanning-tree portfast no shutdown

# Complete Cafe-RT Answer Configuration

hostname Cafe-RT ! username cisco secret cisco enable secret CrC0ffee! service password-encryption ! line console 0 login local logging synchronous ! line vty 0 4 login local ! interface gigabitethernet0/1 description Default gateway for LAN network 192.168.1.0/24 ip address 192.168.1.1 255.255.255.0 ip nat inside no shutdown ! interface gigabitethernet0/0 description To ISP-RT - WAN outside ip address 216.0.5.2 255.255.255.252 ip nat outside no shutdown ! access-list 10 permit 192.168.1.0 0.0.0.255 ip nat pool CAFE_POOL 216.0.5.20 216.0.5.21 netmask 255.255.255.0 ip nat inside source list 10 pool CAFE_POOL ! ip route 0.0.0.0 0.0.0.0 216.0.5.1

# Complete ISP-RT Answer Configuration

hostname ISP-RT ! username cisco secret cisco enable secret CrC0ffee! service password-encryption ! line console 0 login local logging synchronous ! line vty 0 4 login local ! interface gigabitethernet0/0 description To Cafe-RT G0/0 ip address 216.0.5.1 255.255.255.252 no shutdown ! interface loopback0 description Simulated Internet target 1 ip address 1.1.1.1 255.255.255.255 ! interface loopback1 description Simulated Internet target 2 ip address 8.8.8.8 255.255.255.255 ! ip route 216.0.5.20 255.255.255.255 216.0.5.2 ip route 216.0.5.21 255.255.255.255 216.0.5.2

# Final Completion Check

☐ PC2 is 192.168.1.50/24 with gateway 192.168.1.1.

☐ PC1 is 192.168.1.51/24 with gateway 192.168.1.1.

☐ Cafe-LAN-SW VLAN 1 is 192.168.1.2/24 and uses default gateway 192.168.1.1.

☐ Cafe-RT G0/1 is 192.168.1.1/24 and is marked ip nat inside.

☐ Cafe-RT G0/0 is 216.0.5.2/30 and is marked ip nat outside.

☐ ISP-RT G0/0 is 216.0.5.1/30.

☐ ISP-RT loopbacks are 1.1.1.1/32 and 8.8.8.8/32.

☐ Cafe-RT has a default route to 216.0.5.1.

☐ ISP-RT has host routes for 216.0.5.20 and 216.0.5.21 via 216.0.5.2.

☐ Cafe-RT has removed old static NAT entries and has ACL 10 plus NAT pool CAFE_POOL configured without overload.

☐ PC1 and PC2 can ping 1.1.1.1 and 8.8.8.8.

☐ show ip nat translations displays dynamic entries using 216.0.5.20 and 216.0.5.21.