Skill 10 – Lesson 05

# Change Static NAT to Dynamic NAT on Cafe01-RT01 Only

Beginner CCNA Packet Tracer Lab - Fallout-RT01 remains an inside LAN router

Figure 1. Network Coffee Shop and Fallout Shelter topology used for this lab. NAT is configured only on Cafe01-RT01 because it is the edge router connected to the Internet.

Corrected design: Only Cafe01-RT01 is converted from Static NAT to Dynamic NAT. Fallout-RT01 is not configured for NAT. It remains an inside router behind Cafe01-RT01, and it uses routing to reach the Internet through Cafe01-RT01.

# Learning objectives

Identify why Cafe01-RT01 is the only NAT edge router in this topology.

Remove old Static NAT statements from Cafe01-RT01 safely.

Configure Dynamic NAT on Cafe01-RT01 using a NAT pool and an ACL.

Mark both internal-facing paths on Cafe01-RT01 as NAT inside when needed.

Leave Fallout-RT01 as an inside LAN router with routing only.

Verify NAT translations, NAT statistics, routes, and end-to-end connectivity.

Practice removing Dynamic NAT and static routes using the no form of commands.

Vocabulary correction: The topic is NAT, not NAS. NAT means Network Address Translation. NAS usually means Network Attached Storage.

# 1. Topology and baseline addressing

This lab starts from the uploaded static-routing topology. The baseline network has two internal LANs and a WAN link between the routers. Cafe01-RT01 also has an Internet-facing interface, so it becomes the only NAT router.

Table 1. Baseline addressing and NAT roles.

| Area | Device / interface | Address / mask | Role in this NAT lab |
| --- | --- | --- | --- |
| Coffee Shop LAN | PC1 | 192.168.1.51 /24 | Internal host; default gateway 192.168.1.1 |
| Coffee Shop LAN | Server | 192.168.1.5 /24 | Internal server; default gateway 192.168.1.1 |
| Coffee Shop LAN | Cafe01-RT01 LAN | 192.168.1.1 /24 | NAT inside interface for Coffee Shop LAN |
| Router-to-router WAN | Cafe01-RT01 to Fallout-RT01 | 10.0.0.1 /30 | Internal path toward Fallout; mark NAT inside if Fallout must reach Internet |
| Router-to-router WAN | Fallout-RT01 to Cafe01-RT01 | 10.0.0.2 /30 | Fallout WAN interface; no NAT configured here |
| Fallout Shelter LAN | Fallout-RT01 LAN | 192.168.3.1 /24 | Inside gateway for Fallout LAN; no NAT configured here |
| Fallout Shelter LAN | PC02 | 192.168.3.51 /24 | Internal host; default gateway 192.168.3.1 |
| Internet side | Cafe01-RT01 Internet interface | Example: 216.0.5.2 /30 | NAT outside interface toward ISP/Internet |

Interface names may differ: Your Packet Tracer router may use GigabitEthernet0/0, GigabitEthernet0/1, GigabitEthernet0/3/0, Ethernet6, or another name. Always confirm with show ip interface brief before typing interface commands.

# 2. NAT design: only Cafe01-RT01 translates traffic

Table 2. NAT responsibility by device.

| Device | NAT role | What to configure |
| --- | --- | --- |
| Cafe01-RT01 | Edge NAT router | Remove Static NAT, configure Dynamic NAT, mark inside/outside interfaces, keep routes. |
| Fallout-RT01 | Inside router only | Do not configure NAT. Keep/adjust routing so Fallout LAN sends Internet traffic to Cafe01-RT01. |
| Coffee Shop LAN hosts | Inside local hosts | Use 192.168.1.1 as default gateway. |
| Fallout LAN hosts | Inside local hosts behind Fallout-RT01 | Use 192.168.3.1 as default gateway. Fallout-RT01 forwards unknown traffic to Cafe01-RT01. |
| ISP/Internet side | Outside network | Provides next-hop route and, if using a NAT pool, return routes back to Cafe01-RT01. |

Table 3. Cafe01-RT01 NAT inside and outside placement.

| Cafe01-RT01 interface direction | Example interface | NAT command | Why |
| --- | --- | --- | --- |
| Coffee Shop LAN-facing | G0/0 or G0/1 | ip nat inside | Traffic from 192.168.1.0/24 enters here before going to the Internet. |
| Fallout-facing WAN | G0/3/0 | ip nat inside | Traffic from 192.168.3.0/24 enters Cafe01-RT01 here before going to the Internet. |
| Internet-facing | Eth6 or Internet port | ip nat outside | Translated traffic exits here toward the ISP/Internet. |

Why the Fallout-facing interface is NAT inside: From the view of Cafe01-RT01, Fallout LAN traffic enters on the 10.0.0.1/30 link and exits toward the Internet. That makes the Fallout-facing interface an inside NAT interface on Cafe01-RT01. Fallout-RT01 itself still has no NAT commands.

# Task 1 - Verify baseline routing first

Do not troubleshoot NAT until basic routing works. Use the original static-routing baseline to verify LAN and WAN connectivity.

Router baseline verification

! On Cafe01-RT01 show ip interface brief show ip route show running-config ¦ include ip route ping 10.0.0.2 ! On Fallout-RT01 show ip interface brief show ip route show running-config ¦ include ip route ping 10.0.0.1

End-device baseline verification

! From PC1 command prompt ipconfig ping 192.168.1.1 ping 10.0.0.1 ping 10.0.0.2 ping 192.168.3.51 ! From PC02 command prompt ipconfig ping 192.168.3.1 ping 10.0.0.2 ping 10.0.0.1 ping 192.168.1.51

Previous troubleshooting lesson: If PC1 can ping 10.0.0.1 but not 10.0.0.2, Cafe01-RT01 is reachable but the router-to-router path or return route is not correct. Check Fallout-RT01 G0/2/0 and its route back to 192.168.1.0/24.

# Task 2 - Keep routing correct before NAT conversion

For Internet access, Cafe01-RT01 needs a default route to the ISP. Fallout-RT01 should point unknown traffic to Cafe01-RT01 because Fallout is an inside router.

Cafe01-RT01 routing needed for this NAT lab

! Cafe01-RT01 keeps a route back to Fallout LAN ip route 192.168.3.0 255.255.255.0 10.0.0.2 ! Cafe01-RT01 also needs a default route to the ISP/Internet ! Replace 216.0.5.1 with your real ISP next-hop if different. ip route 0.0.0.0 0.0.0.0 216.0.5.1

Fallout-RT01 routing only - no NAT commands

! Fallout-RT01 remains an inside router. ! Use a default route to send unknown traffic, including Internet traffic, to Cafe01-RT01. ip route 0.0.0.0 0.0.0.0 10.0.0.1 ! Optional if you prefer a specific route for the Coffee Shop LAN: ip route 192.168.1.0 255.255.255.0 10.0.0.1

Static route versus Static NAT: Static route commands begin with ip route. Static NAT commands begin with ip nat inside source static. They are different features. Removing Static NAT does not remove static routes.

# Task 3 - Find and remove old Static NAT on Cafe01-RT01

Do this only on Cafe01-RT01. Fallout-RT01 should not be converted to NAT in this corrected design.

Find old Static NAT commands

! On Cafe01-RT01 show running-config ¦ include ip nat inside source static show running-config ¦ include ip nat

Remove exact lines: The examples below may not match your file exactly. Use the exact IP addresses shown by your running configuration and put no in front of the command.

Cafe01-RT01: remove Static NAT entries

enable configure terminal ! Examples only - remove each old one-to-one Static NAT entry found on Cafe01-RT01. no ip nat inside source static 192.168.1.51 216.0.5.20 no ip nat inside source static 192.168.1.5 216.0.5.21 end clear ip nat translation * show running-config ¦ include ip nat inside source static

# Task 4 - Configure Cafe01-RT01 NAT inside and outside interfaces

The exact interface numbers depend on your Packet Tracer router. The command logic is the important part: LAN and Fallout-facing links are NAT inside; Internet-facing link is NAT outside.

Find the correct interfaces

! Confirm interface names and addresses first show ip interface brief

Cafe01-RT01: NAT zone configuration

enable configure terminal ! Coffee Shop LAN side - NAT inside interface gigabitethernet0/0 description Coffee Shop LAN - NAT inside ip nat inside no shutdown exit ! Fallout side - also NAT inside because Fallout is an internal network behind Cafe01-RT01 interface gigabitethernet0/3/0 description WAN to Fallout-RT01 - NAT inside ip nat inside no shutdown exit ! Internet side - NAT outside ! Replace ethernet6 with your actual Internet-facing interface name. interface ethernet6 description To ISP / Internet - NAT outside ip nat outside no shutdown exit end copy running-config startup-config

# Task 5 - Configure Dynamic NAT on Cafe01-RT01

Dynamic NAT requires three parts: an ACL to identify inside local addresses, a pool of inside global addresses, and a command that connects the ACL to the pool.

Table 4. Dynamic NAT address plan.

| NAT item | Example used in this lab | Meaning |
| --- | --- | --- |
| Inside local networks | 192.168.1.0/24 and 192.168.3.0/24 | Private internal networks that need Internet access. |
| Inside global pool | 216.0.5.20 - 216.0.5.21 | Public addresses temporarily borrowed by internal hosts. |
| Outside global targets | 1.1.1.1 and 8.8.8.8 | Simulated Internet destinations used for ping tests. |

Cafe01-RT01: Dynamic NAT configuration

enable configure terminal ! 1) Match both inside networks. ! Standard ACL 10 matches source addresses only. access-list 10 permit 192.168.1.0 0.0.0.255 access-list 10 permit 192.168.3.0 0.0.0.255 ! 2) Create a Dynamic NAT pool. ! Replace these public addresses with the pool assigned by your lab or ISP. ip nat pool CAFE_DYN_POOL 216.0.5.20 216.0.5.21 netmask 255.255.255.0 ! 3) Connect ACL 10 to the NAT pool. ip nat inside source list 10 pool CAFE_DYN_POOL end copy running-config startup-config

Pool size warning: This example pool has only two addresses. That means only two inside hosts can use Dynamic NAT at the same time. If a third host tries while both addresses are in use, translation can fail until one address is free.

# Task 6 - Make sure the ISP can return traffic to the NAT pool

The NAT pool addresses are not directly configured on the Cafe01-RT01 interface. The ISP side must know that 216.0.5.20 and 216.0.5.21 are reachable through Cafe01-RT01.

ISP return routes for Cafe01-RT01 Dynamic NAT pool

! On the ISP router or simulated Internet router, if your lab uses one: ip route 216.0.5.20 255.255.255.255 216.0.5.2 ip route 216.0.5.21 255.255.255.255 216.0.5.2 ! If your outside interface or next hop uses different addresses, ! replace 216.0.5.2 with Cafe01-RT01's actual Internet-facing IP.

If you use Packet Tracer cloud only: Some beginner files use a simple cloud instead of a real ISP router. If you do not have an ISP router, focus your verification on PC pings and show ip nat translations on Cafe01-RT01.

# Task 7 - Verify Dynamic NAT from the Coffee Shop LAN

Generate traffic from a PC: Do not test Dynamic NAT by pinging from Cafe01-RT01 itself. NAT is created when traffic passes through the router from inside to outside.

Verify NAT for Coffee Shop LAN hosts

! On Cafe01-RT01, clear old temporary translations first. clear ip nat translation * ! From PC1 command prompt: ping 1.1.1.1 ping 8.8.8.8 ! Immediately on Cafe01-RT01: show ip nat translations show ip nat statistics show access-lists 10 show running-config ¦ include ip nat show ip route 0.0.0.0

Expected example for PC1 after pinging 1.1.1.1

Pro Inside global Inside local Outside local Outside global icmp 216.0.5.20:1 192.168.1.51:1 1.1.1.1:1 1.1.1.1:1

# Task 8 - Verify Dynamic NAT from the Fallout LAN

Fallout-RT01 still has no NAT commands. PC02 traffic travels from the Fallout LAN to Fallout-RT01, then to Cafe01-RT01, then Cafe01-RT01 performs NAT out to the Internet.

Verify NAT for Fallout LAN hosts through Cafe01-RT01

! On Fallout-RT01, verify it has a default route toward Cafe01-RT01. show ip route show running-config ¦ include ip route ! From PC02 command prompt: ping 192.168.3.1 ping 10.0.0.1 ping 1.1.1.1 ping 8.8.8.8 ! Immediately on Cafe01-RT01: show ip nat translations show ip nat statistics show access-lists 10

Expected example for PC02 after pinging 8.8.8.8

Pro Inside global Inside local Outside local Outside global icmp 216.0.5.21:1 192.168.3.51:1 8.8.8.8:1 8.8.8.8:1

# Task 9 - Troubleshooting checklist

If PC1 cannot ping 192.168.1.1, fix the Coffee Shop LAN before troubleshooting NAT.

If PC02 cannot ping 192.168.3.1, fix the Fallout LAN before troubleshooting NAT.

If PC02 can ping 192.168.3.1 but not 10.0.0.1, check Fallout-RT01 route to Cafe01-RT01 and the 10.0.0.0/30 link.

If Cafe01-RT01 cannot ping the ISP next hop, fix the Internet-facing interface or default route first.

If show ip nat translations is empty, generate traffic from a PC and run the command immediately after the ping.

If the ACL counter does not increase, ACL 10 is not matching the source network. Check access-list 10.

If translations appear but pings fail, check ISP return routes to 216.0.5.20 and 216.0.5.21.

If only two hosts work, remember the Dynamic NAT pool has only two public addresses.

If show ip nat status fails, use show ip nat statistics instead. Packet Tracer IOS may not support show ip nat status.

Quick troubleshooting command set

! Useful commands on Cafe01-RT01 show ip interface brief show running-config ¦ include ip nat show running-config ¦ include access-list show running-config ¦ include ip route show ip route show ip nat translations show ip nat statistics show access-lists clear ip nat translation * ! Useful commands on Fallout-RT01 show ip interface brief show ip route show running-config ¦ include ip route ping 10.0.0.1

# Task 10 - How to remove Dynamic NAT or static routes

Remove in the correct order: Remove the NAT rule first, then the NAT pool, then the ACL. Clear translations after removing NAT.

Remove Dynamic NAT from Cafe01-RT01

! On Cafe01-RT01 - remove Dynamic NAT configuration configure terminal no ip nat inside source list 10 pool CAFE_DYN_POOL no ip nat pool CAFE_DYN_POOL 216.0.5.20 216.0.5.21 netmask 255.255.255.0 no access-list 10 ! Optional: remove NAT labels from interfaces if you are fully resetting the lab interface gigabitethernet0/0 no ip nat inside exit interface gigabitethernet0/3/0 no ip nat inside exit interface ethernet6 no ip nat outside exit end clear ip nat translation * show running-config ¦ include ip nat

Remove Static NAT entries from Cafe01-RT01

! Remove old Static NAT lines if they still exist. ! Use the exact lines shown by your running config. configure terminal no ip nat inside source static 192.168.1.51 216.0.5.20 no ip nat inside source static 192.168.1.5 216.0.5.21 end show running-config ¦ include ip nat inside source static

How to remove static routes

! Static routes are removed with no ip route. ! Remove only when they are wrong or when the lab asks for cleanup. ! On Cafe01-RT01, remove route to Fallout LAN: configure terminal no ip route 192.168.3.0 255.255.255.0 10.0.0.2 end ! On Cafe01-RT01, remove default route to ISP: configure terminal no ip route 0.0.0.0 0.0.0.0 216.0.5.1 end ! On Fallout-RT01, remove default route toward Cafe01-RT01: configure terminal no ip route 0.0.0.0 0.0.0.0 10.0.0.1 end

# Completion checklist

[ ] Cafe01-RT01 has no old ip nat inside source static lines unless the lab asks for Static NAT.

[ ] Cafe01-RT01 Coffee Shop LAN interface is ip nat inside.

[ ] Cafe01-RT01 Fallout-facing interface is ip nat inside if Fallout LAN must reach the Internet.

[ ] Cafe01-RT01 Internet-facing interface is ip nat outside.

[ ] Cafe01-RT01 has ACL 10 permitting 192.168.1.0/24 and 192.168.3.0/24.

[ ] Cafe01-RT01 has NAT pool CAFE_DYN_POOL.

[ ] Cafe01-RT01 has ip nat inside source list 10 pool CAFE_DYN_POOL.

[ ] Cafe01-RT01 has a default route to the ISP/Internet.

[ ] Fallout-RT01 has no NAT commands and has a route/default route toward Cafe01-RT01.

[ ] PC1 can ping simulated Internet targets and creates NAT translations.

[ ] PC02 can ping simulated Internet targets and creates NAT translations on Cafe01-RT01.

[ ] All configurations are saved with copy running-config startup-config.

# Student understanding questions

Why is Cafe01-RT01 the only router configured for NAT in this corrected design?

Answer: ________________________________________________________________________________

________________________________________________________________________________

Why should Fallout-RT01 not have ip nat inside or ip nat outside commands in this lab?

Answer: ________________________________________________________________________________

________________________________________________________________________________

Which Cafe01-RT01 interfaces should be marked ip nat inside?

Answer: ________________________________________________________________________________

________________________________________________________________________________

Which Cafe01-RT01 interface should be marked ip nat outside?

Answer: ________________________________________________________________________________

________________________________________________________________________________

What command shows the old Static NAT entries before you remove them?

Answer: ________________________________________________________________________________

________________________________________________________________________________

How do you remove an old Static NAT entry?

Answer: ________________________________________________________________________________

________________________________________________________________________________

What does access-list 10 match in this lab?

Answer: ________________________________________________________________________________

________________________________________________________________________________

Why can show ip nat translations be empty even after NAT is configured correctly?

Answer: ________________________________________________________________________________

________________________________________________________________________________

Why is pinging from Cafe01-RT01 not the best Dynamic NAT test?

Answer: ________________________________________________________________________________

________________________________________________________________________________

What command shows NAT hits, misses, inside/outside interfaces, and pool usage?

Answer: ________________________________________________________________________________

________________________________________________________________________________

What happens if the Dynamic NAT pool has two addresses but three hosts try to use NAT at the same time?

Answer: ________________________________________________________________________________

________________________________________________________________________________

What is the command to remove the Cafe01-RT01 static route to the Fallout LAN?

Answer: ________________________________________________________________________________

________________________________________________________________________________

# Answer key

Answer key for self-check or instructor review.

| Question | Answer |
| --- | --- |
| 1 | Cafe01-RT01 is the edge router connected to the Internet, so it performs translation for internal networks. |
| 2 | Fallout-RT01 is an internal router only. It forwards traffic to Cafe01-RT01 and does not translate addresses. |
| 3 | The Coffee Shop LAN interface and the Fallout-facing WAN interface on Cafe01-RT01. |
| 4 | The Cafe01-RT01 Internet-facing interface, such as Ethernet6 in the topology. |
| 5 | show running-config ¦ include ip nat inside source static |
| 6 | Use the no form of the exact command, such as no ip nat inside source static 192.168.1.51 216.0.5.20. |
| 7 | It permits source networks 192.168.1.0/24 and 192.168.3.0/24 for Dynamic NAT. |
| 8 | Dynamic NAT entries are temporary and are created only when inside traffic passes through the router. ICMP entries may expire quickly. |
| 9 | Router-sourced traffic does not pass from an inside interface to an outside interface, so it may not create a NAT translation. |
| 10 | show ip nat statistics |
| 11 | The third host may fail to translate until one of the two pool addresses becomes free. |
| 12 | no ip route 192.168.3.0 255.255.255.0 10.0.0.2 |