# CCNA Student Guide
Roastery West Network Diagram, CLI Configuration, and Verification

This guide walks through the Packet Tracer design, switch configuration, endpoint mapping, and verification steps for a small switched network.

Figure 1. Roastery West reference topology and CLI summary.

# 1. Network design overview

RW-CORE-SW acts as the core switch and management anchor at 192.168.42.1/24.

RW-ACC-SW acts as the access switch and management node at 192.168.42.2/24.

A trunk link carries VLAN 42 between the two switches.

Three endpoints live in VLAN 42: BaristaPOS (192.168.42.37), InventoryStation (192.168.42.38), and ManagerConsole (192.168.42.39).

Both switches use default gateway 192.168.42.254 for management reachability.

# 2. Devices and cabling

| Device Name | Suggested Packet Tracer Device | Purpose |
| --- | --- | --- |
| RW-CORE-SW | 2960 switch (or equivalent) | Core switch |
| RW-ACC-SW | 2960 switch (or equivalent) | Access switch |
| BaristaPOS | PC-PT | Endpoint on access switch |
| InventoryStation | PC-PT | Endpoint on access switch |
| ManagerConsole | PC-PT | Endpoint on access switch |

Use copper straight-through cables for all switch-to-endpoint links. Use a copper crossover cable or auto-MDIX-capable link for the switch-to-switch uplink if your Packet Tracer device requires it.

# 3. Place and cable the devices

Place RW-CORE-SW near the top of the workspace and RW-ACC-SW below it.

Place BaristaPOS, InventoryStation, and ManagerConsole below RW-ACC-SW.

Connect RW-CORE-SW Fa0/1 to RW-ACC-SW Fa0/24 as the trunk uplink.

Connect BaristaPOS to RW-ACC-SW Fa0/3.

Connect InventoryStation to RW-ACC-SW Fa0/1.

Connect ManagerConsole to RW-ACC-SW Fa0/2.

Label the links in Packet Tracer so the topology matches the engineering diagram.

# 4. Access credentials

Console / VTY login: cisco / cisco

Privileged EXEC secret: CrC0ffee!

# 5. CLI configuration

Apply the following configuration on each switch. Adjust interface numbers if your Packet Tracer model uses different ports.

## RW-CORE-SW configuration

enable

configure terminal

hostname RW-CORE-SW

enable secret CrC0ffee!

!

username cisco secret cisco

!

line console 0

password cisco

login

line vty 0 15

password cisco

login

transport input telnet ssh

!

vlan 42

name ROASTERY-WEST

!

interface fa0/1

description Uplink to RW-ACC-SW

switchport mode trunk

switchport trunk allowed vlan 42

no shutdown

!

interface vlan 42

description Management SVI

ip address 192.168.42.1 255.255.255.0

no shutdown

!

ip default-gateway 192.168.42.254

end

write memory

## RW-ACC-SW configuration

enable

configure terminal

hostname RW-ACC-SW

enable secret CrC0ffee!

!

username cisco secret cisco

!

line console 0

password cisco

login

line vty 0 15

password cisco

login

transport input telnet ssh

!

vlan 42

name ROASTERY-WEST

!

interface fa0/24

description Uplink to RW-CORE-SW

switchport mode trunk

switchport trunk allowed vlan 42

no shutdown

!

interface fa0/1

description InventoryStation

switchport mode access

switchport access vlan 42

spanning-tree portfast

no shutdown

!

interface fa0/2

description ManagerConsole

switchport mode access

switchport access vlan 42

spanning-tree portfast

no shutdown

!

interface fa0/3

description BaristaPOS

switchport mode access

switchport access vlan 42

spanning-tree portfast

no shutdown

!

interface vlan 42

description Management SVI

ip address 192.168.42.2 255.255.255.0

no shutdown

!

ip default-gateway 192.168.42.254

end

write memory

# 6. Endpoint IP addressing

| Endpoint | IP Address | Mask | Gateway |
| --- | --- | --- | --- |
| BaristaPOS | 192.168.42.37 | 255.255.255.0 | 192.168.42.254 |
| InventoryStation | 192.168.42.38 | 255.255.255.0 | 192.168.42.254 |
| ManagerConsole | 192.168.42.39 | 255.255.255.0 | 192.168.42.254 |

In Packet Tracer PCs, open Desktop > IP Configuration and enter the values above.

# 7. Verification commands

## Switch verification

show ip interface brief

show interfaces trunk

show interfaces status

show running-config interface fa0/1

show running-config interface fa0/24

show mac address-table vlan 42

show arp vlan 42

ping 192.168.42.2 source vlan 42

## Endpoint verification

From BaristaPOS:

ping 192.168.42.38

ping 192.168.42.39

tracert 192.168.42.38

arp -a

From InventoryStation:

ping 192.168.42.37

ping 192.168.42.39

arp -a

# 8. Task workflow for the student

## Task 1 — Establish the core view

Log in to RW-CORE-SW and confirm the hostname prompt.

Identify the trunk toward RW-ACC-SW with show interfaces trunk.

Add or verify the description on the uplink interface.

Record the management SVI and default gateway with show ip interface brief and show running-config.

## Task 2 — Discover the access layer

Log in to RW-ACC-SW and confirm the hostname.

Use show interfaces status to find the active access ports.

Verify Fa0/1 is InventoryStation, Fa0/2 is ManagerConsole, and Fa0/3 is BaristaPOS.

Confirm the uplink trunk to RW-CORE-SW is up and allows VLAN 42.

## Task 3 — Trace each endpoint

Refresh ARP by pinging each endpoint from another node in the VLAN.

Use show arp vlan 42 to map IP addresses to MAC addresses.

Use show mac address-table vlan 42 to map each MAC address to the correct switch port.

Document the final result with IP, MAC, switch, interface, and description.

## Task 4 — Validate continuity

Verify all three workstation MAC addresses are still present in the MAC table.

Check that the trunk remains operational and has not changed state.

Summarize any anomalies such as a missing MAC, downed port, or wrong VLAN.

# 9. Documentation template

| IP Address | MAC Address | Switch | Interface | Description |
| --- | --- | --- | --- | --- |
| 192.168.42.37 |  | RW-ACC-SW | Fa0/3 | BaristaPOS |
| 192.168.42.38 |  | RW-ACC-SW | Fa0/1 | InventoryStation |
| 192.168.42.39 |  | RW-ACC-SW | Fa0/2 | ManagerConsole |

# 10. Expected answers to the engineering questions

The physical tie between RW-CORE-SW and RW-ACC-SW is a trunk uplink carrying VLAN 42.

RW-CORE-SW management IP is 192.168.42.1/24; RW-ACC-SW management IP is 192.168.42.2/24; both use gateway 192.168.42.254.

InventoryStation, ManagerConsole, and BaristaPOS should appear in RW-ACC-SW MAC table entries on Fa0/1, Fa0/2, and Fa0/3 after traffic is generated.

The senior engineer should receive a completed mapping table with IP, MAC, switch, interface, and description for each endpoint.

# 11. Final checklist

Both switch prompts show the correct hostnames.

VLAN 42 exists on both switches.

The trunk between RW-CORE-SW and RW-ACC-SW is up and carrying VLAN 42.

Management SVIs respond to pings between 192.168.42.1 and 192.168.42.2.

Each endpoint is reachable and appears in ARP and MAC tables.

Interface descriptions match the actual connected devices.

- RW-CORE-SW#show interface status

- Port         Name               Status       Vlan       Duplex  Speed Type

- Et0/0                           connected    trunk        full   auto 10/100/1000BaseTX

- Et0/1                           connected    42           full   auto 10/100/1000BaseTX

- Et0/2                           disabled     1            full   auto 10/100/1000BaseTX

- Et0/3                           disabled     1            full   auto 10/100/1000BaseTX

- RW-CORE-SW#show ip interface

- Ethernet0/0 is up, line protocol is up

- Inbound  access list is not set

- Internet protocol processing disabled

- Ethernet0/1 is up, line protocol is up

- Inbound  access list is not set

- Internet protocol processing disabled

- Ethernet0/2 is administratively down, line protocol is down

- Inbound  access list is not set

- Internet protocol processing disabled

- Ethernet0/3 is administratively down, line protocol is down

- Inbound  access list is not set

- Internet protocol processing disabled

- Vlan42 is up, line protocol is up

- Internet address is 192.168.42.1/24

- Broadcast address is 255.255.255.255

- Address determined by configuration file

- MTU is 1500 bytes

- Helper address is not set

- Directed broadcast forwarding is disabled

- Outgoing Common access list is not set

- Outgoing access list is not set

- Inbound Common access list is not set

- Inbound  access list is not set

- Proxy ARP is enabled

- Local Proxy ARP is disabled

- Security level is default

- Split horizon is enabled

- ICMP redirects are always sent

- ICMP unreachables are always sent

- ICMP mask replies are never sent

- IP fast switching is enabled

- IP Flow switching is disabled

- IP CEF switching is enabled

- IP CEF switching turbo vector

- IP Null turbo vector

- Associated unicast routing topologies:

- Topology "base", operation state is UP

- IP multicast fast switching is enabled

- IP multicast distributed fast switching is disabled

- IP route-cache flags are Fast, CEF

- Router Discovery is disabled

- IP output packet accounting is disabled

- IP access violation accounting is disabled

- TCP/IP header compression is disabled

- RTP/IP header compression is disabled

- Probe proxy name replies are disabled

- Policy routing is disabled

- Network address translation is disabled

- BGP Policy Mapping is disabled

- Input features: MCI Check

- IPv4 WCCP Redirect outbound is disabled

- IPv4 WCCP Redirect inbound is disabled

- IPv4 WCCP Redirect exclude is disabled

- IP Clear Dont Fragment is disabled

- RW-CORE-SW#show ip interface

- Ethernet0/0 is up, line protocol is up

- Inbound  access list is not set

- Internet protocol processing disabled

- Ethernet0/1 is up, line protocol is up

- Inbound  access list is not set

- Internet protocol processing disabled

- Ethernet0/2 is administratively down, line protocol is down

- Inbound  access list is not set

- Internet protocol processing disabled

- Ethernet0/3 is administratively down, line protocol is down

- Inbound  access list is not set

- Internet protocol processing disabled

- Vlan42 is up, line protocol is up

- Internet address is 192.168.42.1/24

- Broadcast address is 255.255.255.255

- Address determined by configuration file

- MTU is 1500 bytes

- Helper address is not set

- Directed broadcast forwarding is disabled

- Outgoing Common access list is not set

- Outgoing access list is not set

- Inbound Common access list is not set

- Inbound  access list is not set

- Proxy ARP is enabled

- Local Proxy ARP is disabled

- Security level is default

- Split horizon is enabled

- ICMP redirects are always sent

- ICMP unreachables are always sent

- ICMP mask replies are never sent

- IP fast switching is enabled

- IP Flow switching is disabled

- IP CEF switching is enabled

- IP CEF switching turbo vector

- IP Null turbo vector

- Associated unicast routing topologies:

- Topology "base", operation state is UP

- IP multicast fast switching is enabled

- IP multicast distributed fast switching is disabled

- IP route-cache flags are Fast, CEF

- Router Discovery is disabled

- IP output packet accounting is disabled

- IP access violation accounting is disabled

- TCP/IP header compression is disabled

- RTP/IP header compression is disabled

- Probe proxy name replies are disabled

- Policy routing is disabled

- Network address translation is disabled

- BGP Policy Mapping is disabled

- Input features: MCI Check

- IPv4 WCCP Redirect outbound is disabled

- IPv4 WCCP Redirect inbound is disabled

- IPv4 WCCP Redirect exclude is disabled

- IP Clear Dont Fragment is disabled

- RW-CORE-SW>show mac address-table

- Mac Address Table

- -------------------------------------------

- Vlan    Mac Address       Type        Ports

- ----    -----------       --------    -----

- 42    5254.0009.1354    DYNAMIC     Et0/0

- 42    5254.000d.8d09    DYNAMIC     Et0/1

- 42    5254.00c4.52b3    DYNAMIC     Et0/0

- 42    aabb.cc00.0100    DYNAMIC     Et0/0

- Total Mac Addresses for this criterion: 4

- RW-CORE-SW>ping 192.168.42.37

- Type escape sequence to abort.

- Sending 5, 100-byte ICMP Echos to 192.168.42.37, timeout is 2 seconds:

- !!!!!

- Success rate is 100 percent (5/5), round-trip min/avg/max = 1/1/1 ms

- RW-CORE-SW>ping 192.168.42.38

- Type escape sequence to abort.

- Sending 5, 100-byte ICMP Echos to 192.168.42.38, timeout is 2 seconds:

- !!!!!

- Success rate is 100 percent (5/5), round-trip min/avg/max = 1/1/1 ms

- RW-CORE-SW>ping 192.168.42.39

- Type escape sequence to abort.

- Sending 5, 100-byte ICMP Echos to 192.168.42.39, timeout is 2 seconds:

- !!!!!

- Success rate is 100 percent (5/5), round-trip min/avg/max = 1/1/1 ms

- RW-CORE-SW>show arp

- Protocol  Address          Age (min)  Hardware Addr   Type   Interface

- Internet  192.168.42.1            -   aabb.cc80.0200  ARPA   Vlan42

- Internet  192.168.42.37           0   5254.000d.8d09  ARPA   Vlan42

- Internet  192.168.42.38           0   5254.00c4.52b3  ARPA   Vlan42

- Internet  192.168.42.39           0   5254.0009.1354  ARPA   Vlan42

- RW-CORE-SW>ping 192.168.1.2

- Type escape sequence to abort.

- Sending 5, 100-byte ICMP Echos to 192.168.1.2, timeout is 2 seconds:

- .....

- Success rate is 0 percent (0/5)

- RW-CORE-SW>show arp

- Protocol  Address          Age (min)  Hardware Addr   Type   Interface

- Internet  192.168.42.1            -   aabb.cc80.0200  ARPA   Vlan42

- Internet  192.168.42.37           0   5254.000d.8d09  ARPA   Vlan42

- Internet  192.168.42.38           0   5254.00c4.52b3  ARPA   Vlan42

- Internet  192.168.42.39           0   5254.0009.1354  ARPA   Vlan42

RW-ACC-SW>enable

Password:

RW-ACC-SW#show interface status

Port         Name               Status       Vlan       Duplex  Speed Type

Et0/0                           connected    trunk        full   auto 10/100/1000BaseTX

Et0/1                           connected    42           full   auto 10/100/1000BaseTX

Et0/2                           connected    42           full   auto 10/100/1000BaseTX

Et0/3                           disabled     1            full   auto 10/100/1000BaseTX

RW-ACC-SW#show ip interface brief

Interface              IP-Address      OK? Method Status                Protocol

Ethernet0/0            unassigned      YES unset  up                    up

Ethernet0/1            unassigned      YES unset  up                    up

Ethernet0/2            unassigned      YES unset  up                    up

Ethernet0/3            unassigned      YES unset  administratively down down

Vlan42                 192.168.42.2    YES TFTP   up                    up

RW-ACC-SW#show interface

Ethernet0/0 is up, line protocol is up (connected)

Hardware is Ethernet, address is aabb.cc00.0100 (bia aabb.cc00.0100)

MTU 1500 bytes, BW 10000 Kbit/sec, DLY 1000 usec,

reliability 255/255, txload 1/255, rxload 1/255

Encapsulation ARPA, loopback not set

Keepalive set (10 sec)

Full-duplex, Auto-speed, media type is 10/100/1000BaseTX

input flow-control is off, output flow-control is unsupported

ARP type: ARPA, ARP Timeout 04:00:00

Last input 00:00:00, output 00:00:00, output hang never

Last clearing of "show interface" counters never

Input queue: 0/2000/0/0 (size/max/drops/flushes); Total output drops: 0

Queueing strategy: fifo

Output queue: 0/40 (size/max)

5 minute input rate 2000 bits/sec, 2 packets/sec

5 minute output rate 2000 bits/sec, 2 packets/sec

655 packets input, 75932 bytes, 0 no buffer

Received 35 broadcasts (0 multicasts)

0 runts, 0 giants, 0 throttles

0 input errors, 0 CRC, 0 frame, 0 overrun, 0 ignored

0 input packets with dribble condition detected

829 packets output, 96115 bytes, 0 underruns

Output 260 broadcasts (201 multicasts)

0 output errors, 0 collisions, 1 interface resets

0 unknown protocol drops

0 babbles, 0 late collision, 0 deferred

0 lost carrier, 0 no carrier

0 output buffer failures, 0 output buffers swapped out

Ethernet0/1 is up, line protocol is up (connected)

Hardware is Ethernet, address is aabb.cc00.0110 (bia aabb.cc00.0110)

MTU 1500 bytes, BW 10000 Kbit/sec, DLY 1000 usec,

reliability 255/255, txload 1/255, rxload 1/255

Encapsulation ARPA, loopback not set

Keepalive set (10 sec)

Full-duplex, Auto-speed, media type is 10/100/1000BaseTX

input flow-control is off, output flow-control is unsupported

ARP type: ARPA, ARP Timeout 04:00:00

Last input 00:00:00, output 00:00:00, output hang never

Last clearing of "show interface" counters never

Input queue: 0/2000/0/0 (size/max/drops/flushes); Total output drops: 0

Queueing strategy: fifo

Output queue: 0/40 (size/max)

5 minute input rate 0 bits/sec, 0 packets/sec

5 minute output rate 1000 bits/sec, 1 packets/sec

336 packets input, 39208 bytes, 0 no buffer

Received 15 broadcasts (0 multicasts)

0 runts, 0 giants, 0 throttles

0 input errors, 0 CRC, 0 frame, 0 overrun, 0 ignored

0 input packets with dribble condition detected

536 packets output, 63935 bytes, 0 underruns

Output 247 broadcasts (188 multicasts)

0 output errors, 0 collisions, 1 interface resets

0 unknown protocol drops

0 babbles, 0 late collision, 0 deferred

0 lost carrier, 0 no carrier

0 output buffer failures, 0 output buffers swapped out

Ethernet0/2 is up, line protocol is up (connected)

Hardware is Ethernet, address is aabb.cc00.0120 (bia aabb.cc00.0120)

MTU 1500 bytes, BW 10000 Kbit/sec, DLY 1000 usec,

reliability 255/255, txload 1/255, rxload 1/255

Encapsulation ARPA, loopback not set

Keepalive set (10 sec)

Full-duplex, Auto-speed, media type is 10/100/1000BaseTX

input flow-control is off, output flow-control is unsupported

ARP type: ARPA, ARP Timeout 04:00:00

Last input 00:00:00, output 00:00:00, output hang never

Last clearing of "show interface" counters never

Input queue: 0/2000/0/0 (size/max/drops/flushes); Total output drops: 0

Queueing strategy: fifo

Output queue: 0/40 (size/max)

5 minute input rate 0 bits/sec, 0 packets/sec

5 minute output rate 1000 bits/sec, 1 packets/sec

328 packets input, 38844 bytes, 0 no buffer

Received 2 broadcasts (0 multicasts)

0 runts, 0 giants, 0 throttles

0 input errors, 0 CRC, 0 frame, 0 overrun, 0 ignored

0 input packets with dribble condition detected

528 packets output, 63017 bytes, 0 underruns

Output 240 broadcasts (182 multicasts)

0 output errors, 0 collisions, 1 interface resets

0 unknown protocol drops

0 babbles, 0 late collision, 0 deferred

0 lost carrier, 0 no carrier

0 output buffer failures, 0 output buffers swapped out

Ethernet0/3 is administratively down, line protocol is down (disabled)

Hardware is Ethernet, address is aabb.cc00.0130 (bia aabb.cc00.0130)

MTU 1500 bytes, BW 10000 Kbit/sec, DLY 1000 usec,

reliability 255/255, txload 1/255, rxload 1/255

Encapsulation ARPA, loopback not set

Keepalive set (10 sec)

Full-duplex, Auto-speed, media type is 10/100/1000BaseTX

input flow-control is off, output flow-control is unsupported

ARP type: ARPA, ARP Timeout 04:00:00

Last input never, output never, output hang never

Last clearing of "show interface" counters never

Input queue: 0/2000/0/0 (size/max/drops/flushes); Total output drops: 0

Queueing strategy: fifo

Output queue: 0/40 (size/max)

5 minute input rate 0 bits/sec, 0 packets/sec

5 minute output rate 0 bits/sec, 0 packets/sec

0 packets input, 0 bytes, 0 no buffer

Received 0 broadcasts (0 multicasts)

0 runts, 0 giants, 0 throttles

0 input errors, 0 CRC, 0 frame, 0 overrun, 0 ignored

0 input packets with dribble condition detected

0 packets output, 0 bytes, 0 underruns

Output 0 broadcasts (0 multicasts)

0 output errors, 0 collisions, 1 interface resets

0 unknown protocol drops

0 babbles, 0 late collision, 0 deferred

0 lost carrier, 0 no carrier

0 output buffer failures, 0 output buffers swapped out

Vlan42 is up, line protocol is up , Autostate Enabled

Hardware is Ethernet SVI, address is aabb.cc80.0100 (bia aabb.cc80.0100)

Internet address is 192.168.42.2/24

MTU 1500 bytes, BW 1000000 Kbit/sec, DLY 10 usec,

reliability 255/255, txload 1/255, rxload 1/255

Encapsulation ARPA, loopback not set

Keepalive not supported

ARP type: ARPA, ARP Timeout 04:00:00

Last input 00:00:00, output never, output hang never

Last clearing of "show interface" counters never

Input queue: 0/375/0/0 (size/max/drops/flushes); Total output drops: 0

Queueing strategy: fifo

Output queue: 0/40 (size/max)

5 minute input rate 0 bits/sec, 0 packets/sec

5 minute output rate 0 bits/sec, 0 packets/sec

121 packets input, 31472 bytes, 0 no buffer

Received 108 broadcasts (0 IP multicasts)

0 runts, 0 giants, 0 throttles

0 input errors, 0 CRC, 0 frame, 0 overrun, 0 ignored

0 packets output, 0 bytes, 0 underruns

Output 0 broadcasts (0 IP multicasts)

0 output errors, 0 interface resets

0 unknown protocol drops

0 output buffer failures, 0 output buffers swapped out

RW-ACC-SW#show mac address-table

Mac Address Table

-------------------------------------------

Vlan    Mac Address       Type        Ports

----    -----------       --------    -----

42    5254.0009.1354    DYNAMIC     Et0/2

42    5254.000d.8d09    DYNAMIC     Et0/0

42    5254.00c4.52b3    DYNAMIC     Et0/1

42    aabb.cc80.0200    DYNAMIC     Et0/0

Total Mac Addresses for this criterion: 4

- RW-ACC-SW#ping 192.168.42.38

- Type escape sequence to abort.

- Sending 5, 100-byte ICMP Echos to 192.168.42.38, timeout is 2 seconds:

- .!!!!

- Success rate is 80 percent (4/5), round-trip min/avg/max = 1/1/1 ms

- RW-ACC-SW#ping 192.168.42.39

- Type escape sequence to abort.

- Sending 5, 100-byte ICMP Echos to 192.168.42.39, timeout is 2 seconds:

- .!!!!

- Success rate is 80 percent (4/5), round-trip min/avg/max = 1/1/1 ms

- RW-ACC-SW#show arp

- Protocol  Address          Age (min)  Hardware Addr   Type   Interface

- Internet  192.168.42.2            -   aabb.cc80.0100  ARPA   Vlan42

- Internet  192.168.42.37           3   5254.000d.8d09  ARPA   Vlan42

- Internet  192.168.42.38           0   5254.00c4.52b3  ARPA   Vlan42

- Internet  192.168.42.39           0   5254.0009.1354  ARPA   Vlan42

- RW-ACC-SW#