Skill 5 • Skill Lab 01 — District Shop Bring-Up

Field Briefing

Castle Rysen just unlocked District Shop Delta-7. The doors can’t open until its micro-campus network is online. Supply runners dropped two access switches and a WAN edge router; everything else is up to you. Jeremy’s message is blunt: “You’ve drilled the base configs, banner work, and clean wipes. This is deployment time. Get the district lit, keep it secure, and prove the management team can reach it from the bunker — nothing fancy, just the fundamentals”

Objectives

- Re-establish device identity, banners, and secrets across both switches and the router.

- Lock down console and VTY access with the Castle credential set and enable password encryption.

- Stand up the management VLAN on each switch and tie it to the router’s inside interface.

- Save, verify, and (when asked) wipe configurations without losing control of the site.

Task 1. Identity & Access Hardening

- Update each device so its hostname reflects DS-07-SW1, DS-07-SW2, and DS-07-RTR1.

- Ensure all three systems display the Castle warning banner text Castle Rysen Ops: Authorized engineers only. whenever someone connects.

- Protect privileged access with the shared secret CrC0ffee! and confirm stored passwords are hidden by the standard encryption feature.

Task 2. Console & VTY Security

- Require the console password VaultAccess before anyone can move past the initial prompt on every device.

DS-07-SW1(config)#line console 0

DS-07-SW1(config-line)#password VaultAccess

DS-07-SW1(config-line)#login

DS-07-SW1(config-line)#exit

- Guard the remote-access lines with the password ShelterAccess, limiting them to SSH sessions, and confirm both passwords are stored in encrypted form.

line vty 0 4

password ShelterAccess

login

transport input ssh

exit

show running-config |section line vty

Task 3. Switch Management Interfaces

- Assign DS-07-SW1 the management address 192.168.10.11/24, ensure the virtual interface is active, and document which physical ports face the router and the neighboring switch.

interface vlan1

ip address 192.168.11.24/24 255.255.255.0

no shutdown

show ip interface brief

- Assign DS-07-SW2 the management address 192.168.10.12/24, confirm the interface is active, and label the uplink along with the ports that feed the existing WAP and server drops.

interface vlan1

ip address 192.168.10.12/24 255.255.255.0

no shutdown

show ip interface brief

interface ethernet0.0

description uplink-to-core-distriubtion

end

show running-config

- Point both switches at 192.168.10.1 as their management gateway.

Task 4. Router Management Interface

- Place the router’s inside interface on 192.168.10.1/24, describing the link back to DS-07-SW1.

- Confirm the interface is brought online so switch traffic reaches the router through that connection.

Task 5. Save and Verify

- Compare the live configuration to the saved startup copy on each device to check whether the new banner, hostnames, and secrets have been preserved.

show running-config

- Commit the changes to non-volatile storage, repeat the comparison, and restart DS-07-SW1 to confirm the passwords still take effect (VaultAccess on the console, CrC0ffee! for privileged access).

show startup-config

copy running-config startup-config

show startup-configl

Completion Check

- Every prompt reads DS-07-*, the banner flashes Castle Rysen Ops: Authorized engineers only., and the enable secret reports as type 5 under show running-config.

Show running-config

- Both switches report the management interface as operational with addresses 192.168.10.11 and 192.168.10.12, and their management gateway is set to 192.168.10.1.

- The router lists 192.168.10.1/24 on its inside connection with the Castle security controls active.

- Saved configurations match the running state on every device, and DS-07-SW2 powers up with the hardened settings instead of reverting to the factory prompt.

Record the outputs in the Castle Rysen engineering log. District Shop Delta-7 now has a hardened transport and is ready for survivor traffic.

1. Device Selection List

To replicate this topology, grab the following devices from the bottom-left toolkit in Packet Tracer:

- Router: Choose a Cisco 1941 or ISR 4321 (Rename it Router).

- Switches: Choose two 3560-24PS Multi-layer Switches (Rename them Switch1 and Switch2).

- End Devices: * Two Generic Wireless Access Points (AP-PT) (Rename them WAP1 and WAP2).

  - One Generic Server (Rename it Server).

2. Physical Cabling Matrix

Use Copper Straight-Through cables (the solid black line tool) to connect the ports exactly as shown in the diagram:

| From Device | From Port | To Device | To Port |
| --- | --- | --- | --- |
| Router | Gig0/0 (or FastEth0/0) | Switch1 | Eth 0/3 |
| Router | Gig0/1 (or FastEth0/1) | Internet (Optional Cloud) | Any |
| Switch1 | Eth 0/1 | Switch2 | Eth 0/0 |
| Switch1 | Eth 0/0 | Switch2 | Eth 0/1 |
| Switch1 | Eth 0/2 | WAP1 | Port 0 |
| Switch2 | Eth 0/2 | WAP2 | Port 0 |
| Switch2 | Eth 0/3 | Server | FastEthernet 0 |

3. Quick Configuration Blueprint

Once you lay out the cables, you can apply the IP configuration you've been working on to mirror the lab:

- Router Inside Interface: 192.168.10.1 255.255.255.0

- Switch 1 Management (VLAN 1): 192.168.10.11 255.255.255.0 (Gateway: 192.168.10.1)

- Switch 2 Management (VLAN 1): 192.168.10.12 255.255.255.0 (Gateway: 192.168.10.1)