Skill 08 - Lab - Coffee House and Fallout Local Routing (June 04)

CCNA Packet Tracer Network Design, Configuration, and Path Verification Guide

Figure 1. Coffee House and Fallout local-routing topology. PC0 uses 192.168.1.51/24 on the 192.168.1.0/24 LAN.

Important addressing note: This lab uses PC0 address 192.168.1.51/24. The Coffee House LAN is 192.168.1.0/24, and the default gateway is 192.168.1.1.

# 1. Learning objectives

Place and cable two PCs, two Layer 2 switches, and two routers in Cisco Packet Tracer.

Configure two LANs and a /30 point-to-point router link.

Add static routes so PC0 can reach PC1 across both routers.

Verify Layer 1, Layer 2, and Layer 3 operation with show, ping, ARP, MAC-table, and traceroute commands.

Trace the packet path from PC0 (192.168.1.51) to PC1 (192.168.3.101).

# 2. Required devices

| Quantity | Packet Tracer device | Suggested model | Purpose |
| --- | --- | --- | --- |
| 2 | Router | ISR 4321 | Routes between the two LANs |
| 2 | Switch | 2960-24TT | Provides Layer 2 access connectivity |
| 2 | PC | PC-PT | PC0 and PC1 endpoints |
| 5 | Copper straight-through/automatic | Connections tool | PC-to-switch, switch-to-router, and router-to-router links |

# 3. Network design and addressing plan

| Device | Interface | IP address | Mask | Default gateway / next hop | Role |
| --- | --- | --- | --- | --- | --- |
| PC0 | Fa0 | 192.168.1.51 | 255.255.255.0 | 192.168.1.1 | Coffee House host |
| Cafe-LAN-SW | VLAN 1 | 192.168.1.2 | 255.255.255.0 | 192.168.1.1 | Optional switch management |
| Cafe-RT1 | G0/0/0 | 192.168.1.1 | 255.255.255.0 | — | Coffee House gateway |
| Cafe-RT1 | G0/0/1 | 10.0.0.1 | 255.255.255.252 | 10.0.0.2 | Router transit link |
| Fallout-RT1 | G0/0/1 | 10.0.0.2 | 255.255.255.252 | 10.0.0.1 | Router transit link |
| Fallout-RT1 | G0/0/0 | 192.168.3.1 | 255.255.255.0 | — | Fallout LAN gateway |
| Fallout-LAN-SW | VLAN 1 | 192.168.3.2 | 255.255.255.0 | 192.168.3.1 | Optional switch management |
| PC1 | Fa0 | 192.168.3.101 | 255.255.255.0 | 192.168.3.1 | Fallout host |

# 4. Place and cable the devices

Place the devices: Drag two ISR 4321 routers, two 2960 switches, and two PC-PT computers into the Logical workspace. Rename them PC0, Cafe-LAN-SW, Cafe-RT1, Fallout-RT1, Fallout-LAN-SW, and PC1.

Cable the Coffee House LAN: PC0 Fa0 to Cafe-LAN-SW Fa0/2; Cafe-LAN-SW Fa0/1 to Cafe-RT1 G0/0/0.

Cable the transit link: Cafe-RT1 G0/0/1 to Fallout-RT1 G0/0/1.

Cable the Fallout LAN: Fallout-RT1 G0/0/0 to Fallout-LAN-SW Fa0/1; Fallout-LAN-SW Fa0/2 to PC1 Fa0.

Wait for link lights: Allow several seconds for interfaces to transition. Router interfaces remain down until no shutdown is configured.

# 5. Configure the PCs

Packet Tracer PC-PT endpoints are normally configured through the graphical interface rather than Cisco IOS CLI.

### PC0 configuration

Open PC0 > Desktop > IP Configuration.

IP address: 192.168.1.51

Subnet mask: 255.255.255.0

Default gateway: 192.168.1.1

### PC1 configuration

Open PC1 > Desktop > IP Configuration.

IP address: 192.168.3.101

Subnet mask: 255.255.255.0

Default gateway: 192.168.3.1

### Verify each PC from Desktop > Command Prompt

ipconfig

# 6. Configure Cafe-LAN-SW

enable
configure terminal
hostname Cafe-LAN-SW
no ip domain-lookup

interface fastethernet0/2
 description PC0 - 192.168.1.51
 switchport mode access
 spanning-tree portfast
 no shutdown
 exit

interface fastethernet0/1
 description Uplink to Cafe-RT1 G0/0/0
 switchport mode access
 no shutdown
 exit

interface vlan 1
 description Switch management
 ip address 192.168.1.2 255.255.255.0
 no shutdown
 exit

ip default-gateway 192.168.1.1
end
copy running-config startup-config

# 7. Configure Fallout-LAN-SW

enable
configure terminal
hostname Fallout-LAN-SW
no ip domain-lookup

interface fastethernet0/1
 description Uplink to Fallout-RT1 G0/0/0
 switchport mode access
 no shutdown
 exit

interface fastethernet0/2
 description PC1 - 192.168.3.101
 switchport mode access
 spanning-tree portfast
 no shutdown
 exit

interface vlan 1
 description Switch management
 ip address 192.168.3.2 255.255.255.0
 no shutdown
 exit

ip default-gateway 192.168.3.1
end
copy running-config startup-config

# 8. Configure Cafe-RT1

enable
configure terminal
hostname Cafe-RT1
no ip domain-lookushow cppcop

interface gigabitethernet0/0/0
 description Coffee House LAN to Cafe-LAN-SW
 ip address 192.168.1.1 255.255.255.0
 no shutdown
 exit

interface gigabitethernet0/0/1
 description Point-to-point link to Fallout-RT1
 ip address 10.0.0.1 255.255.255.252
 no shutdown
 exit

ip route 192.168.3.0 255.255.255.0 10.0.0.2
end
copy running-config startup-config

# 9. Configure Fallout-RT1

enable
configure terminal
hostname Fallout-RT1
no ip domain-lookup

interface gigabitethernet0/0/0
 description Fallout LAN to Fallout-LAN-SW
 ip address 192.168.3.1 255.255.255.0
 no shutdown
 exit

interface gigabitethernet0/0/1
 description Point-to-point link to Cafe-RT1
 ip address 10.0.0.2 255.255.255.252
 no shutdown
 exit

ip route 192.168.1.0 255.255.255.0 10.0.0.1
end
copy running-config startup-config

# 10. Verify the design in layers

### A. Verify router interfaces

show ip interface brief
show interfaces description

Expected: G0/0/0 and G0/0/1 should report up/up on both routers.

### B. Verify routing tables

show ip route

Cafe-RT1 should have connected routes for 192.168.1.0/24 and 10.0.0.0/30, plus a static route to 192.168.3.0/24. Fallout-RT1 should show the reverse.

### C. Test each local segment

PC0> ping 192.168.1.1
PC0> ping 192.168.1.2
PC1> ping 192.168.3.1
PC1> ping 192.168.3.2

### D. Test the router transit link

Cafe-RT1# ping 10.0.0.2
Fallout-RT1# ping 10.0.0.1

### E. End-to-end ping

PC0> ping 192.168.3.101

The first ping can lose one packet while ARP entries are learned. Repeat the ping if necessary.

# 11. Trace the path from PC0 to PC1

On PC0, open Desktop > Command Prompt and run:

tracert 192.168.3.101

Expected logical path:

| Hop | Expected address | Device | Reason |
| --- | --- | --- | --- |
| 1 | 192.168.1.1 | Cafe-RT1 G0/0/0 | PC0 sends remote traffic to its default gateway. |
| 2 | 10.0.0.2 | Fallout-RT1 G0/0/1 | Cafe-RT1 forwards the packet across the /30 transit link. |
| 3 | 192.168.3.101 | PC1 | Fallout-RT1 forwards the packet onto the directly connected destination LAN. |

# 12. Inspect ARP and MAC-address learning

### On PC0 after the ping

arp -a

PC0 should learn the MAC address of 192.168.1.1, not PC1. Because PC1 is on a remote network, PC0 sends the frame to its default gateway.

### On the switches

show mac address-table
show interfaces status

The Coffee House switch should learn PC0 and Cafe-RT1 MAC addresses. The Fallout switch should learn PC1 and Fallout-RT1 MAC addresses.

### On the routers

show arp
show ip route
show cdp neighbors

# 13. Troubleshooting checklist

☐ PC0 uses 192.168.1.51/24 and gateway 192.168.1.1.

☐ PC1 uses 192.168.3.101/24 and gateway 192.168.3.1.

☐ All four router interfaces are configured with no shutdown.

☐ The inter-router addresses are in the same 10.0.0.0/30 subnet.

☐ Cafe-RT1 has a route to 192.168.3.0/24 through 10.0.0.2.

☐ Fallout-RT1 has a route to 192.168.1.0/24 through 10.0.0.1.

☐ Switch access ports and uplinks report connected.

☐ No duplicate IP addresses exist.

# 14. Completion check

☐ PC0 can ping 192.168.1.1 and PC1 can ping 192.168.3.1.

☐ The routers can ping each other across 10.0.0.0/30.

☐ PC0 can ping PC1 at 192.168.3.101.

☐ PC0 tracert shows Cafe-RT1, Fallout-RT1, and PC1 in that order.

☐ Router routing tables contain the expected connected and static routes.

☐ ARP and switch MAC tables contain entries learned from the test traffic.

☐ All configurations are saved to startup-config.