Skill 8 - Lesson 01 Lab - Configuring Switch Interfaces

1. Topology and Physical Installation Guidance

This installation standard governs the physical assembly and logical configuration of the Cafe infrastructure network. The environment consists of dual Layer 2 edge switches linking dedicated operational hardware (Point of Sale, HVAC systems, and physical security cameras).

Figure 1.1: Logical Connectivity Topology Mapping

Field Crew Wire Map & Switch Port Handoffs

Field deployment personnel must verify that patches match this explicit matrix identically. Any deviations must be updated in local documentation.

| Switch Device | Local Interface | Connected Node Destination | Remote Port | Cable Standard |
| --- | --- | --- | --- | --- |
| Cafe-SW1 | Ethernet 0/0 | Cafe-SW2 | Ethernet 0/0 | Copper Crossover (or Cat5e Auto-MDIX) |
| Cafe-SW1 | Ethernet 0/1 | BaristaPOS | FastEthernet 0 | Copper Straight-Through |
| Cafe-SW2 | Ethernet 0/0 | Cafe-SW1 | Ethernet 0/0 | Copper Crossover (or Cat5e Auto-MDIX) |
| Cafe-SW2 | Ethernet 0/1 | Cam01 | FastEthernet 0 | Copper Straight-Through |
| Cafe-SW2 | Ethernet 0/3 | Thermostat | FastEthernet 0 | Copper Straight-Through |

2. Infrastructure Device Provisioning Actions

Task 1: Link Labeling & Port Characteristics Verification

Step 1: Connect to Cafe-SW1, confirm the CLI prompt, and review active links:

Cafe-SW1# show interfaces status Cafe-SW1# show ip interface brief

Step 2: Implement localized string tags on Cafe-SW1 interfaces:

Cafe-SW1# configure terminal Cafe-SW1(config)# interface Ethernet0/0 Cafe-SW1(config-if)# description Uplink_to_Cafe-SW2 Cafe-SW1(config-if)# exit Cafe-SW1(config)# interface Ethernet0/1 Cafe-SW1(config-if)# description Handoff_to_BaristaPOS Cafe-SW1(config-if)# end

Step 3: Access Cafe-SW2 via console and label active access ports identically:

Cafe-SW2# configure terminal Cafe-SW2(config)# interface Ethernet0/0 Cafe-SW2(config-if)# description Uplink_to_Cafe-SW1 Cafe-SW2(config-if)# exit Cafe-SW2(config)# interface Ethernet0/1 Cafe-SW2(config-if)# description Handoff_to_Cam01_CameraFeed Cafe-SW2(config-if)# exit Cafe-SW2(config)# interface Ethernet0/3 Cafe-SW2(config-if)# description Circuit_to_Thermostat Cafe-SW2(config-if)# end

Step 4: Verify labels are properly active within the operational database table structure:

Cafe-SW1# show interfaces description Cafe-SW2# show interfaces description

Duplex Locking Configurations

To avoid errors from auto-negotiation, explicitly lock the duplex setting on inter-switch backplane connections and high-priority access paths.

Enforce full duplex properties on Cafe-SW1's trunk link:

Cafe-SW1(config)# interface Ethernet0/0 Cafe-SW1(config-if)# duplex full

Enforce full duplex alignment across Cafe-SW2 paths:

Cafe-SW2(config)# interface Ethernet0/0 Cafe-SW2(config-if)# duplex full Cafe-SW2(config-if)# interface Ethernet0/1 Cafe-SW2(config-if)# duplex full Cafe-SW2(config-if)# interface Ethernet0/3 Cafe-SW2(config-if)# duplex full Cafe-SW2(config-if)# end

Verify operational characteristics to confirm zero collision counter accumulation:

Cafe-SW1# show interfaces Ethernet0/0 ¦ include duplex Cafe-SW2# show interfaces summary

Task 2: Out-of-Band Management IP and Layer 2 Trunking Setup

Isolate network switch management components onto administrative broadcast domains via a dedicated management VLAN structure.

Step 1: Declare administrative VLAN 42 inside the relational database on both appliances:

Cafe-SW1(config)# vlan 42 Cafe-SW1(config-vlan)# name Management_SVI Cafe-SW2(config)# vlan 42 Cafe-SW2(config-vlan)# name Management_SVI

Step 2: Configure 802.1Q encapsulation on inter-switch interface links and restrict transmission vectors exclusively:

Cafe-SW1(config)# interface Ethernet0/0 Cafe-SW1(config-if)# switchport trunk encapsulation dot1q Cafe-SW1(config-if)# switchport mode trunk Cafe-SW1(config-if)# switchport trunk allowed vlan 42 Cafe-SW2(config)# interface Ethernet0/0 Cafe-SW2(config-if)# switchport trunk encapsulation dot1q Cafe-SW2(config-if)# switchport mode trunk Cafe-SW2(config-if)# switchport trunk allowed vlan 42

Step 3: Define logical interface addresses and operational routing pointers on Cafe-SW1:

Cafe-SW1(config)# interface vlan 42 Cafe-SW1(config-if)# ip address 192.168.42.1 255.255.255.0 Cafe-SW1(config-if)# no shutdown Cafe-SW1(config-if)# exit Cafe-SW1(config)# ip default-gateway 192.168.42.254

Step 4: Align logical interface addresses and operational routing pointers on Cafe-SW2:

Cafe-SW2(config)# interface vlan 42 Cafe-SW2(config-if)# ip address 192.168.42.2 255.255.255.0 Cafe-SW2(config-if)# no shutdown Cafe-SW2(config-if)# exit Cafe-SW2(config)# ip default-gateway 192.168.42.254 Cafe-SW2(config-if)# end

Post-Deployment Quality Assurance Diagnostics

Verify that Layer 3 network connectivity is properly active across the management domain:

Cafe-SW1# show ip interface brief ¦ include Vlan42

Diagnostic Note: If initial diagnostic ping requests timeout, allow up to 30 seconds for the Spanning Tree Protocol (STP) convergence process to transition ports into standard forwarding mode, then retest.

Run an ICMP check from the Cafe-SW1 switch supervisor engine to verify reachability across the network backplane:

Cafe-SW1# ping 192.168.42.2

Save active running parameters to non-volatile local storage on both nodes:

Cafe-SW1# copy running-config startup-config Cafe-SW2# copy running-config startup-config