SKILL 08 – Lesson 03

Understanding Basic Interface Counter

Topology Overview & Design

This deployment guide describes the step-by-step implementation of the network infrastructure named "The First Coffee Shop" within Cisco Packet Tracer. The network is configured using a single Class C subnet spanning 192.168.1.0/24, allowing for up to 254 assignable host addresses. The physical layer design utilizes high-speed Ethernet interfaces and implements EtherChannel link aggregation to optimize switch-to-switch trunk bandwidth and introduce link level redundancy.

Figure 1.1: Packet Tracer Network Logical Topology Diagram.

# Addressing and Interface Architecture Matrix

The following table outlines the complete structural breakdown of device interfaces, hostnames, and IP address scopes required to build the infrastructure illustrated above.

| Device Hostname | Device Type | Interface | IP Address | Subnet Mask / VLAN | Default Gateway |
| --- | --- | --- | --- | --- | --- |
| Cafe01-RT01 | Router (e.g., ISR 4331) | G0/0 | 192.168.1.1 | 255.255.255.0 | N/A |
| Cafe01-SW01 | Switch (2960) | VLAN 1 (Management) | 192.168.1.2 | 255.255.255.0 | 192.168.1.1 |
| Cafe01-SW02 | Switch (2960) | VLAN 1 (Management) | 192.168.1.3 | 255.255.255.0 | 192.168.1.1 |
| PC1 | End Device | FastEthernet0 | 192.168.1.10 | 255.255.255.0 | 192.168.1.1 |
| PC2 | End Device | FastEthernet0 | 192.168.1.11 | 255.255.255.0 | 192.168.1.1 |
| Server | End Device | FastEthernet0 | 192.168.1.20 | 255.255.255.0 | 192.168.1.1 |

Note on EtherChannel Configuration: The parallel connections between Cafe01-SW01 and Cafe01-SW02 across interfaces FA0/1 and FA0/2 must be explicitly grouped into a logical Link Aggregation Group (LAG) to prevent Spanning Tree Protocol (STP) blocking loops.

# Step-by-Step Deployment Instructions

## Step 3.1: Deploy Network Nodes to Workspace

- Launch Cisco Packet Tracer.

- From the bottom-left Device Selection Box, drag one Router (such as the ISR 4321 or 2911 model) into the workspace and change its display name to Cafe01-RT01.

- Drag two 2960 Catalyst Switches into the workspace. Label the upper switch Cafe01-SW01 and the lower switch Cafe01-SW02.

- Place two End Device PCs (labeled PC1 and PC2) and one Generic Server asset onto the panel according to the reference blueprint.

## Step 3.2: Complete Cabling Connections

Using the connection tool select the required physical mediums:

  - Copper Straight-Through Cable: Connect PC1 (Fa0) to Cafe01-SW01 (Fa0/3).

  - Copper Straight-Through Cable: Connect Cafe01-RT01 (G0/0) to Cafe01-SW01 (Fa0/24).

  - Copper Cross-Over Cables / EtherChannel Pair: Connect Cafe01-SW01 (Fa0/1) to Cafe01-SW02

(Fa0/1) AND connect Cafe01-SW01 (Fa0/2) to Cafe01-SW02 (Fa0/2).

  - Copper Straight-Through Cable: Connect Cafe01-SW02 (Fa0/3) to PC2 (Fa0).

  - Copper Straight-Through Cable: Connect Cafe01-SW02 (Fa0/24) to the Server (Fa0).

## Step 3.3: Configure the Default Gateway (Cafe01-RT01)

Click on Cafe01-RT01, navigate to the CLI tab, and execute the following configuration commands:

Router> enable

Router# configure terminal Router(config)# hostname Cafe01-RT01

Cafe01-RT01(config)# interface GigabitEthernet0/0

Cafe01-RT01(config-if)# ip address 192.168.1.1 255.255.255.0 Cafe01-RT01(config-if)# no shutdown

Cafe01-RT01(config-if)# exit Cafe01-RT01(config)# exit

## Step 3.4: Configure Switch 1 with EtherChannel (Cafe01-SW01)

Access the CLI of Cafe01-SW01 to assign the management IP address and cluster the uplink interfaces into an active trunk port group channel:

Switch> enable

Switch# configure terminal Switch(config)# hostname Cafe01-SW01

Cafe01-SW01(config)# interface range FastEthernet0/1 - 2 Cafe01-SW01(config-if-range)# channel-group 1 mode desirable Cafe01-SW01(config-if-range)# exit

Cafe01-SW01(config)# interface port-channel 1 Cafe01-SW01(config-if)# switchport mode trunk Cafe01-SW01(config-if)# exit

Cafe01-SW01(config)# interface vlan 1

Cafe01-SW01(config-if)# ip address 192.168.1.2 255.255.255.0 Cafe01-SW01(config-if)# no shutdown

Cafe01-SW01(config-if)# exit

Cafe01-SW01(config)# ip default-gateway 192.168.1.1

## Step 3.5: Configure Switch 2 with EtherChannel (Cafe01-SW02)

Mirror the physical group configurations on Cafe01-SW02 to successfully initialize the load-balancing link aggregation state:

Switch> enable

Switch# configure terminal Switch(config)# hostname Cafe01-SW02

Cafe01-SW02(config)# interface range FastEthernet0/1 - 2 Cafe01-SW02(config-if-range)# channel-group 1 mode desirable Cafe01-SW02(config-if-range)# exit

Cafe01-SW02(config)# interface port-channel 1 Cafe01-SW02(config-if)# switchport mode trunk Cafe01-SW02(config-if)# exit

Cafe01-SW02(config)# interface vlan 1

Cafe01-SW02(config-if)# ip address 192.168.1.3 255.255.255.0 Cafe01-SW02(config-if)# no shutdown

Cafe01-SW02(config-if)# exit

Cafe01-SW02(config)# ip default-gateway 192.168.1.1

## Step 3.6: Assign Host IP Addresses

Click on each individual endpoint node (PC1, PC2, and Server), navigate to the	Desktop tab → IP Configuration menu option, and enter these Static Parameters:

  - PC1: IP Address = 192.168.1.10 | Subnet Mask = 255.255.255.0 | Gateway = 192.168.1.1

  - PC2: IP Address = 192.168.1.11 | Subnet Mask = 255.255.255.0 | Gateway = 192.168.1.1

  - Server: IP Address = 192.168.1.20 | Subnet Mask = 255.255.255.0 | Gateway = 192.168.1.1

# Topology Verification and Troubleshooting

Once the configuration sequences are complete, verify the link health statuses across the core routing pipeline. Execute a network end-to-end connectivity ping verification test. Open the Command Prompt interface window from PC1 and verify response output reachability to the local network boundaries using:

C:\> ping 192.168.1.1

C:\> ping 192.168.1.20

If all responses indicate a 0% loss packet metric parameter value sequence, the workspace has been deployed successfully.