# Skill 09 Lab - Static and Default Internet Routing
(June 10)

Skill 9 - Skill Lab 01: Static and Default Internet Routing

Figure 1. Updated Coffee House Beta, Fallout Shelter, and ISP/Internet topology.

Important addressing note: In the updated diagram, Fallout-RT1 G0/1 uses 192.168.3.1/24 as the default gateway. Shelter-SRV must use a different host address, such as 192.168.3.100/24, with default gateway 192.168.3.1. Do not assign 192.168.3.1 to both the router and the server.

# Field Briefing

Shelter Delta's relay is dark, and the only thing keeping the bunker stocked is the slim uplink through Coffee House Beta. Jeremy drops you at the fallout shelter with a thermos and a mission: bring the routers online without hand-holding, stitch the branch back to headquarters with static routes, and light up the Internet edge so survivors can submit ration requests. No demos. No hints. Just you, the routers, and a wasteland that runs on caffeine.

# Objectives

For this deployment to be successful, you must complete the following:

Verify every interface needed for the Coffee House-Shelter-ISP path is present, addressed, and up so connected routes populate correctly.

Replace any lingering dynamic routing remnants with precise static routes between Cafe-RT1 and Fallout-RT1 to restore inter-shelter reachability.

Establish the default Internet gateway on Cafe-RT1, ensuring outbound traffic rides the ISP link while LAN traffic follows the static path.

Validate routing-table trust decisions and prove the LAN hosts can reach both the remote shelter services and a simulated Internet target.

# Access Credentials

| Item | Value |
| --- | --- |
| Router console / VTY | cisco / cisco |
| Username | cisco |
| Password | cisco |
| Privileged access | CrC0ffee! |

# Addressing Table

| Device | Interface | IP address / prefix | Default gateway | Connected to | Role |
| --- | --- | --- | --- | --- | --- |
| Bunker-Terminal | Fa0 | 192.168.1.50/24 | 192.168.1.1 | Cafe-RT1 G0/0 | Cafe LAN host |
| Cafe-RT1 | G0/0 | 192.168.1.1/24 | N/A | Bunker-Terminal Fa0 | Cafe LAN default gateway |
| Cafe-RT1 | G0/1 | 10.0.2.1/30 | N/A | Fallout-RT1 G0/0 | Point-to-point Castle link |
| Fallout-RT1 | G0/0 | 10.0.2.2/30 | N/A | Cafe-RT1 G0/1 | Point-to-point Castle link |
| Fallout-RT1 | G0/1 | 192.168.3.1/24 | N/A | Shelter-SRV Fa0 | Shelter LAN default gateway |
| Shelter-SRV | Fa0 | 192.168.3.100/24 | 192.168.3.1 | Fallout-RT1 G0/1 | Shelter service host |
| Cafe-RT1 | G0/2 | 216.0.5.2/30 | N/A | Cable Modem / ISP | ISP-facing Internet edge |
| ISP-GW | E0/0 | 216.0.5.1/30 | N/A | Cafe-RT1 G0/2 | Provider gateway |
| ISP-GW | Loopback0 | 203.0.113.8/32 | N/A | Internal loopback | Simulated Internet target |

# Student Rules for This Lab

Use static routing only. Remove EIGRP or any other dynamic routing from this lab before adding static routes.

Do not use the same IP address on a router interface and a host.

Use show commands before and after each task so you can prove what changed.

Save the final configuration after validation succeeds.

Write down the output requested in each engineering log box.

# Task 1 - Reestablish the Castle Link

Objective: Confirm the physical and logical paths are alive before you rely on routing decisions.

On Cafe-RT1, confirm G0/0, G0/1, and G0/2 exist, carry the assigned addresses above, and sit in an up/up state. Correct shutdowns or mislabels before moving on.

On Fallout-RT1, verify G0/0 and G0/1 align with the LAN and point-to-point networks, and ensure no rogue addressing remains from prior drills.

Capture the connected routes each router advertises, meaning the C and L entries, so you have a baseline snapshot for the engineering log.

Cafe-RT1# show ip interface brief
Cafe-RT1# show interfaces description
Cafe-RT1# show ip route connected

Fallout-RT1# show ip interface brief
Fallout-RT1# show interfaces description
Fallout-RT1# show ip route connected

Engineering log - connected route baseline:

| Router | Connected networks seen | Notes / corrections made |
| --- | --- | --- |
| Cafe-RT1 |  |  |
| Fallout-RT1 |  |  |
| ISP-GW |  |  |

# Task 2 - Seal the Inter-Shelter Routes

Objective: Replace guidance-system training wheels with deliberate static routing between the sites.

Audit both routers for leftover router eigrp 1 or other dynamic processes. Remove any entries so only connected networks remain before you add static routes.

Configure Cafe-RT1 with a static route to 192.168.3.0/24 via 10.0.2.2.

Configure Fallout-RT1 with a static route to 192.168.1.0/24 via 10.0.2.1.

Verify the routing tables display S entries with the correct next hops and that end-to-end pings between Bunker-Terminal and Shelter-SRV succeed in both directions.

Cafe-RT1# show running-config | section router
Cafe-RT1# configure terminal
Cafe-RT1(config)# no router eigrp 1
Cafe-RT1(config)# ip route 192.168.3.0 255.255.255.0 10.0.2.2
Cafe-RT1(config)# end
Cafe-RT1# show ip route static

Fallout-RT1# show running-config | section router
Fallout-RT1# configure terminal
Fallout-RT1(config)# no router eigrp 1
Fallout-RT1(config)# ip route 192.168.1.0 255.255.255.0 10.0.2.1
Fallout-RT1(config)# end
Fallout-RT1# show ip route static

Packet Tracer note: If your Packet Tracer router does not support the pipe symbol, use show running-config and manually look for the router eigrp section.

# Task 3 - Anchor the Internet Edge

Objective: Deliver a resilient default path that keeps the Coffee House online when traffic leaves the local prefixes.

Confirm Cafe-RT1 G0/2 is addressed as 216.0.5.2/30 and the provider gateway 216.0.5.1 responds.

Install a default route 0.0.0.0/0 on Cafe-RT1 pointing to 216.0.5.1.

Confirm the Gateway of last resort appears in the routing table.

From Bunker-Terminal, verify pings to 203.0.113.8 fail before the default route, succeed after it is in place, and that the shelter static route remains preferred for 192.168.3.0/24.

Cafe-RT1# ping 216.0.5.1
Cafe-RT1# configure terminal
Cafe-RT1(config)# ip route 0.0.0.0 0.0.0.0 216.0.5.1
Cafe-RT1(config)# end
Cafe-RT1# show ip route
Cafe-RT1# show ip route 0.0.0.0
Cafe-RT1# show ip route 192.168.3.0

Bunker-Terminal> ping 203.0.113.8
Bunker-Terminal> tracert 203.0.113.8

Return-path requirement: ISP-GW needs a route back to the private lab networks. In this lab, ISP-GW should have ip route 192.168.0.0 255.255.0.0 216.0.5.2 so replies can return to Cafe-RT1.

# Task 4 - Verify Route Trust and Failover Logic

Objective: Prove the router selects the intended paths and understands which entries to prefer under pressure.

Record the [administrative distance/metric] details for the static inter-shelter routes and the default route on Cafe-RT1.

Confirm static LAN entries outrank the default path because the router chooses the most specific matching route first, then administrative distance if two routes match the same prefix.

Create a temporary floating static default on Fallout-RT1 toward 10.0.2.1 with an administrative distance of 5, document it, then remove it to restore the baseline.

Log the final routing table snapshot on both routers, highlighting the C, L, S, and S* route codes.

Cafe-RT1# show ip route 192.168.3.0
Cafe-RT1# show ip route 0.0.0.0
Cafe-RT1# show ip route

Fallout-RT1# configure terminal
Fallout-RT1(config)# ip route 0.0.0.0 0.0.0.0 10.0.2.1 5
Fallout-RT1(config)# end
Fallout-RT1# show ip route static
Fallout-RT1# show ip route 0.0.0.0

Fallout-RT1# configure terminal
Fallout-RT1(config)# no ip route 0.0.0.0 0.0.0.0 10.0.2.1 5
Fallout-RT1(config)# end
Fallout-RT1# show ip route static

# Completion Check

☐ Cafe-RT1 and Fallout-RT1 list the correct connected networks and reciprocal static routes.

☐ Bunker-Terminal and Shelter-SRV exchange traffic successfully.

☐ Cafe-RT1 advertises a default route toward 216.0.5.1.

☐ Bunker-Terminal reaches the simulated Internet address 203.0.113.8 through the ISP link.

☐ The static LAN route is still honored for shelter traffic and does not get replaced by the default route.

☐ Final routing tables document expected administrative distances: normal S routes use AD 1, and the floating default uses AD 5 during testing.

☐ No dynamic routing processes remain active.

# Answer Key and Completed Lab Objectives

Use this section after completing the lab. It gives the expected commands and the expected verification results.

| Task objective | Expected proof | Pass criteria |
| --- | --- | --- |
| Verify interfaces and connected routes | show ip interface brief; show ip route connected | All required interfaces are up/up and C/L routes appear. |
| Remove dynamic routing | show running-config ¦ section router | No router eigrp section remains. |
| Configure inter-shelter static routes | show ip route static | Cafe has S 192.168.3.0/24 via 10.0.2.2; Fallout has S 192.168.1.0/24 via 10.0.2.1. |
| Configure Internet default route | show ip route; show ip route 0.0.0.0 | Cafe shows Gateway of last resort is 216.0.5.1 and S* 0.0.0.0/0 via 216.0.5.1. |
| Verify route trust | show ip route 192.168.3.0 and show ip route 0.0.0.0 | Static routes show [1/0]. Floating default shows AD 5 during testing. |
| Validate connectivity | ping and traceroute | LAN-to-LAN and Internet-simulation tests succeed. |

## Completed Cafe-RT1 Configuration

enable
configure terminal
hostname Cafe-RT1

interface gigabitethernet0/0
 description Cafe LAN to Bunker-Terminal
 ip address 192.168.1.1 255.255.255.0
 no shutdown
 exit

interface gigabitethernet0/1
 description Castle Link to Fallout-RT1 G0/0
 ip address 10.0.2.1 255.255.255.252
 no shutdown
 exit

interface gigabitethernet0/2
 description ISP Uplink to Cable Modem / ISP-GW
 ip address 216.0.5.2 255.255.255.252
 no shutdown
 exit

no router eigrp 1

ip route 192.168.3.0 255.255.255.0 10.0.2.2
ip route 0.0.0.0 0.0.0.0 216.0.5.1

end
copy running-config startup-config

## Completed Fallout-RT1 Configuration

enable
configure terminal
hostname Fallout-RT1

interface gigabitethernet0/0
 description Castle Link to Cafe-RT1 G0/1
 ip address 10.0.2.2 255.255.255.252
 no shutdown
 exit

interface gigabitethernet0/1
 description Shelter LAN to Shelter-SRV
 ip address 192.168.3.1 255.255.255.0
 no shutdown
 exit

no router eigrp 1

ip route 192.168.1.0 255.255.255.0 10.0.2.1

end
copy running-config startup-config

## Completed ISP-GW Configuration

enable
configure terminal
hostname ISP-GW

interface ethernet0/0
 description Uplink to Cafe-RT1 G0/2
 ip address 216.0.5.1 255.255.255.252
 no shutdown
 exit

interface loopback0
 description Internet Simulation
 ip address 203.0.113.8 255.255.255.255
 exit

ip route 192.168.0.0 255.255.0.0 216.0.5.2

end
copy running-config startup-config

## Expected Routing Table Highlights

Cafe-RT1 expected highlights:
C 192.168.1.0/24 is directly connected, GigabitEthernet0/0
C 10.0.2.0/30 is directly connected, GigabitEthernet0/1
C 216.0.5.0/30 is directly connected, GigabitEthernet0/2
S 192.168.3.0/24 [1/0] via 10.0.2.2
S* 0.0.0.0/0 [1/0] via 216.0.5.1

Fallout-RT1 expected highlights:
C 10.0.2.0/30 is directly connected, GigabitEthernet0/0
C 192.168.3.0/24 is directly connected, GigabitEthernet0/1
S 192.168.1.0/24 [1/0] via 10.0.2.1

ISP-GW expected highlights:
C 216.0.5.0/30 is directly connected, Ethernet0/0
C 203.0.113.8/32 is directly connected, Loopback0
S 192.168.0.0/16 [1/0] via 216.0.5.2

## Expected Testing Matrix

| Source | Destination | Command | Expected result |
| --- | --- | --- | --- |
| Cafe-RT1 | Fallout-RT1 WAN | ping 10.0.2.2 | Success |
| Fallout-RT1 | Cafe-RT1 WAN | ping 10.0.2.1 | Success |
| Bunker-Terminal | Cafe gateway | ping 192.168.1.1 | Success |
| Shelter-SRV | Shelter gateway | ping 192.168.3.1 | Success |
| Bunker-Terminal | Shelter-SRV | ping 192.168.3.100 | Success after reciprocal static routes |
| Shelter-SRV | Bunker-Terminal | ping 192.168.1.50 | Success after reciprocal static routes |
| Cafe-RT1 | ISP gateway | ping 216.0.5.1 | Success |
| Bunker-Terminal | Internet simulation | ping 203.0.113.8 | Success after Cafe default route and ISP return route |
| Bunker-Terminal | Internet simulation path | tracert 203.0.113.8 | Path should go through 192.168.1.1, then 216.0.5.1 |

## CCNA Concept Recap

Connected route C: A route installed automatically when a router interface has an IP address and is up/up.

Local route L: A /32 route representing the router interface IP address itself.

Static route S: A manually configured path to a destination network. Normal static routes have administrative distance 1.

Default route S*: A route of 0.0.0.0/0 used when no more-specific route matches the destination.

Administrative distance: A trust value. Lower is more trusted when multiple sources provide the same prefix.

Longest prefix match: The router chooses the most specific matching route first. For example, 192.168.3.0/24 beats 0.0.0.0/0 for shelter traffic.

Floating static route: A static route with a higher AD, such as 5, used as backup if a better route disappears.

Final save command on every router:
copy running-config startup-config