# CCNA Lab Guide
Internet and Local LAN Routing

Coffee House LAN, Fallout Shelter LAN, EIGRP, default routes, and Internet simulation

Figure 1. Uploaded topology for the local LANs and simulated Internet uplink.

Important addressing note: In the diagram, Shelter-SRV is labeled 192.168.3.1, but Fallout-RT1 G0/1 is also labeled 192.168.3.1/24. Two devices on the same LAN cannot use the same IP address. Keep Fallout-RT1 G0/1 as 192.168.3.1/24 because it is the default gateway. Change Shelter-SRV to an unused address such as 192.168.3.10/24 or 192.168.3.50/24 with default gateway 192.168.3.1.

# 1. Purpose of this lab

Understand the difference between a local LAN route and an Internet/default route.

Build routing between the Cafe LAN and Shelter LAN using EIGRP.

Connect the Cafe router to a simulated ISP gateway.

Use static default routes so inside networks can reach the simulated Internet.

Read routing tables and explain C, L, D, S, and S* routes.

Test end-to-end connectivity with ping and traceroute/tracert.

# 2. Addressing plan

| Device | Interface | IP address / mask | Default gateway | Purpose |
| --- | --- | --- | --- | --- |
| Bunker-Terminal | Fa0 | 192.168.1.50 /24 | 192.168.1.1 | Cafe LAN host |
| Cafe-RT1 | G0/0 or E0/0 | 192.168.1.1 /24 | - | Cafe LAN gateway |
| Cafe-RT1 | G0/1 or E0/1 | 192.168.2.1 /30 | - | Router-to-router link |
| Fallout-RT1 | G0/0 or E0/0 | 192.168.2.2 /30 | - | Router-to-router link |
| Fallout-RT1 | G0/1 or E0/1 | 192.168.3.1 /24 | - | Shelter LAN gateway |
| Shelter-SRV | Fa0 | 192.168.3.10 /24 recommended | 192.168.3.1 | Shelter LAN server |
| Cafe-RT1 | G0/2 or E0/2 | 216.0.5.2 /30 | - | Uplink toward ISP-GW |
| ISP-GW | E0/0 | 216.0.5.1 /30 | - | ISP side of the WAN link |
| ISP-GW | Loopback0 | 203.0.113.8 /32 | - | Simulated Internet destination |

Interface name note: Your diagram uses GigabitEthernet labels, while the uploaded CLI traces show Ethernet0/x. Use the interface names that appear on your device when you run show ip interface brief.

# 3. What your uploaded traces already show

| Device | Evidence from trace | Meaning |
| --- | --- | --- |
| Cafe-RT1 | 192.168.1.1/24, 192.168.2.1/30, and 216.0.5.2/30 are up/up | Cafe has a LAN, a router-to-router link, and an ISP uplink. |
| Cafe-RT1 | D 192.168.3.0/24 via 192.168.2.2 | Cafe is learning the Shelter LAN dynamically through EIGRP. |
| Fallout-RT1 | D 192.168.1.0/24 via 192.168.2.1 | Fallout is learning the Cafe LAN dynamically through EIGRP. |
| ISP-GW | Loopback0 203.0.113.8/32 and E0/0 216.0.5.1/30 | The ISP router has a simulated Internet address. |
| ISP-GW | ip route 192.168.0.0 255.255.0.0 216.0.5.2 | The ISP has a return route back to the private lab networks. |

# 4. Local LAN vs Internet route

| Concept | Example in this lab | Explanation |
| --- | --- | --- |
| Local LAN | 192.168.1.0/24 and 192.168.3.0/24 | Private networks used inside the organization or lab. |
| Transit/WAN link | 192.168.2.0/30 | Small point-to-point network used only between routers. |
| ISP uplink | 216.0.5.0/30 | Network between Cafe-RT1 and ISP-GW. |
| Simulated Internet | 203.0.113.8/32 | Loopback address on ISP-GW used as a test Internet destination. |
| Default route | 0.0.0.0/0 | Route used when the router has no more specific route. |

A router checks its routing table from most specific to least specific. If it has a route for 192.168.3.0/24, it uses that route. If it does not have a specific route for a destination such as 203.0.113.8, it needs a default route.

# 5. Configure the PCs and server

| Host | IP address | Subnet mask | Default gateway |
| --- | --- | --- | --- |
| Bunker-Terminal | 192.168.1.50 | 255.255.255.0 | 192.168.1.1 |
| Shelter-SRV | 192.168.3.10 | 255.255.255.0 | 192.168.3.1 |

## 6. Configure Cafe-RT1 interfaces

enable

configure terminal

hostname Cafe-RT1

no ip domain-lookup

interface ethernet0/0

description Cafe-LAN to Bunker-Terminal

ip address 192.168.1.1 255.255.255.0

no shutdown

exit

interface ethernet0/1

description Castle Link to Fallout-RT1 E0/0

ip address 192.168.2.1 255.255.255.252

no shutdown

exit

interface ethernet0/2

description ISP Uplink to ISP-GW E0/0

ip address 216.0.5.2 255.255.255.252

no shutdown

exit

## 7. Configure Fallout-RT1 interfaces

enable

configure terminal

hostname Fallout-RT1

no ip domain-lookup

interface ethernet0/0

description Castle Link to Cafe-RT1 E0/1

ip address 192.168.2.2 255.255.255.252

no shutdown

exit

interface ethernet0/1

description Shelter LAN to Shelter-SRV

ip address 192.168.3.1 255.255.255.0

no shutdown

exit

## 8. Configure ISP-GW interfaces

enable

configure terminal

hostname ISP-GW

no ip domain-lookup

interface ethernet0/0

description Uplink to Cafe-RT1 E0/2

ip address 216.0.5.1 255.255.255.252

no shutdown

exit

interface loopback0

description Internet Simulation

ip address 203.0.113.8 255.255.255.255

exit

# 9. Configure dynamic routing for the local LANs with EIGRP

Goal: Cafe-RT1 and Fallout-RT1 should automatically learn each other's LAN routes over the 192.168.2.0/30 link.

## Cafe-RT1 EIGRP

router eigrp 1

no auto-summary

network 192.168.1.0 0.0.0.255

network 192.168.2.0 0.0.0.3

passive-interface ethernet0/0

exit

## Fallout-RT1 EIGRP

router eigrp 1

no auto-summary

network 192.168.2.0 0.0.0.3

network 192.168.3.0 0.0.0.255

passive-interface ethernet0/1

exit

Use passive-interface on LAN-facing interfaces because PCs and servers do not form EIGRP neighbor relationships. Do not make the router-to-router link passive.

# 10. Configure Internet/default routing

Important: EIGRP is only handling the two local LANs. To reach the simulated Internet address 203.0.113.8, the inside routers need a path toward ISP-GW, and ISP-GW needs a return path back to the private networks.

## Cafe-RT1 default route toward ISP-GW

configure terminal

ip route 0.0.0.0 0.0.0.0 216.0.5.1

end

## Fallout-RT1 default route toward Cafe-RT1

configure terminal

ip route 0.0.0.0 0.0.0.0 192.168.2.1

end

## ISP-GW return route toward Cafe-RT1

configure terminal

ip route 192.168.0.0 255.255.0.0 216.0.5.2

end

Why this works: Cafe-RT1 sends unknown destinations to 216.0.5.1. Fallout-RT1 sends unknown destinations to Cafe-RT1 at 192.168.2.1. ISP-GW sends replies for 192.168.x.x networks back to Cafe-RT1 at 216.0.5.2.

# 11. Optional: advertise the default route with EIGRP instead of a static default on Fallout-RT1

For a beginner lab, the static default route on Fallout-RT1 is simple and clear. If your instructor wants Cafe-RT1 to advertise a default route dynamically to Fallout-RT1, use this instead:

## Cafe-RT1: redistribute the static default into EIGRP

configure terminal

router eigrp 1

redistribute static

exit

end

Then remove the static default route from Fallout-RT1 if it is no longer needed:

## Fallout-RT1: remove static default if using EIGRP-learned default

configure terminal

no ip route 0.0.0.0 0.0.0.0 192.168.2.1

end

# 12. NAT note for real Internet vs lab Internet

In a real home or business network, private addresses such as 192.168.1.50 and 192.168.3.10 usually need NAT/PAT before going to the public Internet. In this lab, NAT is not required if ISP-GW has a return route for 192.168.0.0/16. That static route makes the simulated Internet know how to send replies back to the private lab networks.

## Optional realistic NAT/PAT on Cafe-RT1

configure terminal

access-list 1 permit 192.168.0.0 0.0.255.255

interface ethernet0/0

ip nat inside

exit

interface ethernet0/1

ip nat inside

exit

interface ethernet0/2

ip nat outside

exit

ip nat inside source list 1 interface ethernet0/2 overload

end

Only add NAT if your lab requires realistic Internet behavior. For basic routing-table learning, keep the lab simple with static return routes.

# 13. Verification commands

## Run on Cafe-RT1

show ip interface brief

show ip eigrp neighbors

show ip route

show ip route eigrp

show ip route static

## Run on Fallout-RT1

show ip interface brief

show ip eigrp neighbors

show ip route

show ip route eigrp

show ip route static

## Run on ISP-GW

show ip interface brief

show ip route

show running-config | include ip route

# 14. Expected routing-table results

| Router | Expected route | Meaning |
| --- | --- | --- |
| Cafe-RT1 | D 192.168.3.0/24 via 192.168.2.2 | EIGRP route to the Shelter LAN. |
| Cafe-RT1 | S* 0.0.0.0/0 via 216.0.5.1 | Default route to the ISP. |
| Fallout-RT1 | D 192.168.1.0/24 via 192.168.2.1 | EIGRP route to the Cafe LAN. |
| Fallout-RT1 | S* 0.0.0.0/0 via 192.168.2.1 | Default route to the Internet through Cafe-RT1. |
| ISP-GW | S 192.168.0.0/16 via 216.0.5.2 | Return route back to private lab networks. |

# 15. Ping and traceroute tests

## From Bunker-Terminal

ping 192.168.1.1

ping 192.168.2.2

ping 192.168.3.1

ping 192.168.3.10

ping 216.0.5.1

ping 203.0.113.8

tracert 192.168.3.10

tracert 203.0.113.8

## From Shelter-SRV

ping 192.168.3.1

ping 192.168.2.1

ping 192.168.1.1

ping 192.168.1.50

ping 216.0.5.1

ping 203.0.113.8

tracert 192.168.1.50

tracert 203.0.113.8

## From Cafe-RT1

ping 192.168.2.2

ping 192.168.3.1

ping 216.0.5.1

ping 203.0.113.8

traceroute 203.0.113.8

## From Fallout-RT1

ping 192.168.2.1

ping 192.168.1.1

ping 216.0.5.1

ping 203.0.113.8

traceroute 203.0.113.8

# 16. Troubleshooting checklist

If Bunker-Terminal cannot ping 192.168.1.1, check its IP address, subnet mask, default gateway, and cable to Cafe-RT1.

If Cafe-RT1 cannot ping Fallout-RT1 at 192.168.2.2, check the 192.168.2.0/30 addressing and make sure both router interfaces are up/up.

If EIGRP neighbor is missing, verify both routers use the same EIGRP AS number and the 192.168.2.0 0.0.0.3 network command.

If Cafe-RT1 does not learn 192.168.3.0/24, verify Fallout-RT1 advertises network 192.168.3.0 0.0.0.255.

If Fallout-RT1 does not learn 192.168.1.0/24, verify Cafe-RT1 advertises network 192.168.1.0 0.0.0.255.

If inside devices cannot reach 203.0.113.8, check the default routes on Cafe-RT1 and Fallout-RT1.

If replies from ISP-GW fail, check the ISP-GW return route: ip route 192.168.0.0 255.255.0.0 216.0.5.2.

If Shelter-SRV uses 192.168.3.1, change it to a different address because 192.168.3.1 belongs to Fallout-RT1.

# 17. Beginner Q&A

Q: What is a LAN?

A: A local area network. In this lab, 192.168.1.0/24 is the Cafe LAN and 192.168.3.0/24 is the Shelter LAN.

Q: What is the Internet link?

A: The Internet link is the path from Cafe-RT1 to ISP-GW using 216.0.5.0/30.

Q: Why do we use /30 on router links?

A: A /30 gives two usable IP addresses, which is enough for a point-to-point router link.

Q: What does EIGRP do?

A: EIGRP allows Cafe-RT1 and Fallout-RT1 to dynamically learn each other's LAN networks.

Q: What does D mean in show ip route?

A: D means the route was learned by EIGRP.

Q: What does S* mean?

A: S means static, and the asterisk means candidate default route.

Q: Why do we need a default route?

A: The router uses the default route when it does not have a more specific route for the destination.

Q: Why does ISP-GW need a route back?

A: Routing must work in both directions. ISP-GW must know how to send replies back to 192.168.1.0/24 and 192.168.3.0/24.

# 18. Completion checklist

☐ Bunker-Terminal can ping 192.168.1.1.

☐ Cafe-RT1 can ping Fallout-RT1 at 192.168.2.2.

☐ Cafe-RT1 learns 192.168.3.0/24 through EIGRP.

☐ Fallout-RT1 learns 192.168.1.0/24 through EIGRP.

☐ Cafe-RT1 can ping ISP-GW at 216.0.5.1.

☐ Bunker-Terminal can ping Shelter-SRV.

☐ Shelter-SRV can ping Bunker-Terminal.

☐ Inside devices can ping the simulated Internet address 203.0.113.8.

☐ All router configurations are saved with copy running-config startup-config.