# Skill 10 • Skill Lab 01
Configuring Basic NAT

CCNA student lab: static NAT, dynamic NAT, and NAT overload on Cafe-Rtr

Figure 1. Basic NAT topology using Cafe-LAN-SW, Cafe-Rtr, and ISP-Rtr.

# Field Briefing

District Theta's cafe bunker is back online, but the fallout shelters upstream keep filing tickets: some endpoints can reach the emulated internet, others stall the moment they leave the LAN. Jeremy dispatches you to the site, puts you beside Cafe-Rtr, and expects you to rebuild the entire NAT stack without a script. The ISP emulator is on standby, the cafe floor is buzzing, and Castle Rysen needs one engineer who can go from raw router ports to a fully overloaded edge in a single push.

Lab focus: You will rebuild Cafe-Rtr and ISP-Rtr from scratch. The switch and PCs only need correct cabling, IP addresses, and default gateways. NAT is performed on Cafe-Rtr only.

# Objectives

Stand up the ISP-side infrastructure so the cafe has a functional upstream with public DNS loopbacks.

Publish one cafe host with a one-to-one static NAT mapping.

Shift from static NAT to dynamic NAT using a public address pool.

Finish with interface-based NAT overload so all cafe devices share one public identity.

Verify each NAT phase with ping, show ip nat translations, show ip nat statistics, and routing checks.

# Access Credentials

| Device access | Value |
| --- | --- |
| Console password | Cisco |
| VTY password | Cisco |
| Enable secret | Cisco |
| Important note | Passwords are case-sensitive in this lab. |

# Interface Naming Note

Packet Tracer note: This lab uses Ethernet0/0 and Ethernet0/1 because the written scenario names Ethernet ports. If your router shows GigabitEthernet0/0 and GigabitEthernet0/1 instead, use the same commands but replace Ethernet with GigabitEthernet.

# Addressing Plan

| Device | Interface | IP address | Mask | Purpose |
| --- | --- | --- | --- | --- |
| PC-A / PC1 in scenario | Fa0 | 192.168.1.50 | 255.255.255.0 | Inside host for Static NAT test |
| PC-B / PC2 in scenario | Fa0 | 192.168.1.51 | 255.255.255.0 | Second inside host for dynamic and overload tests |
| Cafe-LAN-SW | VLAN 1 | 192.168.1.2 | 255.255.255.0 | Switch management address |
| Cafe-Rtr | E0/0 | 192.168.1.1 | 255.255.255.0 | LAN gateway / NAT inside |
| Cafe-Rtr | E0/1 | 216.0.5.2 | 255.255.255.252 | WAN to ISP / NAT outside |
| ISP-Rtr | E0/0 | 216.0.5.1 | 255.255.255.252 | ISP gateway toward Cafe-Rtr |
| ISP-Rtr | Loopback0 | 1.1.1.1 | 255.255.255.255 | Simulated public DNS target |
| ISP-Rtr | Loopback1 | 8.8.8.8 | 255.255.255.255 | Simulated public DNS target |

Diagram note: The uploaded diagram labels 192.168.1.50 and 192.168.1.51 as inside-local hosts. If your PC labels differ from the written scenario, trust the IP address. The Static NAT host in this lab is 192.168.1.50.

# NAT Progression Used in This Lab

| Phase | Cafe-Rtr NAT command focus | What students should observe |
| --- | --- | --- |
| Static NAT | ip nat inside source static 192.168.1.50 216.0.5.20 | 192.168.1.50 is permanently represented by 216.0.5.20. |
| Dynamic NAT | ip nat inside source list 1 pool Cafe-Public | Inside hosts borrow public addresses from 216.0.5.50-216.0.5.100. |
| NAT Overload | ip nat inside source list 1 interface ethernet0/1 overload | All inside hosts share 216.0.5.2 using unique identifiers. |

# Task 0 - Prepare the End Devices and Cabling

Objective: Confirm the physical topology and host addressing before rebuilding the routers.

Connect PC 192.168.1.50 and PC 192.168.1.51 to Cafe-LAN-SW.

Connect Cafe-LAN-SW to Cafe-Rtr Ethernet0/0, the LAN-facing router port.

Connect Cafe-Rtr Ethernet0/1 to ISP-Rtr Ethernet0/0, the WAN link.

Set PC 192.168.1.50/24 default gateway to 192.168.1.1.

Set PC 192.168.1.51/24 default gateway to 192.168.1.1.

PC command prompt checks
ipconfig
ping 192.168.1.1

# Task 1 - Rebuild the ISP Edge

Objective: Deliver a live ISP emulator with secured access, an active WAN link, and public DNS targets.

Before you start: If you are reusing an old Packet Tracer file, erase old addresses and NAT-related routes only if they conflict with this lab. A fresh router can receive the full configuration below.

## ISP-Rtr configuration

enable
configure terminal
hostname ISP-Rtr

! Castle Rysen access controls
no ip domain-lookup
enable secret Cisco
service password-encryption

line console 0
 password Cisco
 login
 logging synchronous
 exit
line vty 0 4
 password Cisco
 login
 exit

interface ethernet0/0
 description WAN to Cafe-Rtr Ethernet0/1
 ip address 216.0.5.1 255.255.255.252
 no shutdown
 exit

interface loopback0
 description Simulated Internet target 1
 ip address 1.1.1.1 255.255.255.255
 exit

interface loopback1
 description Simulated Internet target 2
 ip address 8.8.8.8 255.255.255.255
 exit

! Return routes for the NAT addresses used later in this lab
ip route 216.0.5.20 255.255.255.255 216.0.5.2
ip route 216.0.5.0 255.255.255.0 216.0.5.2

end
copy running-config startup-config

## Cafe-Rtr baseline configuration

enable
configure terminal
hostname Cafe-Rtr

! Castle Rysen access controls
no ip domain-lookup
enable secret Cisco
service password-encryption

line console 0
 password Cisco
 login
 logging synchronous
 exit
line vty 0 4
 password Cisco
 login
 exit

interface ethernet0/0
 description Cafe LAN - NAT inside
 ip address 192.168.1.1 255.255.255.0
 ip nat inside
 no shutdown
 exit

interface ethernet0/1
 description WAN to ISP-Rtr - NAT outside
 ip address 216.0.5.2 255.255.255.252
 ip nat outside
 no shutdown
 exit

ip route 0.0.0.0 0.0.0.0 216.0.5.1

end
copy running-config startup-config

Default route meaning: The default route on Cafe-Rtr sends unknown outside traffic to ISP-Rtr at 216.0.5.1.

## Task 1 validation

On ISP-Rtr
show ip interface brief
show ip route
ping 216.0.5.1
ping 216.0.5.2
ping 1.1.1.1
ping 8.8.8.8

On Cafe-Rtr
show ip interface brief
show ip route
ping 216.0.5.1
ping 1.1.1.1
ping 8.8.8.8

Required result: Cafe-Rtr must be able to ping 216.0.5.1, 1.1.1.1, and 8.8.8.8 before NAT testing begins. If Cafe-Rtr cannot ping 216.0.5.1, fix cabling, interface status, or IP addresses before continuing.

# Task 2 - Publish the Cafe Menu with Static NAT

Objective: Expose the inside host 192.168.1.50 to the public network with a one-to-one translation while keeping the LAN boundary clear.

Inside and outside roles: Cafe-Rtr Ethernet0/0 is the inside interface. Cafe-Rtr Ethernet0/1 is the outside interface. Static NAT will not work if these roles are reversed.

## Static NAT configuration on Cafe-Rtr

enable
configure terminal

interface ethernet0/0
 ip nat inside
 exit
interface ethernet0/1
 ip nat outside
 exit

! Publish the inside host 192.168.1.50 as 216.0.5.20
ip nat inside source static 192.168.1.50 216.0.5.20

end
clear ip nat translation *
copy running-config startup-config

## Task 2 validation

On Cafe-Rtr
show ip nat translations
show ip nat statistics
show running-config | include ip nat

From PC 192.168.1.50
ping 1.1.1.1
ping 8.8.8.8

From ISP-Rtr
ping 216.0.5.20
ping 216.0.5.21

Expected result: ISP-Rtr should be able to ping 216.0.5.20 because it is statically mapped to 192.168.1.50. ISP-Rtr should not be able to ping 216.0.5.21 because no static mapping exists for that address in Task 2.

Expected static NAT table example
Pro  Inside global     Inside local       Outside local      Outside global
---  216.0.5.20        192.168.1.50       ---                ---

# Task 3 - Shift to Dynamic NAT Pooling

Objective: Allow the entire 192.168.1.0/24 cafe subnet to borrow public IP addresses from a defined pool.

## Remove static NAT and configure Dynamic NAT

enable
configure terminal

! Remove the static mapping from Task 2
no ip nat inside source static 192.168.1.50 216.0.5.20

! Clean up old dynamic or overload NAT rules if you practiced earlier
no ip nat inside source list 1 interface ethernet0/1 overload
no ip nat inside source list 1 pool Cafe-Public
no access-list 1

! Match the entire cafe LAN
access-list 1 permit 192.168.1.0 0.0.0.255

! Create a public pool for dynamic one-to-one translations
ip nat pool Cafe-Public 216.0.5.50 216.0.5.100 netmask 255.255.255.0

! Bind access list 1 to the NAT pool
ip nat inside source list 1 pool Cafe-Public

end
clear ip nat translation *
copy running-config startup-config

Wildcard mask reminder: The ACL uses 0.0.0.255, not 255.255.255.0. Standard ACLs use wildcard masks.

## Task 3 validation

From PC 192.168.1.50
ping 1.1.1.1
ping 8.8.8.8

From PC 192.168.1.51
ping 1.1.1.1
ping 8.8.8.8

On Cafe-Rtr immediately after the pings
show ip nat translations
show ip nat statistics
show access-lists 1

Expected result: Both inside hosts should reach the loopbacks. Cafe-Rtr should show each inside host borrowing a different address from the 216.0.5.50-216.0.5.100 pool.

Expected dynamic NAT table example
Pro  Inside global     Inside local       Outside local      Outside global
---  216.0.5.50        192.168.1.50       ---                ---
---  216.0.5.51        192.168.1.51       ---                ---
icmp 216.0.5.50:1      192.168.1.50:1     1.1.1.1:1          1.1.1.1:1
icmp 216.0.5.51:2      192.168.1.51:2     8.8.8.8:2          8.8.8.8:2

# Task 4 - Consolidate with NAT Overload

Objective: Collapse the cafe subnet behind the WAN interface so all hosts share 216.0.5.2 by unique session identifiers.

Console safety: This shutdown sequence is for Packet Tracer console practice. In a real remote session, shutting down both interfaces could disconnect you.

## Withdraw dynamic NAT and apply overload

enable
configure terminal

! Remove the dynamic NAT rule but keep access list 1 for reuse
no ip nat inside source list 1 pool Cafe-Public
no ip nat pool Cafe-Public 216.0.5.50 216.0.5.100 netmask 255.255.255.0

! Temporarily halt new translations by shutting down the NAT-facing ports
interface ethernet0/0
 shutdown
 exit
interface ethernet0/1
 shutdown
 exit

! Apply interface-based NAT overload using ACL 1
ip nat inside source list 1 interface ethernet0/1 overload

! Restore connectivity
interface ethernet0/0
 no shutdown
 exit
interface ethernet0/1
 no shutdown
 exit

end
clear ip nat translation *
copy running-config startup-config

Command focus: The overload keyword is the key difference. It allows many inside local addresses to share the outside interface address 216.0.5.2.

## Task 4 validation

From PC 192.168.1.50
ping 1.1.1.1
ping 8.8.8.8

From PC 192.168.1.51
ping 1.1.1.1
ping 8.8.8.8

On Cafe-Rtr immediately after the pings
show ip nat translations
show ip nat statistics
show access-lists 1
show running-config | include ip nat|access-list

Expected result: Both inside hosts should map to 216.0.5.2, but with different ICMP identifiers or transport port numbers. That proves NAT overload is active.

Expected NAT overload table example
Pro  Inside global     Inside local       Outside local      Outside global
icmp 216.0.5.2:10      192.168.1.50:10    1.1.1.1:10         1.1.1.1:10
icmp 216.0.5.2:11      192.168.1.51:11    1.1.1.1:11         1.1.1.1:11
icmp 216.0.5.2:12      192.168.1.50:12    8.8.8.8:12         8.8.8.8:12
icmp 216.0.5.2:13      192.168.1.51:13    8.8.8.8:13         8.8.8.8:13

# Troubleshooting Guide

| Problem | Most likely cause | Check or fix |
| --- | --- | --- |
| PC can ping 192.168.1.1 but not 216.0.5.1 | Cafe-Rtr WAN link or ISP-Rtr interface problem | Cafe-Rtr: ping 216.0.5.1; both WAN interfaces must be up/up. |
| Cafe-Rtr cannot ping 1.1.1.1 | Default route or ISP loopback missing | Cafe-Rtr: show ip route. ISP-Rtr: show ip interface brief. |
| PCs cannot ping 1.1.1.1 after NAT | NAT inside/outside missing, ACL wrong, or NAT rule wrong | show running-config ¦ include ip nat¦access-list and show access-lists 1. |
| show ip nat translations is empty | No inside-to-outside traffic has crossed the router yet | Generate traffic from a PC, not from Cafe-Rtr itself. |
| ACL has no matches | Wrong wildcard mask or wrong source network | Use access-list 1 permit 192.168.1.0 0.0.0.255. |
| NAT overload still uses pool addresses | Dynamic NAT rule was not removed | Remove ip nat inside source list 1 pool Cafe-Public before adding overload. |

# Useful Verification Commands

| Command | Why you use it |
| --- | --- |
| show ip interface brief | Checks addressing and up/up status. |
| show ip route | Checks connected routes and the default route. |
| show running-config ¦ include ip nat¦access-list | Quickly shows NAT and ACL statements. |
| show ip nat translations | Displays static, dynamic, or overload translations. |
| show ip nat statistics | Shows NAT interfaces, hits, misses, and translation counts. |
| show access-lists 1 | Confirms the ACL is matching inside traffic. |
| clear ip nat translation * | Clears dynamic/overload translations before retesting. |

# Final Expected Configuration - NAT Overload State

## Cafe-Rtr final NAT overload configuration

hostname Cafe-Rtr
!
enable secret Cisco
service password-encryption
!
line console 0
 password Cisco
 login
 logging synchronous
!
line vty 0 4
 password Cisco
 login
!
interface ethernet0/0
 description Cafe LAN - NAT inside
 ip address 192.168.1.1 255.255.255.0
 ip nat inside
 no shutdown
!
interface ethernet0/1
 description WAN to ISP-Rtr - NAT outside
 ip address 216.0.5.2 255.255.255.252
 ip nat outside
 no shutdown
!
access-list 1 permit 192.168.1.0 0.0.0.255
ip nat inside source list 1 interface ethernet0/1 overload
ip route 0.0.0.0 0.0.0.0 216.0.5.1

## ISP-Rtr final configuration

hostname ISP-Rtr
!
enable secret Cisco
service password-encryption
!
line console 0
 password Cisco
 login
 logging synchronous
!
line vty 0 4
 password Cisco
 login
!
interface ethernet0/0
 description WAN to Cafe-Rtr Ethernet0/1
 ip address 216.0.5.1 255.255.255.252
 no shutdown
!
interface loopback0
 description Simulated Internet target 1
 ip address 1.1.1.1 255.255.255.255
!
interface loopback1
 description Simulated Internet target 2
 ip address 8.8.8.8 255.255.255.255
!
! The static and dynamic NAT routes were useful in Tasks 2 and 3.
! They can remain, but overload only needs reachability to Cafe-Rtr at 216.0.5.2.
ip route 216.0.5.20 255.255.255.255 216.0.5.2
ip route 216.0.5.0 255.255.255.0 216.0.5.2

# Completion Check

☐ ISP-Rtr delivers 216.0.5.1/30 on the WAN link.

☐ ISP-Rtr loopbacks 1.1.1.1/32 and 8.8.8.8/32 respond to Cafe-Rtr reachability tests.

☐ Cafe-Rtr Ethernet0/0 is NAT inside and Ethernet0/1 is NAT outside.

☐ Static NAT for 192.168.1.50 to 216.0.5.20 was observed in the NAT table.

☐ Dynamic NAT pool translations from Cafe-Public were observed in the NAT table.

☐ Final NAT overload translations show multiple inside hosts sharing 216.0.5.2.

☐ PC 192.168.1.50 and PC 192.168.1.51 can ping 1.1.1.1 and 8.8.8.8 in the final overload state.

# Student Understanding Questions

1. Which Cafe-Rtr interface is NAT inside, and why?

2. Which Cafe-Rtr interface is NAT outside, and why?

3. What is the difference between inside local and inside global in this lab?

4. Why can ISP-Rtr ping 216.0.5.20 during Static NAT but not necessarily during Dynamic NAT?

5. Why does Dynamic NAT need a pool, but interface-based overload does not?

6. What does the overload keyword allow the router to do?

7. Why must traffic be generated from the PC instead of Cafe-Rtr when testing NAT translations?

8. Why is access-list 1 written with 0.0.0.255 instead of 255.255.255.0?

9. In the final state, what inside global address should PC 192.168.1.50 and PC 192.168.1.51 share?

10. Which command proves that multiple inside hosts are using NAT overload?

# Instructor Answer Key

1. Ethernet0/0 is NAT inside because it faces the private 192.168.1.0/24 LAN.

2. Ethernet0/1 is NAT outside because it faces ISP-Rtr and the simulated internet.

3. Inside local is the real private address, such as 192.168.1.50. Inside global is the public address used to represent it, such as 216.0.5.20, a pool address, or 216.0.5.2 with overload.

4. Static NAT creates a permanent one-to-one mapping. Dynamic NAT creates temporary mappings only when inside hosts initiate traffic.

5. Dynamic NAT needs one public address from a pool per active translation. Interface-based overload reuses the outside interface address and separates sessions with identifiers/ports.

6. The overload keyword enables many inside devices to share one public address.

7. NAT translates traffic passing through the router. Traffic sourced by the router itself does not create an inside-to-outside translation for a PC.

8. Standard ACLs use wildcard masks. 0.0.0.255 matches all hosts in 192.168.1.0/24.

9. Both inside hosts should share 216.0.5.2 in the final overload state.

10. show ip nat translations proves overload when multiple inside local addresses map to 216.0.5.2 with different identifiers.