# Skill 12 Lab Manual

# Configuring and Securing VLAN Trunks

ADMIN_SERVICES (VLAN 10) and PATRON_SERVICES (VLAN 20)

| Source file | Skill 12 - Lab - Locking Down Legacy Trunk Protocols (June 25) Solution.pkt |
| --- | --- |
| Packet Tracer | Version 9.0.0.0810 |
| Switch platforms | Cisco Catalyst 2960-24TT (IOS 15) |
| Audience | CCNA students practicing VLANs, 802.1Q trunks, DTP, and VTP containment |

Student name: ________________________________    Date: __________________

Purpose: build two VLANs, force trunk links into a static state, limit the VLANs carried, and verify segmentation without relying on legacy negotiation protocols.

# 1. Mission briefing

The cafe network uses two access switches, two wireless access points, and a router uplink. Your task is to create separate Layer 2 broadcast domains for administrative devices and patron wireless devices. You will configure access ports, build static 802.1Q trunks, disable Dynamic Trunking Protocol (DTP) negotiation, restrict the allowed VLAN list, and prevent VTP advertisements from rewriting the local VLAN database.

# 2. Learning objectives

Create VLAN 10 named ADMIN_SERVICES and VLAN 20 named PATRON_SERVICES on both switches.

Assign FastEthernet0/1–0/4 to VLAN 10 and FastEthernet0/5–0/8 to VLAN 20.

Configure the router-to-switch and switch-to-switch uplinks as static 802.1Q trunks.

Limit each trunk to VLANs 10 and 20 and disable DTP negotiation with switchport nonegotiate.

Place each 2960 switch in VTP transparent mode to protect the local VLAN database.

Configure the four requested endpoint addresses and connect the laptops to their access points.

Verify same-VLAN connectivity, VLAN isolation, trunk state, and VTP status.

# 3. Equipment and starting topology

| Quantity | Device | Packet Tracer model | Role |
| --- | --- | --- | --- |
| 1 | Cafe01-RT01 | Cisco 2911 | Existing router uplink; optional router-on-a-stick |
| 2 | Cafe01-SW01 / Cafe01-SW02 | Cisco 2960-24TT | Layer 2 access and trunk switches |
| 2 | WAP1 / WAP2 | AccessPoint-PT | Patron wireless bridges |
| 2 | PC1 / PC2 | PC-PT | ADMIN_SERVICES endpoints |
| 2 | Laptop1 / Laptop2 | Laptop-PT | PATRON_SERVICES wireless endpoints |

Figure 1. Logical connections and active interfaces found in the uploaded Packet Tracer solution.

# 4. Cabling and interface map

| Device A | Interface A | Device B | Interface B | Purpose |
| --- | --- | --- | --- | --- |
| Cafe01-RT01 | GigabitEthernet0/1 | Cafe01-SW01 | GigabitEthernet0/1 | Router trunk |
| Cafe01-SW01 | GigabitEthernet0/2 | Cafe01-SW02 | GigabitEthernet0/2 | Inter-switch trunk |
| Cafe01-SW01 | FastEthernet0/2 | PC1 | FastEthernet0 | VLAN 10 access |
| Cafe01-SW01 | FastEthernet0/5 | WAP1 | Port 0 | VLAN 20 access |
| Cafe01-SW02 | FastEthernet0/2 | PC2 | FastEthernet0 | VLAN 10 access |
| Cafe01-SW02 | FastEthernet0/5 | WAP2 | Port 0 | VLAN 20 access |
| Cafe01-SW02 | FastEthernet0/1 | Server | FastEthernet0 | Existing VLAN 10 device |

Interface discovery shortcut Hover over a cable in Packet Tracer or enable Options > Preferences > Always Show Port Labels. On Cisco devices, show cdp neighbors confirms switch-to-switch and router-to-switch interface pairs.

# 5. VLAN and endpoint addressing plan

| VLAN | Name | Access ports on both switches | Intended devices |
| --- | --- | --- | --- |
| 10 | ADMIN_SERVICES | FastEthernet0/1–0/4 | PC1, PC2, existing server |
| 20 | PATRON_SERVICES | FastEthernet0/5–0/8 | WAP1/Laptop1 and WAP2/Laptop2 |

| Device | VLAN | IP address | Subnet mask | Default gateway stored in file |
| --- | --- | --- | --- | --- |
| PC1 | 10 | 10.0.18.21 | 255.255.255.192 (/26) | 10.0.18.1 |
| PC2 | 10 | 10.0.18.22 | 255.255.255.192 (/26) | 10.0.18.1 |
| Laptop1 | 20 | 10.0.18.31 | 255.255.255.192 (/26) | 10.0.18.1 |
| Laptop2 | 20 | 10.0.18.32 | 255.255.255.192 (/26) | 10.0.18.1 |

Addressing consistency check All four requested addresses are members of 10.0.18.0/26. The uploaded solution also configures router subinterface G0/1.20 as 10.0.18.65/26, which is the gateway for the different subnet 10.0.18.64/26. Therefore Laptop1 and Laptop2 cannot be routed from VLAN 20 while they use 10.0.18.31, 10.0.18.32, and gateway 10.0.18.1. The required steps below preserve your exact addresses for Layer 2 VLAN testing. Section 15 provides a corrected routed option.

# 6. Prepare the Packet Tracer file

Open the uploaded solution file in Cisco Packet Tracer.

Immediately select File > Save As and create a student working copy. Do not overwrite the original solution file.

Switch to the Logical workspace and enable interface labels so every connection can be verified before configuration.

Open each switch CLI. Press Enter, type enable, and then type show cdp neighbors to compare the live connections with the interface map in Section 4.

# 7. Configure Cafe01-SW01

## 7.1 Apply basic settings and contain VTP

Enter on Cafe01-SW01:

enable configure terminal hostname Cafe01-SW01 no ip domain-lookup vtp mode transparent

Why VTP transparent mode? The Packet Tracer 2960 image supports transparent mode consistently. It prevents received VTP advertisements from changing the local VLAN database. On production IOS versions that support vtp mode off, off provides even stronger isolation.

## 7.2 Create and name the VLANs

vlan 10 name ADMIN_SERVICES exit vlan 20 name PATRON_SERVICES exit

Verify immediately: do show vlan brief

## 7.3 Assign the ADMIN_SERVICES access-port range

interface range fastEthernet 0/1 - 4 description ADMIN_SERVICES_ACCESS switchport mode access switchport access vlan 10 spanning-tree portfast spanning-tree bpduguard enable exit

## 7.4 Assign the PATRON_SERVICES access-port range

interface range fastEthernet 0/5 - 8 description PATRON_SERVICES_ACCESS switchport mode access switchport access vlan 20 spanning-tree portfast spanning-tree bpduguard enable exit

Access-port protection PortFast is appropriate on endpoint-facing ports, not on switch-to-switch trunks. BPDU Guard shuts an access port if another switch begins sending BPDUs, reducing the risk of an unauthorized switch joining the topology.

## 7.5 Lock down the router trunk on GigabitEthernet0/1

interface gigabitEthernet 0/1 description STATIC_TRUNK_TO_Cafe01-RT01_G0/1 switchport mode trunk switchport trunk allowed vlan 10,20 switchport nonegotiate no shutdown exit

## 7.6 Lock down the inter-switch trunk on GigabitEthernet0/2

interface gigabitEthernet 0/2 description STATIC_TRUNK_TO_Cafe01-SW02_G0/2 switchport mode trunk switchport trunk allowed vlan 10,20 switchport nonegotiate no shutdown exit end copy running-config startup-config

# 8. Configure Cafe01-SW02

## 8.1 Apply basic settings, VTP containment, and VLAN names

enable configure terminal hostname Cafe01-SW02 no ip domain-lookup vtp mode transparent vlan 10 name ADMIN_SERVICES exit vlan 20 name PATRON_SERVICES exit

## 8.2 Configure the two access-port ranges

interface range fastEthernet 0/1 - 4 description ADMIN_SERVICES_ACCESS switchport mode access switchport access vlan 10 spanning-tree portfast spanning-tree bpduguard enable exit interface range fastEthernet 0/5 - 8 description PATRON_SERVICES_ACCESS switchport mode access switchport access vlan 20 spanning-tree portfast spanning-tree bpduguard enable exit

## 8.3 Lock down the active inter-switch trunk

interface gigabitEthernet 0/2 description STATIC_TRUNK_TO_Cafe01-SW01_G0/2 switchport mode trunk switchport trunk allowed vlan 10,20 switchport nonegotiate no shutdown exit

## 8.4 Secure the unused GigabitEthernet0/1 port

The uploaded solution configures this port as a trunk even though no cable is attached. For a hardened student build, leave it disabled until it has an approved trunk peer:

interface gigabitEthernet 0/1 description UNUSED_UPLINK_SHUTDOWN shutdown exit end copy running-config startup-config

# 9. Configure the wireless access points

VLAN placement The access points are Layer 2 bridges in this topology. Their wired switch ports determine the VLAN for associated clients: WAP1 and WAP2 connect to FastEthernet0/5, which is assigned to VLAN 20.

## 9.1 Configure WAP1

Select WAP1 and open the Config tab.

Open the wireless settings and set the SSID to WAP1.

Select WPA2 Personal / WPA2-PSK with AES encryption.

Set the pre-shared key to password, matching the uploaded solution.

Confirm the wired Port 0 link to Cafe01-SW01 FastEthernet0/5 is up.

## 9.2 Configure WAP2

Select WAP2 and open the Config tab.

Set the SSID to WAP2.

Select WPA2 Personal / WPA2-PSK with AES encryption.

Set the pre-shared key to password.

Confirm the wired Port 0 link to Cafe01-SW02 FastEthernet0/5 is up.

# 10. Configure PC1 and PC2

## 10.1 Configure PC1

Select PC1, open Desktop, and select IP Configuration.

Select Static.

Enter IP address 10.0.18.21.

Enter subnet mask 255.255.255.192.

Enter default gateway 10.0.18.1.

Close the window. Packet Tracer saves the values automatically.

## 10.2 Configure PC2

Select PC2, open Desktop, and select IP Configuration.

Select Static.

Enter IP address 10.0.18.22.

Enter subnet mask 255.255.255.192.

Enter default gateway 10.0.18.1.

Close the window. Packet Tracer saves the values automatically.

# 11. Connect and address Laptop1 and Laptop2

## 11.1 Connect Laptop1 to WAP1

Select Laptop1, open Desktop, and select PC Wireless.

Open the Connect tab, refresh the wireless network list, and select SSID WAP1.

Enter the pre-shared key password and connect.

Return to Desktop > IP Configuration and select Static.

Enter IP address 10.0.18.31, subnet mask 255.255.255.192, and default gateway 10.0.18.1.

## 11.2 Connect Laptop2 to WAP2

Select Laptop2, open Desktop, and select PC Wireless.

Select SSID WAP2 and enter the pre-shared key password.

Return to Desktop > IP Configuration and select Static.

Enter IP address 10.0.18.32, subnet mask 255.255.255.192, and default gateway 10.0.18.1.

# 12. Verify the switch configuration

## 12.1 Verify VLAN names and access ports

show vlan brief

| Expected item | Expected result |
| --- | --- |
| VLAN 10 | Name ADMIN_SERVICES; FastEthernet0/1–0/4 assigned |
| VLAN 20 | Name PATRON_SERVICES; FastEthernet0/5–0/8 assigned |
| Gigabit uplinks | Not listed under an access VLAN because they are trunks |

## 12.2 Verify trunk state and allowed VLANs

show interfaces trunk show interfaces gigabitEthernet 0/2 switchport

| Field | Expected value |
| --- | --- |
| Administrative Mode | trunk |
| Operational Mode | trunk |
| Negotiation of Trunking | Off |
| Trunking VLANs Enabled | 10,20 |
| VLANs allowed and active | 10,20 |

## 12.3 Verify VTP containment

show vtp status

Expected: VTP Operating Mode: Transparent

## 12.4 Verify neighbor interfaces

show cdp neighbors

| Local device | Local interface | Neighbor | Neighbor interface |
| --- | --- | --- | --- |
| Cafe01-SW01 | GigabitEthernet0/1 | Cafe01-RT01 | GigabitEthernet0/1 |
| Cafe01-SW01 | GigabitEthernet0/2 | Cafe01-SW02 | GigabitEthernet0/2 |
| Cafe01-SW02 | GigabitEthernet0/2 | Cafe01-SW01 | GigabitEthernet0/2 |

# 13. Test connectivity and segmentation

Allow time for convergence The first ping may fail while ARP resolves or spanning tree finishes transitioning. Repeat the ping before treating one initial timeout as a configuration failure.

| Source | Destination | Command | Expected result | Reason |
| --- | --- | --- | --- | --- |
| PC1 | PC2 | ping 10.0.18.22 | Pass | Both endpoints are in VLAN 10 across the inter-switch trunk. |
| Laptop1 | Laptop2 | ping 10.0.18.32 | Pass | Both wireless bridges enter VLAN 20 and cross the trunk. |
| PC1 | Laptop1 | ping 10.0.18.31 | Fail | VLAN 10 and VLAN 20 are isolated without valid inter-VLAN routing. |
| Laptop1 | 10.0.18.1 | ping 10.0.18.1 | Fail with exact values | The gateway is attached to VLAN 10, not VLAN 20. |

# 14. Troubleshooting guide

| Symptom | Likely cause | Commands or action |
| --- | --- | --- |
| PC1 cannot ping PC2 | VLAN 10 missing, incorrect access VLAN, or VLAN 10 not allowed on trunk | show vlan brief; show interfaces trunk; inspect Fa0/2 on both switches |
| Laptop1 cannot ping Laptop2 | Wrong SSID/key, WAP port not in VLAN 20, or VLAN 20 missing from trunk | Verify wireless association; show vlan brief; show interfaces trunk |
| Trunk does not form | One side is access/dynamic, interface is shutdown, or cable uses the wrong port | show interfaces status; show interfaces switchport; compare CDP neighbors |
| Negotiation is still enabled | switchport nonegotiate was omitted or rejected before static mode was set | Enter switchport mode trunk first, then switchport nonegotiate |
| VLAN names differ between switches | VLAN was created without the name command or VTP changed it | Re-enter vlan 10 / name ADMIN_SERVICES and vlan 20 / name PATRON_SERVICES |
| Laptop cannot reach gateway | The required laptop address is in the VLAN 10 subnet while the laptop is in VLAN 20 | Use the corrected routed option in Section 15 |

# 15. Optional corrected inter-VLAN routing configuration

Use this section only when routed connectivity is required To route between VLAN 10 and VLAN 20, each VLAN must use a different IP subnet. Keep PC1 and PC2 in 10.0.18.0/26. Move the patron laptops into 10.0.18.64/26, which matches router subinterface G0/1.20.

## 15.1 Corrected routed addressing

| Device | VLAN | Corrected IP address | Subnet mask | Gateway |
| --- | --- | --- | --- | --- |
| PC1 | 10 | 10.0.18.21 | 255.255.255.192 | 10.0.18.1 |
| PC2 | 10 | 10.0.18.22 | 255.255.255.192 | 10.0.18.1 |
| Laptop1 | 20 | 10.0.18.81 | 255.255.255.192 | 10.0.18.65 |
| Laptop2 | 20 | 10.0.18.82 | 255.255.255.192 | 10.0.18.65 |

## 15.2 Configure router-on-a-stick on Cafe01-RT01

enable configure terminal interface gigabitEthernet 0/1 no ip address no shutdown exit interface gigabitEthernet 0/1.10 encapsulation dot1Q 10 ip address 10.0.18.1 255.255.255.192 exit interface gigabitEthernet 0/1.20 encapsulation dot1Q 20 ip address 10.0.18.65 255.255.255.192 exit end copy running-config startup-config

## 15.3 Verify routed connectivity

show ip interface brief show ip route connected

| Test | Expected result |
| --- | --- |
| PC1 → 10.0.18.65 | Pass; reaches the VLAN 20 gateway |
| Laptop1 → 10.0.18.1 | Pass; reaches the VLAN 10 gateway |
| PC1 → 10.0.18.81 | Pass; inter-VLAN routing is operational |
| Laptop1 → 10.0.18.82 | Pass; same-VLAN wireless traffic remains operational |

# 16. Final configuration checklist

☐  VLAN 10 is named ADMIN_SERVICES on both switches.

☐  VLAN 20 is named PATRON_SERVICES on both switches.

☐  FastEthernet0/1–0/4 are static access ports in VLAN 10.

☐  FastEthernet0/5–0/8 are static access ports in VLAN 20.

☐  GigabitEthernet0/2 is a static trunk on both switches.

☐  Cafe01-SW01 GigabitEthernet0/1 is a static trunk to the router.

☐  Only VLANs 10 and 20 are allowed across active trunks.

☐  DTP negotiation is disabled on active trunks.

☐  VTP mode is transparent on both switches.

☐  PC1 and PC2 can communicate across VLAN 10.

☐  Laptop1 and Laptop2 can communicate across VLAN 20.

☐  The running configuration has been saved to startup-config.

# Appendix A. Complete switch configurations

## A.1 Cafe01-SW01

enable configure terminal hostname Cafe01-SW01 no ip domain-lookup vtp mode transparent vlan 10 name ADMIN_SERVICES vlan 20 name PATRON_SERVICES interface range fastEthernet 0/1 - 4 description ADMIN_SERVICES_ACCESS switchport mode access switchport access vlan 10 spanning-tree portfast spanning-tree bpduguard enable interface range fastEthernet 0/5 - 8 description PATRON_SERVICES_ACCESS switchport mode access switchport access vlan 20 spanning-tree portfast spanning-tree bpduguard enable interface gigabitEthernet 0/1 description STATIC_TRUNK_TO_Cafe01-RT01_G0/1 switchport mode trunk switchport trunk allowed vlan 10,20 switchport nonegotiate no shutdown interface gigabitEthernet 0/2 description STATIC_TRUNK_TO_Cafe01-SW02_G0/2 switchport mode trunk switchport trunk allowed vlan 10,20 switchport nonegotiate no shutdown end copy running-config startup-config

## A.2 Cafe01-SW02

enable configure terminal hostname Cafe01-SW02 no ip domain-lookup vtp mode transparent vlan 10 name ADMIN_SERVICES vlan 20 name PATRON_SERVICES interface range fastEthernet 0/1 - 4 description ADMIN_SERVICES_ACCESS switchport mode access switchport access vlan 10 spanning-tree portfast spanning-tree bpduguard enable interface range fastEthernet 0/5 - 8 description PATRON_SERVICES_ACCESS switchport mode access switchport access vlan 20 spanning-tree portfast spanning-tree bpduguard enable interface gigabitEthernet 0/2 description STATIC_TRUNK_TO_Cafe01-SW01_G0/2 switchport mode trunk switchport trunk allowed vlan 10,20 switchport nonegotiate no shutdown interface gigabitEthernet 0/1 description UNUSED_UPLINK_SHUTDOWN shutdown end copy running-config startup-config

# Appendix B. Command reference

| Command | Purpose |
| --- | --- |
| show vlan brief | Shows VLAN names and access-port membership. |
| show interfaces trunk | Shows trunk ports and allowed VLANs. |
| show interfaces interface-id switchport | Shows port mode and DTP negotiation state. |
| show vtp status | Shows VTP mode and revision information. |
| show cdp neighbors | Shows adjacent Cisco devices and interfaces. |
| show interfaces status | Shows link, VLAN/trunk, duplex, and speed. |
| show mac address-table dynamic | Shows learned MAC addresses by VLAN and port. |
| copy running-config startup-config | Saves the configuration for the next reload. |