SKILL 12 LAB

# Using Trunks to Extend VLANs

(June 24)

Introductory CCNA Switching Lab

| OBJECTIVE | Lab Objective: Introduce CCNA students to VLAN trunking by configuring two Cisco Catalyst 2960 switches so VLAN 10 and VLAN 20 can extend across inter-switch links while remaining logically separated. |
| --- | --- |

Figure 1. Cafe-SW1 and Cafe-SW2 are connected by two inter-switch links: Fa0/1-to-Fa0/1 and Fa0/2-to-Fa0/2. Each switch uses Fa0/3 for its attached client device.

| Student | Date | Estimated Time | Difficulty |
| --- | --- | --- | --- |
| ________________________ | ________________ | 50-70 minutes | Introductory |

Lab focus: 802.1Q trunks  |  VLAN 10 ADMIN_SERVICES  |  VLAN 20 PATRON_DEVICES  |  STP redundancy

# 1. Lab Overview

## Purpose

A VLAN trunk is a Layer 2 link that carries traffic for more than one VLAN. On an IEEE 802.1Q trunk, the switch inserts a VLAN tag into Ethernet frames so the receiving switch can identify the correct VLAN. This lab demonstrates how VLAN 10 and VLAN 20 can span two switches over the same physical links without combining their broadcast domains.

| CONCEPT | Important Layer Distinction: VLANs operate at Layer 2. Communication between different VLANs requires Layer 3 routing. A trunk extends VLANs between switches; it does not route traffic from one VLAN to another. |
| --- | --- |

## Learning Objectives

Explain the difference between an access port and a trunk port.

Create VLAN 10 ADMIN_SERVICES and VLAN 20 PATRON_DEVICES on both switches.

Configure Fa0/1 and Fa0/2 as 802.1Q trunks on Cafe-SW1 and Cafe-SW2.

Limit the trunks to VLANs 10 and 20 and verify the allowed and active VLAN lists.

Assign Fa0/3 as an access port and test same-VLAN connectivity across the trunks.

Move Cafe-Client1 to VLAN 20 and verify that inter-VLAN communication fails without a router or multilayer switch.

Interpret Spanning Tree Protocol behavior when two parallel trunk links form a redundant Layer 2 path.

## Prerequisites and Equipment

Two Cisco Catalyst 2960 switches named Cafe-SW1 and Cafe-SW2.

Cafe-SW1 Fa0/1 connected to Cafe-SW2 Fa0/1.

Cafe-SW1 Fa0/2 connected to Cafe-SW2 Fa0/2.

Cafe-Admin1 connected to Cafe-SW1 Fa0/3 and Cafe-Client1 connected to Cafe-SW2 Fa0/3.

Console access to both switches and terminal access to both client devices.

Basic familiarity with Cisco IOS modes and IPv4 subnet masks.

## Topology and Addressing Plan

| Device | Interface | Connected To | VLAN / Mode | IPv4 Address |
| --- | --- | --- | --- | --- |
| Cafe-SW1 | Fa0/1 | Cafe-SW2 Fa0/1 | Trunk; allow 10,20 | Not assigned |
| Cafe-SW1 | Fa0/2 | Cafe-SW2 Fa0/2 | Trunk; allow 10,20 | Not assigned |
| Cafe-SW1 | Fa0/3 | Cafe-Admin1 Fa0 | Access VLAN 10 | Host: 10.0.18.2/27 |
| Cafe-SW2 | Fa0/1 | Cafe-SW1 Fa0/1 | Trunk; allow 10,20 | Not assigned |
| Cafe-SW2 | Fa0/2 | Cafe-SW1 Fa0/2 | Trunk; allow 10,20 | Not assigned |
| Cafe-SW2 | Fa0/3 | Cafe-Client1 Fa0 | Phase 1: VLAN 10; Phase 2: VLAN 20 | 10.0.18.3/27, then 10.0.18.34/27 |

## Key Terms

| Term | Meaning |
| --- | --- |
| Access port | Carries traffic for one VLAN. Frames sent to an end device are normally untagged. |
| Trunk port | Carries traffic for multiple VLANs between network devices. |
| 802.1Q tag | A field added to an Ethernet frame to identify its VLAN. |
| Native VLAN | The VLAN whose frames are sent untagged on an 802.1Q trunk. This lab leaves the native VLAN at the default VLAN 1. |
| STP | Spanning Tree Protocol prevents Layer 2 loops by placing a redundant link into a blocking or alternate state. |

# 2. Task 0 - Inspect the Starting Configuration

Before configuring VLAN trunks, verify the physical links and record the starting VLAN and interface state on both switches.

## Step 1: Verify the Switch Names and Interfaces

Cafe-SW1> enable Cafe-SW1# show interfaces status Cafe-SW1# show vlan brief Cafe-SW1# show cdp neighbors Cafe-SW2> enable Cafe-SW2# show interfaces status Cafe-SW2# show vlan brief Cafe-SW2# show cdp neighbors

## Step 2: Confirm the Cabling

Fa0/1 on Cafe-SW1 must connect to Fa0/1 on Cafe-SW2.

Fa0/2 on Cafe-SW1 must connect to Fa0/2 on Cafe-SW2.

Fa0/3 on Cafe-SW1 must connect to Cafe-Admin1.

Fa0/3 on Cafe-SW2 must connect to Cafe-Client1.

| CHECK | Baseline Expectation: Before configuration, the switch ports normally belong to VLAN 1 and the inter-switch links are not statically configured as trunks. |
| --- | --- |

## Student Observation

Which interfaces are physically connected on Cafe-SW1 and Cafe-SW2?

____________________________________________________________________________________________________________

____________________________________________________________________________________________________________

Which VLAN currently contains Fa0/1, Fa0/2, and Fa0/3?

____________________________________________________________________________________________________________

____________________________________________________________________________________________________________

## Cafe-SW2 Configuration

Cafe-SW2# configure terminal Cafe-SW2(config)# vlan 10 Cafe-SW2(config-vlan)# name ADMIN_SERVICES Cafe-SW2(config-vlan)# exit Cafe-SW2(config)# vlan 20 Cafe-SW2(config-vlan)# name PATRON_DEVICES Cafe-SW2(config-vlan)# end Cafe-SW2# show vlan brief

| VERIFY | Expected Result: Both switches should list VLAN 10 ADMIN_SERVICES and VLAN 20 PATRON_DEVICES as active. No access ports are assigned to these VLANs yet. |
| --- | --- |

# 3. Task 1 - Create VLAN 10 and VLAN 20

VLANs are locally significant to each switch. Create the same VLAN IDs and names on both switches before configuring the trunks.

## Cafe-SW1 Configuration

Cafe-SW1# configure terminal Cafe-SW1(config)# vlan 10 Cafe-SW1(config-vlan)# name ADMIN_SERVICES Cafe-SW1(config-vlan)# exit Cafe-SW1(config)# vlan 20 Cafe-SW1(config-vlan)# name PATRON_DEVICES Cafe-SW1(config-vlan)# end Cafe-SW1# show vlan brief

# 4. Task 2 - Configure the Inter-Switch Trunks

Configure both parallel inter-switch links as static trunks. The allowed VLAN list is restricted to VLANs 10 and 20 so only the lab VLANs traverse the links.

| 2960 NOTE | Catalyst 2960 Command Note: The Cisco Catalyst 2960 uses IEEE 802.1Q trunking and does not require the command switchport trunk encapsulation dot1q. On many 2960 models, that command is unavailable. Configure the interface directly with switchport mode trunk. |
| --- | --- |

## Step 1: Configure Fa0/1 and Fa0/2 on Cafe-SW1

Cafe-SW1# configure terminal Cafe-SW1(config)# interface range fa0/1 - 2 Cafe-SW1(config-if-range)# description TRUNKS_TO_CAFE-SW2 Cafe-SW1(config-if-range)# switchport mode trunk Cafe-SW1(config-if-range)# switchport trunk allowed vlan 10,20 Cafe-SW1(config-if-range)# no shutdown Cafe-SW1(config-if-range)# end

## Step 2: Configure Fa0/1 and Fa0/2 on Cafe-SW2

Cafe-SW2# configure terminal Cafe-SW2(config)# interface range fa0/1 - 2 Cafe-SW2(config-if-range)# description TRUNKS_TO_CAFE-SW1 Cafe-SW2(config-if-range)# switchport mode trunk Cafe-SW2(config-if-range)# switchport trunk allowed vlan 10,20 Cafe-SW2(config-if-range)# no shutdown Cafe-SW2(config-if-range)# end

## Step 3: Verify the Trunks

Cafe-SW1# show interfaces trunk Cafe-SW1# show interfaces fa0/1 switchport Cafe-SW1# show interfaces fa0/2 switchport Cafe-SW2# show interfaces trunk Cafe-SW2# show interfaces fa0/1 switchport Cafe-SW2# show interfaces fa0/2 switchport

Expected trunk summary (abbreviated):

Port Mode Encapsulation Status Native vlan Fa0/1 on 802.1q trunking 1 Fa0/2 on 802.1q trunking 1 Port Vlans allowed on trunk Fa0/1 10,20 Fa0/2 10,20 Port Vlans allowed and active in management domain Fa0/1 10,20 Fa0/2 10,20

## Step 4: Observe Spanning Tree Behavior

Two separate trunk links between the same switches create a redundant Layer 2 path. STP should prevent a switching loop by placing one path into a blocking or alternate state for each VLAN.

Cafe-SW1# show spanning-tree vlan 10 Cafe-SW1# show spanning-tree vlan 20 Cafe-SW2# show spanning-tree vlan 10 Cafe-SW2# show spanning-tree vlan 20

| STP | Normal STP Result: Both interfaces may appear as trunking in show interfaces trunk, but one of the two links can be blocked by STP. This is expected. To make both physical links forward traffic as one logical link, configure EtherChannel in a later lab. |
| --- | --- |

## Trunk Checkpoint

☐ Fa0/1 is a trunk on both switches.

☐ Fa0/2 is a trunk on both switches.

☐ VLANs 10 and 20 are allowed and active on the trunks.

☐ The native VLAN matches on both ends.

☐ STP prevents the two parallel links from forming a Layer 2 loop.

# 5. Task 3 - Extend VLAN 10 Across the Trunks

Place both client-facing Fa0/3 ports in VLAN 10. The two hosts should then share the same Layer 2 broadcast domain even though they connect to different switches.

## Step 1: Assign Cafe-SW1 Fa0/3 to VLAN 10

Cafe-SW1# configure terminal Cafe-SW1(config)# interface fa0/3 Cafe-SW1(config-if)# description CAFE_ADMIN1_VLAN10 Cafe-SW1(config-if)# switchport mode access Cafe-SW1(config-if)# switchport access vlan 10 Cafe-SW1(config-if)# spanning-tree portfast Cafe-SW1(config-if)# no shutdown Cafe-SW1(config-if)# end Cafe-SW1# show vlan brief

## Step 2: Assign Cafe-SW2 Fa0/3 to VLAN 10

Cafe-SW2# configure terminal Cafe-SW2(config)# interface fa0/3 Cafe-SW2(config-if)# description CAFE_CLIENT1_VLAN10 Cafe-SW2(config-if)# switchport mode access Cafe-SW2(config-if)# switchport access vlan 10 Cafe-SW2(config-if)# spanning-tree portfast Cafe-SW2(config-if)# no shutdown Cafe-SW2(config-if)# end Cafe-SW2# show vlan brief

| CAUTION | PortFast Safety: Use spanning-tree portfast only on ports connected to end devices. Do not enable PortFast on the inter-switch trunk links in this lab. |
| --- | --- |

## Step 3: Configure the Client Addresses

On Cafe-Admin1, log in as tc and enter:

sudo ifconfig eth0 10.0.18.2 netmask 255.255.255.224 up

On Cafe-Client1, log in as tc and enter:

sudo ifconfig eth0 10.0.18.3 netmask 255.255.255.224 up

| NOTE | Gateway Note: A default gateway is not required for this same-subnet ping test. No router or multilayer switch is present in the topology. |
| --- | --- |

## Step 4: Test Same-VLAN Connectivity

tc@cafe-admin1:~$ ping -c 5 10.0.18.3 Expected result: 5 packets transmitted, 5 packets received, 0% packet loss

## Step 5: Verify MAC Address Learning

Cafe-SW1# show mac address-table dynamic vlan 10 Cafe-SW2# show mac address-table dynamic vlan 10

Each switch should learn the directly attached host MAC address on Fa0/3 and the remote host MAC address through the forwarding trunk port.

# 6. Task 4 - Move Cafe-Client1 to VLAN 20

Reassign Cafe-SW2 Fa0/3 to VLAN 20 and update Cafe-Client1 with an address from the VLAN 20 subnet. The ping from Cafe-Admin1 should fail because this topology contains no Layer 3 device for inter-VLAN routing.

## Step 1: Reassign Cafe-SW2 Fa0/3

Cafe-SW2# configure terminal Cafe-SW2(config)# interface fa0/3 Cafe-SW2(config-if)# description CAFE_CLIENT1_VLAN20 Cafe-SW2(config-if)# switchport access vlan 20 Cafe-SW2(config-if)# end Cafe-SW2# show vlan brief

## Step 2: Change the Client Address

tc@cafe-client1:~$ sudo ifconfig eth0 10.0.18.34 netmask 255.255.255.224 up

## Step 3: Test Across the VLAN Boundary

tc@cafe-admin1:~$ ping -c 5 10.0.18.34 Expected result: 5 packets transmitted, 0 packets received, 100% packet loss

| EXPLANATION | Why the Ping Fails: VLAN 10 and VLAN 20 are separate Layer 2 broadcast domains. Cafe-Admin1 cannot use ARP to reach a host in VLAN 20 through Layer 2 switching alone. A router or multilayer switch must perform inter-VLAN routing. |
| --- | --- |

## Verification Commands

Cafe-SW1# show vlan brief Cafe-SW1# show interfaces trunk Cafe-SW1# show spanning-tree vlan 10 Cafe-SW1# show mac address-table dynamic Cafe-SW2# show vlan brief Cafe-SW2# show interfaces trunk Cafe-SW2# show spanning-tree vlan 20 Cafe-SW2# show mac address-table dynamic

## Expected Port Membership

| Switch | Port | Expected Role | Expected VLANs |
| --- | --- | --- | --- |
| Cafe-SW1 | Fa0/1 | Trunk to Cafe-SW2 Fa0/1 | 10,20 |
| Cafe-SW1 | Fa0/2 | Trunk to Cafe-SW2 Fa0/2 | 10,20 |
| Cafe-SW1 | Fa0/3 | Access port to Cafe-Admin1 | 10 |
| Cafe-SW2 | Fa0/1 | Trunk to Cafe-SW1 Fa0/1 | 10,20 |
| Cafe-SW2 | Fa0/2 | Trunk to Cafe-SW1 Fa0/2 | 10,20 |
| Cafe-SW2 | Fa0/3 | Access port to Cafe-Client1 | 20 |

# 7. Troubleshooting Guide

| Symptom | Likely Cause | Corrective Action |
| --- | --- | --- |
| switchport trunk encapsulation dot1q is rejected | Catalyst 2960 uses fixed 802.1Q encapsulation. | Omit the encapsulation command and use switchport mode trunk. |
| interface range command is rejected | Incorrect range syntax. | Use interface range fa0/1 - 2 with spaces around the hyphen. |
| VLAN 10 or 20 is missing from active trunk VLANs | The VLAN was not created locally on one switch. | Create the VLAN on both switches and verify with show vlan brief. |
| One trunk is not forwarding user traffic | STP blocked the redundant path. | This is expected with two separate parallel links. Verify with show spanning-tree vlan 10. |
| Same-VLAN ping fails | Access port VLAN mismatch, trunk mismatch, wrong IP/mask, or blocked physical link. | Check show vlan brief, show interfaces trunk, show interfaces status, and host addressing. |
| Different-VLAN ping fails | No Layer 3 routing is configured. | This is the expected result. Add a router or multilayer switch only in a later inter-VLAN routing lab. |

| SAFETY | Do Not Disable STP: Do not solve a redundant-link problem by disabling Spanning Tree Protocol. STP protects the network from broadcast storms and MAC address-table instability. |
| --- | --- |

# 8. Completion Check

☐ VLAN 10 ADMIN_SERVICES exists on Cafe-SW1 and Cafe-SW2.

☐ VLAN 20 PATRON_DEVICES exists on Cafe-SW1 and Cafe-SW2.

☐ Fa0/1 and Fa0/2 are configured as trunks on both switches.

☐ The trunks allow VLANs 10 and 20.

☐ Fa0/3 on Cafe-SW1 is an access port in VLAN 10.

☐ Fa0/3 on Cafe-SW2 is an access port in VLAN 20 at the end of the lab.

☐ Same-VLAN communication succeeded while both hosts were in VLAN 10.

☐ Communication failed after Cafe-Client1 moved to VLAN 20.

☐ STP behavior was verified for the two parallel trunk links.

☐ The switch configurations were saved.

Save the Configuration

Cafe-SW1# copy running-config startup-config Cafe-SW2# copy running-config startup-config

# 9. Knowledge Check and Reflection

1. What is the main purpose of a VLAN trunk?

____________________________________________________________________________________________________________

____________________________________________________________________________________________________________

2. What information does an IEEE 802.1Q tag provide to the receiving switch?

____________________________________________________________________________________________________________

____________________________________________________________________________________________________________

3. Why can two devices in separate VLANs not communicate through Layer 2 switching alone?

____________________________________________________________________________________________________________

____________________________________________________________________________________________________________

4. Why might one of the two parallel trunk links be placed into a blocking or alternate state?

____________________________________________________________________________________________________________

____________________________________________________________________________________________________________

5. What is the difference between an access port and a trunk port?

____________________________________________________________________________________________________________

____________________________________________________________________________________________________________

6. Why is the command switchport trunk encapsulation dot1q normally unnecessary on a Catalyst 2960?

____________________________________________________________________________________________________________

____________________________________________________________________________________________________________

7. Which device or feature would be required to allow VLAN 10 and VLAN 20 to communicate?

____________________________________________________________________________________________________________

____________________________________________________________________________________________________________

# 10. Instructor Answer Key

1. Purpose of a trunk

A trunk carries traffic for multiple VLANs over one physical link between network devices.

2. 802.1Q tag

The tag identifies the VLAN to which the Ethernet frame belongs.

3. Separate VLAN communication

Each VLAN is a separate Layer 2 broadcast domain. Layer 2 switches do not route packets between VLANs, so a router or multilayer switch is required.

4. One redundant link blocked

STP blocks one path to prevent a Layer 2 loop. Both links can be combined and used simultaneously only after configuring an EtherChannel.

5. Access versus trunk

An access port carries one VLAN for an end device; a trunk carries multiple VLANs between network devices.

6. 2960 encapsulation command

The Catalyst 2960 supports 802.1Q as its trunk encapsulation, so there is no encapsulation choice to configure on many models.

7. Inter-VLAN communication

A router, router-on-a-stick configuration, or a multilayer switch with switched virtual interfaces can provide inter-VLAN routing.

| FINAL | End State: Cafe-SW1 Fa0/3 remains in VLAN 10, Cafe-SW2 Fa0/3 ends in VLAN 20, and Fa0/1 plus Fa0/2 remain trunks that carry VLANs 10 and 20. Inter-VLAN routing is intentionally not configured. |
| --- | --- |