CCNA Lab Manual

Part 1 - Configuring a Static IPv4 Network

District Shop #1 - Single-Broadcast-Domain Baseline

| Student: ____________________ | Date: ____________________ | Lab Time: __________ |
| --- | --- | --- |

Figure 1. District Shop #1 static IPv4 topology

# Lab Overview

Purpose. Build the network shown in Figure 1, configure a single IPv4 LAN with static addressing, configure routing to the simulated Internet router, and verify end-to-end connectivity to 1.1.1.1 and 8.8.8.8.

Baseline design: This lab intentionally keeps all switch ports in VLAN 1. It establishes a working static-IP baseline before later VLAN, trunking, inter-VLAN routing, and DHCP exercises.

## Learning Objectives

Cable the routers, switches, wired hosts, wireless access points, and laptops according to the topology.

Configure static IPv4 addresses, subnet masks, and default gateways.

Configure Layer 2 switch ports for one flat LAN.

Configure a default route on Cafe01-RT01 and a return route on the Internet router.

Use show, ping, and tracert/traceroute commands to verify connectivity and isolate failures.

## Equipment and Assumptions

Two Cisco 2900-series routers: Cafe01-RT01 and Internet.

Two Cisco Catalyst 2960 switches: Cafe01-SW01 and Cafe01-SW02.

Two PCs, one server, two wireless access points, and two wireless laptops.

Packet Tracer interface names may vary. Use show ip interface brief and show interfaces status to confirm the actual ports.

The simulated Internet router uses loopback addresses 1.1.1.1 and 8.8.8.8. NAT is not required because the Internet router receives a return route to the private LAN.

## Addressing Plan

| Device | Interface | IPv4 Address | Mask / Prefix | Default Gateway | Purpose |
| --- | --- | --- | --- | --- | --- |
| Cafe01-RT01 | Gi0/1 | 10.0.18.1 | 255.255.255.192 /26 | - | LAN gateway |
| WAP1 | Fa0 | 10.0.18.5 | 255.255.255.192 /26 | 10.0.18.1 | Wireless AP 1 management |
| Server | Fa0 | 10.0.18.6 | 255.255.255.192 /26 | 10.0.18.1 | Static server |
| WAP2 | Fa0 | 10.0.18.7 | 255.255.255.192 /26 | 10.0.18.1 | Wireless AP 2 management |
| PC1 | Fa0 | 10.0.18.21 | 255.255.255.192 /26 | 10.0.18.1 | Wired client |
| PC2 | Fa0 | 10.0.18.22 | 255.255.255.192 /26 | 10.0.18.1 | Wired client |
| Laptop1 | Wireless0 | 10.0.18.32 | 255.255.255.192 /26 | 10.0.18.1 | Wireless client |
| Laptop2 | Wireless0 | 10.0.18.33 | 255.255.255.192 /26 | 10.0.18.1 | Wireless client |
| Cafe01-RT01 | Gi0/0 | 216.0.5.2 | 255.255.255.252 /30 | - | WAN link |
| Internet | Gi0/0 | 216.0.5.1 | 255.255.255.252 /30 | - | WAN peer |
| Internet | Loopback0 | 1.1.1.1 | 255.255.255.255 /32 | - | Simulated Internet host |
| Internet | Loopback1 | 8.8.8.8 | 255.255.255.255 /32 | - | Simulated Internet host |

Subnet check: The LAN is 10.0.18.0/26. Usable host addresses are 10.0.18.1-10.0.18.62, and 10.0.18.63 is the broadcast address.

## Physical Port Map

| Local Device | Local Port | Remote Device | Remote Port |
| --- | --- | --- | --- |
| Cafe01-RT01 | Gi0/0 | Internet | Gi0/0 |
| Cafe01-RT01 | Gi0/1 | Cafe01-SW01 | Fa0/1 |
| Cafe01-SW01 | Fa0/2 | PC1 | Fa0 |
| Cafe01-SW01 | Fa0/3 | WAP1 | Fa0 |
| Cafe01-SW01 | Gi0/2 | Cafe01-SW02 | Gi0/2 |
| Cafe01-SW02 | Fa0/1 | Server | Fa0 |
| Cafe01-SW02 | Fa0/2 | PC2 | Fa0 |
| Cafe01-SW02 | Fa0/3 | WAP2 | Fa0 |

# Part 1 - Build and Inspect the Topology

## Step 1: Place and Cable the Devices

1.  Place the two routers, two switches, two PCs, one server, two access points, and two laptops as shown in Figure 1.

2.  Connect Cafe01-RT01 Gi0/0 to Internet Gi0/0.

3.  Connect Cafe01-RT01 Gi0/1 to Cafe01-SW01 Fa0/1.

4.  Connect Cafe01-SW01 Gi0/2 to Cafe01-SW02 Gi0/2.

5.  Connect PC1 to Cafe01-SW01 Fa0/2 and WAP1 to Cafe01-SW01 Fa0/3.

6.  Connect the Server to Cafe01-SW02 Fa0/1, PC2 to Fa0/2, and WAP2 to Fa0/3.

7.  Associate Laptop1 with WAP1 and Laptop2 with WAP2 after configuring the wireless settings.

Cable selection: Packet Tracer generally supports automatic MDI/MDIX. Copper straight-through cables are appropriate for unlike devices; the switch-to-switch link may use crossover or automatic selection.

## Step 2: Record the Initial Interface State

On each router:

enable show ip interface brief

On each switch:

enable show interfaces status show vlan brief

Before configuration, router interfaces may be administratively down. Switch ports connected to powered devices should eventually show connected/up.

## Checkpoint 1

| Check | Expected Result | Student Initials |
| --- | --- | --- |
| All devices are placed and cabled | Matches Figure 1 |  |
| Router-to-router link | Gi0/0 <-> Gi0/0 |  |
| Router-to-switch link | RT01 Gi0/1 <-> SW01 Fa0/1 |  |
| Inter-switch link | SW01 Gi0/2 <-> SW02 Gi0/2 |  |

# Part 2 - Configure the Internet Router

Goal. Configure the WAN address, create two loopbacks that simulate Internet destinations, and add a return route to the District Shop LAN.

## Step 1: Configure Interfaces and Loopbacks

Internet> enable Internet# configure terminal Internet(config)# hostname Internet Internet(config)# interface GigabitEthernet0/0 Internet(config-if)# description TO_CAFE01-RT01 Internet(config-if)# ip address 216.0.5.1 255.255.255.252 Internet(config-if)# no shutdown Internet(config-if)# exit Internet(config)# interface Loopback0 Internet(config-if)# description SIMULATED_INTERNET_1.1.1.1 Internet(config-if)# ip address 1.1.1.1 255.255.255.255 Internet(config-if)# exit Internet(config)# interface Loopback1 Internet(config-if)# description SIMULATED_INTERNET_8.8.8.8 Internet(config-if)# ip address 8.8.8.8 255.255.255.255 Internet(config-if)# exit

## Step 2: Correct the Return Route

Important: The supplied router configuration contained host routes to 216.0.5.20 and 216.0.5.21. Those destinations are not the internal LAN. Replace them with one route to 10.0.18.0/26.

Internet(config)# no ip route 216.0.5.20 255.255.255.255 216.0.5.2 Internet(config)# no ip route 216.0.5.21 255.255.255.255 216.0.5.2 Internet(config)# ip route 10.0.18.0 255.255.255.192 216.0.5.2 Internet(config)# end Internet# copy running-config startup-config

## Step 3: Verify the Internet Router

Internet# show ip interface brief Internet# show ip route

Expected route: S 10.0.18.0/26 [1/0] via 216.0.5.2

# Part 3 - Configure Cafe01-RT01

Goal. Configure the LAN gateway, configure the WAN connection, and add a default route to the Internet router.

## Step 1: Configure Gi0/0 and Gi0/1

Cafe01-RT01> enable Cafe01-RT01# configure terminal Cafe01-RT01(config)# hostname Cafe01-RT01 Cafe01-RT01(config)# interface GigabitEthernet0/0 Cafe01-RT01(config-if)# description WAN_TO_INTERNET Cafe01-RT01(config-if)# ip address 216.0.5.2 255.255.255.252 Cafe01-RT01(config-if)# no shutdown Cafe01-RT01(config-if)# exit Cafe01-RT01(config)# interface GigabitEthernet0/1 Cafe01-RT01(config-if)# description LAN_TO_CAFE01-SW01 Cafe01-RT01(config-if)# ip address 10.0.18.1 255.255.255.192 Cafe01-RT01(config-if)# no shutdown Cafe01-RT01(config-if)# exit

## Step 2: Configure the Default Route

Cafe01-RT01(config)# ip route 0.0.0.0 0.0.0.0 216.0.5.1 Cafe01-RT01(config)# end Cafe01-RT01# copy running-config startup-config

Routing logic: Cafe01-RT01 uses a default route for all unknown destinations. The Internet router uses a specific return route to send replies back to 10.0.18.0/26.

## Step 3: Verify Cafe01-RT01

Cafe01-RT01# show ip interface brief Cafe01-RT01# show ip route Cafe01-RT01# ping 216.0.5.1 Cafe01-RT01# ping 8.8.8.8

Expected: Gi0/0 and Gi0/1 are up/up, the default route points to 216.0.5.1, and both pings succeed.

# Part 4 - Configure the Layer 2 Switches

Layer 2 note: The switches do not require IP addresses to forward Ethernet frames. All connected ports remain in VLAN 1 for this baseline lab.

## Step 1: Configure Cafe01-SW01

Switch> enable Switch# configure terminal Switch(config)# hostname Cafe01-SW01 Cafe01-SW01(config)# interface FastEthernet0/1 Cafe01-SW01(config-if)# description TO_CAFE01-RT01_G0/1 Cafe01-SW01(config-if)# switchport mode access Cafe01-SW01(config-if)# switchport access vlan 1 Cafe01-SW01(config-if)# no shutdown Cafe01-SW01(config-if)# exit Cafe01-SW01(config)# interface FastEthernet0/2 Cafe01-SW01(config-if)# description TO_PC1 Cafe01-SW01(config-if)# switchport mode access Cafe01-SW01(config-if)# switchport access vlan 1 Cafe01-SW01(config-if)# no shutdown Cafe01-SW01(config-if)# exit Cafe01-SW01(config)# interface FastEthernet0/3 Cafe01-SW01(config-if)# description TO_WAP1 Cafe01-SW01(config-if)# switchport mode access Cafe01-SW01(config-if)# switchport access vlan 1 Cafe01-SW01(config-if)# no shutdown Cafe01-SW01(config-if)# exit Cafe01-SW01(config)# interface GigabitEthernet0/2 Cafe01-SW01(config-if)# description TO_CAFE01-SW02_G0/2 Cafe01-SW01(config-if)# switchport mode access Cafe01-SW01(config-if)# switchport access vlan 1 Cafe01-SW01(config-if)# no shutdown Cafe01-SW01(config-if)# end Cafe01-SW01# copy running-config startup-config

## Step 2: Configure Cafe01-SW02

Switch> enable Switch# configure terminal Switch(config)# hostname Cafe01-SW02 Cafe01-SW02(config)# interface FastEthernet0/1 Cafe01-SW02(config-if)# description TO_SERVER Cafe01-SW02(config-if)# switchport mode access Cafe01-SW02(config-if)# switchport access vlan 1 Cafe01-SW02(config-if)# no shutdown Cafe01-SW02(config-if)# exit Cafe01-SW02(config)# interface FastEthernet0/2 Cafe01-SW02(config-if)# description TO_PC2 Cafe01-SW02(config-if)# switchport mode access Cafe01-SW02(config-if)# switchport access vlan 1 Cafe01-SW02(config-if)# no shutdown Cafe01-SW02(config-if)# exit Cafe01-SW02(config)# interface FastEthernet0/3 Cafe01-SW02(config-if)# description TO_WAP2 Cafe01-SW02(config-if)# switchport mode access Cafe01-SW02(config-if)# switchport access vlan 1 Cafe01-SW02(config-if)# no shutdown Cafe01-SW02(config-if)# exit Cafe01-SW02(config)# interface GigabitEthernet0/2 Cafe01-SW02(config-if)# description TO_CAFE01-SW01_G0/2 Cafe01-SW02(config-if)# switchport mode access Cafe01-SW02(config-if)# switchport access vlan 1 Cafe01-SW02(config-if)# no shutdown Cafe01-SW02(config-if)# end Cafe01-SW02# copy running-config startup-config

## Step 3: Verify the Switches

Cafe01-SW01# show interfaces status Cafe01-SW01# show vlan brief Cafe01-SW02# show interfaces status Cafe01-SW02# show vlan brief

Expected: The connected ports show connected, and Fa0/1-Fa0/3 plus Gi0/2 appear in VLAN 1.

# Part 5 - Configure Static IP Addresses

## Step 1: Configure PC1, PC2, and the Server

1.  Open the device.

2.  Select Desktop > IP Configuration.

3.  Select Static.

4.  Enter the IPv4 address, subnet mask, default gateway, and optional DNS address shown in the table below.

| Device | IP Address | Subnet Mask | Gateway | DNS (optional) |
| --- | --- | --- | --- | --- |
| Server | 10.0.18.6 | 255.255.255.192 | 10.0.18.1 | 8.8.8.8 |
| PC1 | 10.0.18.21 | 255.255.255.192 | 10.0.18.1 | 8.8.8.8 |
| PC2 | 10.0.18.22 | 255.255.255.192 | 10.0.18.1 | 8.8.8.8 |

## Step 2: Configure WAP1 and WAP2

1.  Open WAP1 and select Config.

2.  Assign WAP1 the address 10.0.18.5, mask 255.255.255.192, and gateway 10.0.18.1.

3.  Configure an SSID such as DISTRICT-SHOP-WAP1. Use instructor-provided wireless security settings.

4.  Open WAP2 and assign 10.0.18.7, mask 255.255.255.192, and gateway 10.0.18.1.

5.  Configure an SSID such as DISTRICT-SHOP-WAP2 using the same approved security method.

Packet Tracer variation: The exact access-point tabs vary by device model. Configure the wired management interface and the wireless SSID. The AP must not run a conflicting DHCP service in this static-address lab.

## Step 3: Configure Laptop1 and Laptop2

1.  Associate Laptop1 with DISTRICT-SHOP-WAP1.

2.  Assign Laptop1 10.0.18.32/26 with gateway 10.0.18.1.

3.  Associate Laptop2 with DISTRICT-SHOP-WAP2.

4.  Assign Laptop2 10.0.18.33/26 with gateway 10.0.18.1.

| Device | IP Address | Subnet Mask | Gateway | SSID |
| --- | --- | --- | --- | --- |
| Laptop1 | 10.0.18.32 | 255.255.255.192 | 10.0.18.1 | DISTRICT-SHOP-WAP1 |
| Laptop2 | 10.0.18.33 | 255.255.255.192 | 10.0.18.1 | DISTRICT-SHOP-WAP2 |

# Part 6 - Verify Connectivity

## Step 1: Verify Local LAN Connectivity

From PC1, open Desktop > Command Prompt and run:

ipconfig ping 10.0.18.1 ping 10.0.18.6 ping 10.0.18.22 ping 10.0.18.32 ping 10.0.18.33

First-ping behavior: The first echo may time out while ARP resolves a MAC address. Repeat the ping before concluding that the path is broken.

## Step 2: Verify the WAN and Simulated Internet

ping 216.0.5.2 ping 216.0.5.1 ping 1.1.1.1 ping 8.8.8.8 tracert 8.8.8.8

| Test | What It Verifies | Expected |
| --- | --- | --- |
| Ping 10.0.18.1 | Access port, local switching, and LAN gateway | Success |
| Ping another 10.0.18.x host | Layer 2 forwarding across one or both switches | Success |
| Ping 216.0.5.2 | Cafe01-RT01 routes from LAN to its WAN interface | Success |
| Ping 216.0.5.1 | WAN link and Internet-router return route | Success |
| Ping 8.8.8.8 | Complete end-to-end simulated Internet routing | Success |

## Step 3: Verify Routing from Both Routers

Cafe01-RT01:

show ip interface brief show ip route ping 216.0.5.1 ping 8.8.8.8

Internet router:

show ip interface brief show ip route ping 216.0.5.2 ping 10.0.18.1

Key lesson: A working forward path is not enough. The Internet router needs the route 10.0.18.0/26 via 216.0.5.2 so replies can return to the private LAN.

# Part 7 - Troubleshooting Guide

| Symptom | Likely Cause | Corrective Action |
| --- | --- | --- |
| PC cannot ping 10.0.18.1 | Wrong IP/mask/gateway, access port down, or router Gi0/1 down | Verify static settings, cabling, switch status, and no shutdown. |
| PC can ping 216.0.5.2 but not 216.0.5.1 | Internet router lacks return route to 10.0.18.0/26 | Configure ip route 10.0.18.0 255.255.255.192 216.0.5.2. |
| Router can ping 8.8.8.8 but PC cannot | PC gateway is missing or return route is incorrect | Verify 10.0.18.1 on the PC and the Internet-router route. |
| Hosts on different switches cannot ping | Gi0/2 link is down or ports are in different VLANs | Verify both Gi0/2 ports are connected/up and in VLAN 1. |
| Laptop cannot reach the LAN | SSID/security mismatch or wrong static IP | Reconnect to the correct SSID and verify /26 mask and gateway. |
| Intermittent or incorrect replies | Duplicate static IP address | Check the addressing table and assign every device a unique address. |
| 169.254.x.x address appears | The device is using DHCP and failed to obtain a lease | Select Static and enter the assigned address manually. |

## Useful Diagnostic Commands

! Router show ip interface brief show ip route show running-config ping <destination> traceroute <destination> ! Switch show interfaces status show vlan brief show mac address-table show cdp neighbors ! PC / Laptop ipconfig ping <destination> tracert <destination>

# Lab Completion Checklist

| Completion Item | Complete |
| --- | --- |
| Cafe01-RT01 Gi0/1 is 10.0.18.1/26 and is up/up. | [ ] |
| Cafe01-RT01 Gi0/0 is 216.0.5.2/30 and is up/up. | [ ] |
| Internet Gi0/0 is 216.0.5.1/30 and is up/up. | [ ] |
| Internet has Loopback0 1.1.1.1/32 and Loopback1 8.8.8.8/32. | [ ] |
| Cafe01-RT01 has a default route via 216.0.5.1. | [ ] |
| Internet has a return route to 10.0.18.0/26 via 216.0.5.2. | [ ] |
| All wired and wireless devices have the assigned static IPv4 settings. | [ ] |
| PC1 can ping PC2, Server, Laptop1, and Laptop2. | [ ] |
| PC1 and PC2 can ping 8.8.8.8. | [ ] |
| Router and switch configurations are saved. | [ ] |

# Knowledge Check

1. What is the usable host range for 10.0.18.0/26?

________________________________________________________________________________

________________________________________________________________________________

2. Why is 10.0.18.63 not a valid host address?

________________________________________________________________________________

________________________________________________________________________________

3. Why does Cafe01-RT01 need a default route?

________________________________________________________________________________

________________________________________________________________________________

4. Why does the Internet router need a return route to 10.0.18.0/26?

________________________________________________________________________________

________________________________________________________________________________

5. Why can the two switches forward LAN traffic without having management IP addresses?

________________________________________________________________________________

________________________________________________________________________________

6. What does a successful ping to 216.0.5.2 prove? What additional function is tested by pinging 216.0.5.1?

________________________________________________________________________________

________________________________________________________________________________

# Student Reflection

Describe one configuration error you encountered and the command or test that helped you identify it.

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________