CCNA Lab Manual

Part 2 - Changing a Static LAN to VLAN 10 and VLAN 20

Using Router-on-a-Stick to Demonstrate Router Subinterfaces

| Student: ____________________ | Date: ____________________ | Lab Time: __________ |
| --- | --- | --- |

Figure 1. Corrected router-on-a-stick topology: Cafe01-SW01 Gi0/1 to Cafe01-RT01 Gi0/1 and Cafe01-SW01 Gi0/2 to Cafe01-SW02 Gi0/2

# Lab Overview

Purpose. Begin with the previously verified static IPv4 network, then divide the switched LAN into VLAN 10 ADMIN_SERVICES and VLAN 20 PATRON_SERVICES. Use one physical link between Cafe01-SW01 and Cafe01-RT01 as an 802.1Q trunk. Configure Gi0/1.10 and Gi0/1.20 on Cafe01-RT01 so students can observe how router subinterfaces provide a separate Layer 3 gateway for each VLAN.

Starting condition: The static baseline must already work. Before changing the VLAN design, PC1 should be able to ping 10.0.18.1, 216.0.5.1, and 8.8.8.8.

## Learning Objectives

Explain why each VLAN requires its own IPv4 subnet and default gateway.

Create and name VLAN 10 ADMIN_SERVICES and VLAN 20 PATRON_SERVICES on both switches.

Assign Fa0/1-Fa0/4 to VLAN 10 and Fa0/5-Fa0/8 to VLAN 20.

Configure Cafe01-SW01 Gi0/1 as the router-facing trunk and Gi0/2 on both switches as the inter-switch trunk.

Convert Cafe01-RT01 Gi0/1 from a single Layer 3 interface into router-on-a-stick subinterfaces.

Verify VLAN membership, trunk operation, inter-VLAN routing, and Internet reachability.

## VLAN and Addressing Plan

| VLAN | Name | IPv4 Network | Gateway | Access Ports |
| --- | --- | --- | --- | --- |
| 10 | ADMIN_SERVICES | 10.0.18.0/26 | 10.0.18.1 | Fa0/1-Fa0/4 |
| 20 | PATRON_SERVICES | 10.0.18.64/26 | 10.0.18.65 | Fa0/5-Fa0/8 |

Subnet reminder: For VLAN 10, 10.0.18.63 is the broadcast address. For VLAN 20, 10.0.18.64 is the network address and 10.0.18.65 is the first usable host address.

## Post-Conversion Static Addressing

| Device | Switch / Port | VLAN | IPv4 Address | Mask | Gateway |
| --- | --- | --- | --- | --- | --- |
| Cafe01-RT01 | Gi0/1.10 | 10 | 10.0.18.1 | 255.255.255.192 | - |
| Cafe01-RT01 | Gi0/1.20 | 20 | 10.0.18.65 | 255.255.255.192 | - |
| Server | SW02 Fa0/1 | 10 | 10.0.18.6 | 255.255.255.192 | 10.0.18.1 |
| PC1 | SW01 Fa0/2 | 10 | 10.0.18.21 | 255.255.255.192 | 10.0.18.1 |
| PC2 | SW02 Fa0/2 | 10 | 10.0.18.22 | 255.255.255.192 | 10.0.18.1 |
| WAP1 | SW01 Fa0/5 | 20 | 10.0.18.66 | 255.255.255.192 | 10.0.18.65 |
| WAP2 | SW02 Fa0/5 | 20 | 10.0.18.67 | 255.255.255.192 | 10.0.18.65 |
| Laptop1 | Wireless via WAP1 | 20 | 10.0.18.84 | 255.255.255.192 | 10.0.18.65 |
| Laptop2 | Wireless via WAP2 | 20 | 10.0.18.85 | 255.255.255.192 | 10.0.18.65 |
| Cafe01-RT01 | Gi0/0 | WAN | 216.0.5.2 | 255.255.255.252 | - |
| Internet | Gi0/0 | WAN | 216.0.5.1 | 255.255.255.252 | - |

## Router-on-a-Stick Concept

Core idea: A single physical router interface carries tagged frames for multiple VLANs. The physical interface remains enabled but has no IP address. Each subinterface is bound to one 802.1Q VLAN and receives that VLAN's gateway address.

| Router Interface | 802.1Q Association | IPv4 Address | Role |
| --- | --- | --- | --- |
| Gi0/1 | Physical trunk; no VLAN number | No IP address | Carries tagged frames between the switch and router |
| Gi0/1.10 | encapsulation dot1Q 10 | 10.0.18.1/26 | Default gateway for VLAN 10 ADMIN_SERVICES |
| Gi0/1.20 | encapsulation dot1Q 20 | 10.0.18.65/26 | Default gateway for VLAN 20 PATRON_SERVICES |

### Packet Flow Through the Router-on-a-Stick

1.  A host sends traffic for another subnet to its VLAN default gateway.

2.  The access switch accepts the untagged frame on an access port and associates it with VLAN 10 or VLAN 20.

3.  The switch adds an 802.1Q tag when forwarding the frame over the trunk to Cafe01-RT01.

4.  Cafe01-RT01 selects Gi0/1.10 or Gi0/1.20 according to the VLAN tag, routes the packet, and sends the reply back over the same physical link.

5.  The switch removes the VLAN tag before transmitting the frame through the destination access port.

## Physical Changes from the Static Baseline

Keep PC1 on Cafe01-SW01 Fa0/2.

Keep Server on Cafe01-SW02 Fa0/1 and PC2 on Cafe01-SW02 Fa0/2.

Move WAP1 from Cafe01-SW01 Fa0/3 to Fa0/5 so it enters VLAN 20.

Move WAP2 from Cafe01-SW02 Fa0/3 to Fa0/5 so it enters VLAN 20.

Connect Cafe01-SW01 Gi0/1 to Cafe01-RT01 Gi0/1 and configure the switch interface as an 802.1Q trunk.

Connect Cafe01-SW01 Gi0/2 to Cafe01-SW02 Gi0/2 and configure both ends as 802.1Q trunks.

Cafe01-SW02 Gi0/1 is not used in this topology and should remain at its default configuration unless assigned to another link.

# Part 1 - Verify the Static Baseline

## Step 1: Confirm PC1 Addressing

C:\> ipconfig C:\> ping 10.0.18.1 C:\> ping 216.0.5.1 C:\> ping 8.8.8.8

Expected: All pings succeed before any VLAN changes are made.

## Step 2: Record the Existing Switch and Router State

! On both switches show vlan brief show interfaces status ! On Cafe01-RT01 show ip interface brief show ip route

Do not skip this checkpoint: A known-good static baseline separates existing routing or cabling problems from errors introduced during the VLAN conversion.

# Part 2 - Create VLANs and Assign Access Ports

## Step 1: Configure Cafe01-SW01

Cafe01-SW01> enable Cafe01-SW01# configure terminal Cafe01-SW01(config)# vlan 10 Cafe01-SW01(config-vlan)# name ADMIN_SERVICES Cafe01-SW01(config-vlan)# exit Cafe01-SW01(config)# vlan 20 Cafe01-SW01(config-vlan)# name PATRON_SERVICES Cafe01-SW01(config-vlan)# exit Cafe01-SW01(config)# interface range FastEthernet0/1 - 4 Cafe01-SW01(config-if-range)# switchport mode access Cafe01-SW01(config-if-range)# switchport access vlan 10 Cafe01-SW01(config-if-range)# spanning-tree portfast Cafe01-SW01(config-if-range)# exit Cafe01-SW01(config)# interface range FastEthernet0/5 - 8 Cafe01-SW01(config-if-range)# switchport mode access Cafe01-SW01(config-if-range)# switchport access vlan 20 Cafe01-SW01(config-if-range)# spanning-tree portfast Cafe01-SW01(config-if-range)# end Cafe01-SW01# copy running-config startup-config

## Step 2: Configure Cafe01-SW02

Cafe01-SW02> enable Cafe01-SW02# configure terminal Cafe01-SW02(config)# vlan 10 Cafe01-SW02(config-vlan)# name ADMIN_SERVICES Cafe01-SW02(config-vlan)# exit Cafe01-SW02(config)# vlan 20 Cafe01-SW02(config-vlan)# name PATRON_SERVICES Cafe01-SW02(config-vlan)# exit Cafe01-SW02(config)# interface range FastEthernet0/1 - 4 Cafe01-SW02(config-if-range)# switchport mode access Cafe01-SW02(config-if-range)# switchport access vlan 10 Cafe01-SW02(config-if-range)# spanning-tree portfast Cafe01-SW02(config-if-range)# exit Cafe01-SW02(config)# interface range FastEthernet0/5 - 8 Cafe01-SW02(config-if-range)# switchport mode access Cafe01-SW02(config-if-range)# switchport access vlan 20 Cafe01-SW02(config-if-range)# spanning-tree portfast Cafe01-SW02(config-if-range)# end Cafe01-SW02# copy running-config startup-config

## Step 3: Verify VLAN Membership

Cafe01-SW01# show vlan brief Cafe01-SW02# show vlan brief

Expected: Fa0/1-Fa0/4 appear under VLAN 10, and Fa0/5-Fa0/8 appear under VLAN 20 on both switches.

# Part 3 - Configure the 802.1Q Trunks

## Step 1: Configure Trunks on Cafe01-SW01

Cafe01-SW01# configure terminal Cafe01-SW01(config)# interface GigabitEthernet0/1 Cafe01-SW01(config-if)# description TRUNK_TO_CAFE01-RT01 Cafe01-SW01(config-if)# switchport mode trunk Cafe01-SW01(config-if)# switchport trunk allowed vlan 10,20 Cafe01-SW01(config-if)# no shutdown Cafe01-SW01(config-if)# exit Cafe01-SW01(config)# interface GigabitEthernet0/2 Cafe01-SW01(config-if)# description TRUNK_TO_CAFE01-SW02 Cafe01-SW01(config-if)# switchport mode trunk Cafe01-SW01(config-if)# switchport trunk allowed vlan 10,20 Cafe01-SW01(config-if)# no shutdown Cafe01-SW01(config-if)# end Cafe01-SW01# copy running-config startup-config

## Step 2: Configure the Inter-Switch Trunk on Cafe01-SW02

Cafe01-SW02# configure terminal Cafe01-SW02(config)# interface GigabitEthernet0/2 Cafe01-SW02(config-if)# description TRUNK_TO_CAFE01-SW01 Cafe01-SW02(config-if)# switchport mode trunk Cafe01-SW02(config-if)# switchport trunk allowed vlan 10,20 Cafe01-SW02(config-if)# no shutdown Cafe01-SW02(config-if)# end Cafe01-SW02# copy running-config startup-config

Catalyst 2960 note: Do not enter switchport trunk encapsulation dot1q on a Catalyst 2960. The platform uses IEEE 802.1Q. In this topology, Cafe01-SW01 Gi0/1 is the trunk to Cafe01-RT01, and Gi0/2 on both switches forms the inter-switch trunk. Cafe01-SW02 Gi0/1 is unused.

## Step 3: Verify Trunk Operation

Cafe01-SW01# show interfaces trunk Cafe01-SW01# show cdp neighbors Cafe01-SW02# show interfaces trunk Cafe01-SW02# show cdp neighbors

Expected: Cafe01-SW01 Gi0/1 and Gi0/2 are trunking. Cafe01-SW02 Gi0/2 is trunking. VLANs 10 and 20 appear in each active trunk's allowed and active VLAN lists.

# Part 4 - Convert Cafe01-RT01 to Router-on-a-Stick

## Step 1: Remove the Old Physical LAN Address

Cafe01-RT01> enable Cafe01-RT01# configure terminal Cafe01-RT01(config)# interface GigabitEthernet0/1 Cafe01-RT01(config-if)# no ip address Cafe01-RT01(config-if)# description TRUNK_TO_CAFE01-SW01 Cafe01-RT01(config-if)# no shutdown Cafe01-RT01(config-if)# exit

## Step 2: Configure the VLAN Subinterfaces

Cafe01-RT01(config)# interface GigabitEthernet0/1.10 Cafe01-RT01(config-subif)# description VLAN10_ADMIN_SERVICES Cafe01-RT01(config-subif)# encapsulation dot1Q 10 Cafe01-RT01(config-subif)# ip address 10.0.18.1 255.255.255.192 Cafe01-RT01(config-subif)# exit Cafe01-RT01(config)# interface GigabitEthernet0/1.20 Cafe01-RT01(config-subif)# description VLAN20_PATRON_SERVICES Cafe01-RT01(config-subif)# encapsulation dot1Q 20 Cafe01-RT01(config-subif)# ip address 10.0.18.65 255.255.255.192 Cafe01-RT01(config-subif)# exit Cafe01-RT01(config)# ip route 0.0.0.0 0.0.0.0 216.0.5.1 Cafe01-RT01(config)# end Cafe01-RT01# copy running-config startup-config

## Step 3: Verify the Router

Cafe01-RT01# show ip interface brief Cafe01-RT01# show ip route

Expected interfaces: Gi0/1 is unassigned/up/up, Gi0/1.10 is 10.0.18.1/up/up, and Gi0/1.20 is 10.0.18.65/up/up. The routing table should also show 10.0.18.0/26 directly connected through Gi0/1.10 and 10.0.18.64/26 directly connected through Gi0/1.20.

Subinterface verification: Run show interfaces GigabitEthernet0/1.10 and show interfaces GigabitEthernet0/1.20. Each subinterface should report the expected IP address and 802.1Q VLAN association. If the parent Gi0/1 interface is down, both subinterfaces will be down.

# Part 5 - Update the Internet Router Return Routes

Why this is required. The Internet router must know how to return traffic to both VLAN subnets. A route to VLAN 10 alone will not return replies to VLAN 20.

Internet> enable Internet# configure terminal Internet(config)# ip route 10.0.18.0 255.255.255.192 216.0.5.2 Internet(config)# ip route 10.0.18.64 255.255.255.192 216.0.5.2 Internet(config)# end Internet# copy running-config startup-config

## Verify the Return Routes

Internet# show ip route Internet# ping 10.0.18.1 Internet# ping 10.0.18.65

Expected routes: 10.0.18.0/26 and 10.0.18.64/26 both use next hop 216.0.5.2.

# Part 6 - Reconnect Patron Devices and Change Static Addresses

## Step 1: Move the Access Points to VLAN 20 Ports

1.  Disconnect WAP1 from Cafe01-SW01 Fa0/3 and reconnect it to Cafe01-SW01 Fa0/5.

2.  Disconnect WAP2 from Cafe01-SW02 Fa0/3 and reconnect it to Cafe01-SW02 Fa0/5.

3.  Wait for the switch links to transition to the forwarding state.

## Step 2: Change the WAP and Laptop Static Addresses

| Device | New IPv4 Address | Mask | Default Gateway |
| --- | --- | --- | --- |
| WAP1 | 10.0.18.66 | 255.255.255.192 | 10.0.18.65 |
| WAP2 | 10.0.18.67 | 255.255.255.192 | 10.0.18.65 |
| Laptop1 | 10.0.18.84 | 255.255.255.192 | 10.0.18.65 |
| Laptop2 | 10.0.18.85 | 255.255.255.192 | 10.0.18.65 |

Important: A host moved to VLAN 20 cannot keep a 10.0.18.0/26 address. Its IP address and default gateway must match the 10.0.18.64/26 subnet.

## Step 3: Keep Administrative Devices in VLAN 10

| Device | IPv4 Address | Mask | Default Gateway |
| --- | --- | --- | --- |
| Server | 10.0.18.6 | 255.255.255.192 | 10.0.18.1 |
| PC1 | 10.0.18.21 | 255.255.255.192 | 10.0.18.1 |
| PC2 | 10.0.18.22 | 255.255.255.192 | 10.0.18.1 |

# Part 7 - Verification Tests

## Test 1: Verify VLAN 10

C:\> ping 10.0.18.1 C:\> ping 10.0.18.6 C:\> ping 216.0.5.1 C:\> ping 8.8.8.8

Run from PC1 or PC2. Expected: All four pings succeed.

## Test 2: Verify VLAN 20

C:\> ping 10.0.18.65 C:\> ping 10.0.18.67 C:\> ping 216.0.5.1 C:\> ping 8.8.8.8

Run from Laptop1 or Laptop2. Expected: All four pings succeed.

## Test 3: Verify Inter-VLAN Routing

! From PC1 in VLAN 10 C:\> ping 10.0.18.84 ! From Laptop1 in VLAN 20 C:\> ping 10.0.18.21

Interpretation: The pings should succeed because Cafe01-RT01 routes between the VLAN 10 and VLAN 20 subinterfaces. VLANs separate Layer 2 broadcast domains, but they do not automatically block routed communication. An ACL is needed if patron devices must be denied access to administrative devices.

## Switch and Router Verification Commands

! Both switches show vlan brief show interfaces trunk show interfaces status show mac address-table ! Cafe01-RT01 show ip interface brief show ip route ! Internet router show ip route

# Part 8 - Troubleshooting Guide

| Symptom | Likely Cause | Corrective Action |
| --- | --- | --- |
| VLAN 10 host cannot ping 10.0.18.1 | The access port is not in VLAN 10, Cafe01-SW01 Gi0/1 is not trunking, or Cafe01-RT01 Gi0/1.10 is missing. | Verify VLAN membership, the SW01 Gi0/1 router trunk, and the router subinterface. |
| VLAN 20 host cannot ping 10.0.18.65 | Host is still on Fa0/1-Fa0/4, has a VLAN 10 address, or Gi0/1.20 is missing | Move the cable to Fa0/5-Fa0/8 and use a 10.0.18.64/26 host address. |
| Hosts work on one switch but not across switches | Cafe01-SW01 Gi0/2 or Cafe01-SW02 Gi0/2 is not trunking, or VLAN 10/20 is not allowed. | Verify the Gi0/2-to-Gi0/2 cable and configure trunk mode with VLANs 10 and 20 allowed on both ends. |
| Cafe01-RT01 can ping 8.8.8.8 but VLAN 20 cannot | Internet router lacks the 10.0.18.64/26 return route | Add ip route 10.0.18.64 255.255.255.192 216.0.5.2. |
| show interfaces trunk does not list an unused Gi port | The interface is physically down | This is normal. Connect the interface or verify the active trunk only. |
| First ping loses one packet | ARP resolution or spanning-tree convergence | Repeat the ping after a few seconds. |
| VLAN 20 host has 10.0.18.21/26 | Address belongs to VLAN 10 subnet | Assign a valid VLAN 20 address such as 10.0.18.84/26 with gateway 10.0.18.65. |
| Gi0/1.10 and Gi0/1.20 show down/down or up/down | The physical Gi0/1 link is down, the switch port is not trunking, or VLANs 10 and 20 are not allowed | Verify cabling, no shutdown, show interfaces trunk, and switchport trunk allowed vlan 10,20. |

# Completion Checklist

| Completion Item | Complete |
| --- | --- |
| VLAN 10 ADMIN_SERVICES exists on both switches. | [ ] |
| VLAN 20 PATRON_SERVICES exists on both switches. | [ ] |
| Fa0/1-Fa0/4 are access ports in VLAN 10. | [ ] |
| Fa0/5-Fa0/8 are access ports in VLAN 20. | [ ] |
| Cafe01-SW01 Gi0/1 is the router trunk; Cafe01-SW01 Gi0/2 and Cafe01-SW02 Gi0/2 form the inter-switch trunk. | [ ] |
| Cafe01-RT01 Gi0/1 has no IP address and remains up/up. | [ ] |
| Cafe01-RT01 Gi0/1.10 is 10.0.18.1/26. | [ ] |
| Cafe01-RT01 Gi0/1.20 is 10.0.18.65/26. | [ ] |
| Internet has return routes to both /26 VLAN networks. | [ ] |
| A VLAN 10 host can ping 8.8.8.8. | [ ] |
| A VLAN 20 host can ping 8.8.8.8. | [ ] |
| Configurations are saved. | [ ] |

# Student Knowledge Check

1. Why must VLAN 10 and VLAN 20 use different IPv4 subnets?

________________________________________________________________________________

________________________________________________________________________________

2. What is the purpose of the command switchport mode trunk?

________________________________________________________________________________

________________________________________________________________________________

3. What does switchport trunk allowed vlan 10,20 control?

________________________________________________________________________________

________________________________________________________________________________

4. Why must the physical Gi0/1 interface have no IP address while Gi0/1.10 and Gi0/1.20 receive gateway addresses?

________________________________________________________________________________

________________________________________________________________________________

5. What is the valid host range for VLAN 20, 10.0.18.64/26?

________________________________________________________________________________

________________________________________________________________________________

6. True or False: A Layer 2 switch alone can forward traffic between VLAN 10 and VLAN 20. Explain your answer.

________________________________________________________________________________

________________________________________________________________________________

7. A PC in VLAN 20 can ping 10.0.18.65 but cannot ping 8.8.8.8. Which routing entries should you verify?

________________________________________________________________________________

________________________________________________________________________________

8. Why did WAP1 and WAP2 need to move from Fa0/3 to Fa0/5 in this lab?

________________________________________________________________________________

________________________________________________________________________________

9. Which commands verify the active trunks and the router subinterfaces?

________________________________________________________________________________

________________________________________________________________________________

10. What additional control would you configure to prevent VLAN 20 patron devices from reaching VLAN 10 administrative devices?

________________________________________________________________________________

________________________________________________________________________________

11. What does the command encapsulation dot1Q 20 tell Cafe01-RT01 to do?

________________________________________________________________________________

# Student Reflection

Describe one configuration error you encountered during the static-to-VLAN conversion. Identify the command or test that revealed the error and explain how you corrected it.

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________