CCNA Lab Manual

Part 2 - Converting the Static LAN to VLAN 10 and VLAN 20

Using 802.1Q Trunks, Layer 3 Switch SVIs, and a Separate 10.0.19.0/30 Transit Network

| Student: ____________________ | Date: ____________________ | Lab Time: __________ |
| --- | --- | --- |

Figure 1. District Shop #1 SVI topology. Cafe01-SW01 routes VLAN 10 and VLAN 20 locally. The switch-to-router connection is a separate Layer 3 transit network: Cafe01-SW01 Gi0/1 = 10.0.19.2/30 and Cafe01-RT01 Gi0/1 = 10.0.19.1/30.

# Lab Overview

Purpose. Convert the verified Part 1 single-broadcast-domain network into VLAN 10 ADMIN_SERVICES and VLAN 20 PATRON_SERVICES. Extend both VLANs between the switches with an 802.1Q trunk, route between them with SVIs on Cafe01-SW01, and create a separate 10.0.19.0/30 transit network between Cafe01-SW01 and Cafe01-RT01 for upstream routing to the simulated Internet.

Starting condition: Complete Part 1 first. PC1 must be able to ping 10.0.18.1, 216.0.5.1, 1.1.1.1, and 8.8.8.8 before the conversion begins. This provides a known-good baseline.

## Design Change from Part 1

Replace Cafe01-SW01 with a multilayer switch such as a Catalyst 3560. Cafe01-SW02 may remain a Catalyst 2960 because it performs only Layer 2 access and trunking in this lab.

Move the Cafe01-RT01 uplink from SW01 Fa0/1 to SW01 Gi0/1. Convert Gi0/1 into a Layer 3 routed port with no switchport and place it in the dedicated 10.0.19.0/30 transit network.

Use SVIs Vlan10 and Vlan20 on Cafe01-SW01 as the host default gateways.

Keep SW01 Gi0/2 to SW02 Gi0/2 as the only 802.1Q trunk in this topology.

Move WAP1 and WAP2 from Fa0/3 to Fa0/5 so the patron wireless devices enter VLAN 20.

## Learning Objectives

Create and name VLAN 10 ADMIN_SERVICES and VLAN 20 PATRON_SERVICES.

Assign Fa0/1-Fa0/4 to VLAN 10 and Fa0/5-Fa0/8 to VLAN 20 on both switches.

Configure an 802.1Q trunk that carries VLANs 10 and 20 between the switches.

Configure Vlan10 and Vlan20 SVIs with the default-gateway addresses for both VLANs.

Enable Layer 3 switching with ip routing and verify hardware-based inter-VLAN routing.

Configure the dedicated 10.0.19.0/30 routed transit link, a default route on Cafe01-SW01, and static return routes on Cafe01-RT01.

Verify VLAN membership, SVI status, inter-VLAN routing, and end-to-end reachability.

Not router-on-a-stick: Cafe01-RT01 does not use 802.1Q subinterfaces. The inter-VLAN routing function is performed entirely by Cafe01-SW01 through its SVIs.

CRITICAL DESIGN RULE - DO NOT REUSE 10.0.18.1/26 ON THE ROUTER UPLINK 10.0.18.1/26 is the VLAN 10 SVI address and the default gateway for VLAN 10 hosts. It belongs only on interface Vlan10 of Cafe01-SW01. The routed link to Cafe01-RT01 is a different Layer 3 segment, so it uses 10.0.19.0/30: RT01 Gi0/1 = 10.0.19.1/30 and SW01 Gi0/1 = 10.0.19.2/30. Reusing 10.0.18.1 on RT01 would create a duplicate IP address and mix the user VLAN with the infrastructure transit link.

# Addressing and VLAN Plan

| Device | Interface / Port | VLAN | IPv4 Address | Mask | Gateway / Next Hop |
| --- | --- | --- | --- | --- | --- |
| Cafe01-SW01 | Vlan10 | 10 | 10.0.18.1 | 255.255.255.192 | - |
| Cafe01-SW01 | Vlan20 | 20 | 10.0.18.65 | 255.255.255.192 | - |
| Cafe01-SW01 | Gi0/1 routed | Transit | 10.0.19.2 | 255.255.255.252 | 10.0.19.1 default route |
| Cafe01-RT01 | Gi0/1 | Transit | 10.0.19.1 | 255.255.255.252 | Routes to both VLANs via 10.0.19.2 |
| Cafe01-RT01 | Gi0/0 | WAN | 216.0.5.2 | 255.255.255.252 | 216.0.5.1 default route |
| Internet | Gi0/0 | WAN | 216.0.5.1 | 255.255.255.252 | Return routes via 216.0.5.2 |
| Server | SW02 Fa0/1 | 10 | 10.0.18.6 | 255.255.255.192 | 10.0.18.1 |
| PC1 | SW01 Fa0/2 | 10 | 10.0.18.21 | 255.255.255.192 | 10.0.18.1 |
| PC2 | SW02 Fa0/2 | 10 | 10.0.18.22 | 255.255.255.192 | 10.0.18.1 |
| WAP1 | SW01 Fa0/5 | 20 | 10.0.18.66 | 255.255.255.192 | 10.0.18.65 |
| WAP2 | SW02 Fa0/5 | 20 | 10.0.18.67 | 255.255.255.192 | 10.0.18.65 |
| Laptop1 | Wireless via WAP1 | 20 | 10.0.18.84 | 255.255.255.192 | 10.0.18.65 |
| Laptop2 | Wireless via WAP2 | 20 | 10.0.18.85 | 255.255.255.192 | 10.0.18.65 |

Subnet reminder: VLAN 10 is 10.0.18.0/26 with usable hosts 10.0.18.1-10.0.18.62. VLAN 20 is 10.0.18.64/26 with usable hosts 10.0.18.65-10.0.18.126. The routed transit network is 10.0.19.0/30.

## Post-Conversion Physical Port Map

| Local Device | Local Port | Remote Device | Remote Port | Role |
| --- | --- | --- | --- | --- |
| Cafe01-SW01 | Gi0/1 | Cafe01-RT01 | Gi0/1 | Layer 3 routed uplink |
| Cafe01-SW01 | Gi0/2 | Cafe01-SW02 | Gi0/2 | 802.1Q trunk, VLANs 10 and 20 |
| Cafe01-SW01 | Fa0/2 | PC1 | Fa0 | VLAN 10 access |
| Cafe01-SW01 | Fa0/5 | WAP1 | Fa0 | VLAN 20 access |
| Cafe01-SW02 | Fa0/1 | Server | Fa0 | VLAN 10 access |
| Cafe01-SW02 | Fa0/2 | PC2 | Fa0 | VLAN 10 access |
| Cafe01-SW02 | Fa0/5 | WAP2 | Fa0 | VLAN 20 access |
| Cafe01-RT01 | Gi0/0 | Internet | Gi0/0 | 216.0.5.0/30 WAN |

# Critical Design Rule - Use a Separate Transit Network

The VLAN subnets and the switch-to-router link have different purposes and must remain separate. VLAN 10 and VLAN 20 carry endpoint traffic, while 10.0.19.0/30 is an infrastructure-only point-to-point routed link.

| Item | Correct Assignment | Reason |
| --- | --- | --- |
| VLAN 10 gateway | SW01 interface Vlan10 10.0.18.1/26 | This address is the default gateway for VLAN 10 hosts such as PC1. It identifies the VLAN 10 Layer 3 boundary. |
| VLAN 20 gateway | SW01 interface Vlan20 10.0.18.65/26 | This address is the default gateway for VLAN 20 hosts and keeps VLAN 20 in its own IP subnet. |
| Transit endpoint 1 | RT01 Gi0/1 10.0.19.1/30 | This is the router side of the point-to-point Layer 3 connection. |
| Transit endpoint 2 | SW01 Gi0/1 10.0.19.2/30 | This is the multilayer-switch side. The port uses no switchport and does not belong to VLAN 10 or VLAN 20. |
| Why /30? | 10.0.19.0/30 Usable: .1 and .2 | A /30 provides exactly two usable addresses for a point-to-point link and creates a clear routing boundary. |

## Why 10.0.18.1/26 must not be assigned to Cafe01-RT01 Gi0/1:

Duplicate address prevention: 10.0.18.1 already belongs to SW01 interface Vlan10. The same address on RT01 would create an IP conflict.

Subnet separation: a routed port connects two Layer 3 devices and must be in a different IP network from the user VLANs.

Clear forwarding: SW01 uses a default route to 10.0.19.1, and RT01 uses static routes back to both VLANs through 10.0.19.2.

Better troubleshooting: the transit subnet makes each routing hop visible and prevents VLAN 10 host traffic from being confused with infrastructure traffic.

Routing relationship Cafe01-SW01 knows VLAN 10 and VLAN 20 as directly connected networks. Its default route sends all other traffic to 10.0.19.1. Cafe01-RT01 knows both VLAN networks through static routes pointing to 10.0.19.2. The Internet router retains return routes through 216.0.5.2.

# SVI Inter-VLAN Routing Concept

Core idea. An SVI is a logical Layer 3 interface associated with a VLAN. Hosts in each VLAN use the corresponding SVI address as their default gateway. When ip routing is enabled, the multilayer switch routes packets between the VLANs in hardware.

| Layer 3 Interface | IPv4 Address | Function |
| --- | --- | --- |
| Vlan10 | 10.0.18.1/26 | Default gateway for ADMIN_SERVICES |
| Vlan20 | 10.0.18.65/26 | Default gateway for PATRON_SERVICES |
| Gi0/1 routed port | 10.0.19.2/30 | Layer 3 uplink to Cafe01-RT01 |

## Packet Flow Between VLANs

1. PC1 sends traffic for 10.0.18.84 to its Vlan10 default gateway, 10.0.18.1.

2. Cafe01-SW01 routes the packet through the directly connected Vlan20 SVI.

3. If the destination is on SW02, the frame crosses the Gi0/2 trunk with a VLAN 20 tag and exits the destination access port untagged.

Key performance benefit: Inter-VLAN traffic does not travel to Cafe01-RT01. It is routed locally by Cafe01-SW01, reducing latency and avoiding a single router trunk bottleneck.

# Part 1 - Verify the Static Baseline

## Step 1: Test PC1 Before Conversion

C:\> ipconfig C:\> ping 10.0.18.1 C:\> ping 216.0.5.1 C:\> ping 1.1.1.1 C:\> ping 8.8.8.8

## Step 2: Record Existing State

! Existing switches show interfaces status show vlan brief show cdp neighbors ! Cafe01-RT01 show ip interface brief show ip route ! Internet show ip route

# Part 2 - Install or Prepare the Multilayer Switch

## Step 1: Replace Cafe01-SW01 if Necessary

1. If Cafe01-SW01 is a Catalyst 2960, remove it and place a Catalyst 3560 multilayer switch in the same location.

2. Reconnect PC1 to Fa0/2, WAP1 temporarily to Fa0/3, and Cafe01-SW02 Gi0/2 to Cafe01-SW01 Gi0/2.

3. Connect Cafe01-SW01 Gi0/1 to Cafe01-RT01 Gi0/1. Do not use Fa0/1 for the router uplink.

4. Verify that the multilayer switch accepts the ip routing command. If it does not, the selected switch is Layer 2 only.

## Step 2: Configure the Hostname and VLANs on Cafe01-SW01

Switch> enable Switch# configure terminal Switch(config)# hostname Cafe01-SW01 Cafe01-SW01(config)# vlan 10 Cafe01-SW01(config-vlan)# name ADMIN_SERVICES Cafe01-SW01(config-vlan)# exit Cafe01-SW01(config)# vlan 20 Cafe01-SW01(config-vlan)# name PATRON_SERVICES Cafe01-SW01(config-vlan)# exit

## Step 3: Configure Access-Port Ranges on Cafe01-SW01

Cafe01-SW01(config)# interface range FastEthernet0/1 - 4 Cafe01-SW01(config-if-range)# switchport mode access Cafe01-SW01(config-if-range)# switchport access vlan 10 Cafe01-SW01(config-if-range)# spanning-tree portfast Cafe01-SW01(config-if-range)# exit Cafe01-SW01(config)# interface range FastEthernet0/5 - 8 Cafe01-SW01(config-if-range)# switchport mode access Cafe01-SW01(config-if-range)# switchport access vlan 20 Cafe01-SW01(config-if-range)# spanning-tree portfast Cafe01-SW01(config-if-range)# exit Cafe01-SW01(config)# interface FastEthernet0/2 Cafe01-SW01(config-if)# description PC1_VLAN10 Cafe01-SW01(config-if)# exit Cafe01-SW01(config)# interface FastEthernet0/5 Cafe01-SW01(config-if)# description WAP1_VLAN20 Cafe01-SW01(config-if)# end Cafe01-SW01# copy running-config startup-config

## Step 4: Verify SW01 VLAN Membership

Cafe01-SW01# show vlan brief Cafe01-SW01# show interfaces FastEthernet0/2 switchport Cafe01-SW01# show interfaces FastEthernet0/5 switchport

# Part 3 - Configure VLANs and Access Ports on Cafe01-SW02

Cafe01-SW02> enable Cafe01-SW02# configure terminal Cafe01-SW02(config)# vlan 10 Cafe01-SW02(config-vlan)# name ADMIN_SERVICES Cafe01-SW02(config-vlan)# exit Cafe01-SW02(config)# vlan 20 Cafe01-SW02(config-vlan)# name PATRON_SERVICES Cafe01-SW02(config-vlan)# exit Cafe01-SW02(config)# interface range FastEthernet0/1 - 4 Cafe01-SW02(config-if-range)# switchport mode access Cafe01-SW02(config-if-range)# switchport access vlan 10 Cafe01-SW02(config-if-range)# spanning-tree portfast Cafe01-SW02(config-if-range)# exit Cafe01-SW02(config)# interface range FastEthernet0/5 - 8 Cafe01-SW02(config-if-range)# switchport mode access Cafe01-SW02(config-if-range)# switchport access vlan 20 Cafe01-SW02(config-if-range)# spanning-tree portfast Cafe01-SW02(config-if-range)# exit Cafe01-SW02(config)# interface FastEthernet0/1 Cafe01-SW02(config-if)# description SERVER_VLAN10 Cafe01-SW02(config-if)# exit Cafe01-SW02(config)# interface FastEthernet0/2 Cafe01-SW02(config-if)# description PC2_VLAN10 Cafe01-SW02(config-if)# exit Cafe01-SW02(config)# interface FastEthernet0/5 Cafe01-SW02(config-if)# description WAP2_VLAN20 Cafe01-SW02(config-if)# end Cafe01-SW02# copy running-config startup-config

## Verify VLAN Membership on Both Switches

Cafe01-SW01# show vlan brief Cafe01-SW02# show vlan brief

| Check | Expected Result | Student Initials |
| --- | --- | --- |
| VLAN 10 exists on both switches | ADMIN_SERVICES |  |
| VLAN 20 exists on both switches | PATRON_SERVICES |  |
| Fa0/1-Fa0/4 on both switches | Access VLAN 10 |  |
| Fa0/5-Fa0/8 on both switches | Access VLAN 20 |  |

# Part 4 - Configure the Inter-Switch 802.1Q Trunk

## Step 1: Configure SW01 Gi0/2

Cafe01-SW01# configure terminal Cafe01-SW01(config)# interface GigabitEthernet0/2 Cafe01-SW01(config-if)# description TRUNK_TO_CAFE01-SW02_G0/2 Cafe01-SW01(config-if)# switchport trunk encapsulation dot1q Cafe01-SW01(config-if)# switchport mode trunk Cafe01-SW01(config-if)# switchport trunk allowed vlan 10,20 Cafe01-SW01(config-if)# no shutdown Cafe01-SW01(config-if)# end Cafe01-SW01# copy running-config startup-config

Platform variation: Some multilayer switches use only IEEE 802.1Q and do not accept switchport trunk encapsulation dot1q. If the command is unavailable, omit it and continue with switchport mode trunk.

## Step 2: Configure SW02 Gi0/2

Cafe01-SW02# configure terminal Cafe01-SW02(config)# interface GigabitEthernet0/2 Cafe01-SW02(config-if)# description TRUNK_TO_CAFE01-SW01_G0/2 Cafe01-SW02(config-if)# switchport mode trunk Cafe01-SW02(config-if)# switchport trunk allowed vlan 10,20 Cafe01-SW02(config-if)# no shutdown Cafe01-SW02(config-if)# end Cafe01-SW02# copy running-config startup-config

## Step 3: Verify the Trunk

Cafe01-SW01# show interfaces trunk Cafe01-SW01# show interfaces GigabitEthernet0/2 switchport Cafe01-SW02# show interfaces trunk Cafe01-SW02# show interfaces GigabitEthernet0/2 switchport Cafe01-SW01# show cdp neighbors Cafe01-SW02# show cdp neighbors

Expected: Gi0/2 is trunking on both switches. VLANs 10 and 20 appear in the allowed, active, and spanning-tree forwarding lists.

Scope of trunking: Only the switch-to-switch Gi0/2 link is a trunk. SW01 Gi0/1 to Cafe01-RT01 is a Layer 3 routed link and must not use switchport mode trunk.

# Part 5 - Configure SVIs and Enable Layer 3 Switching

## Step 1: Configure the VLAN 10 and VLAN 20 SVIs

Cafe01-SW01# configure terminal Cafe01-SW01(config)# interface Vlan10 Cafe01-SW01(config-if)# description GATEWAY_ADMIN_SERVICES Cafe01-SW01(config-if)# ip address 10.0.18.1 255.255.255.192 Cafe01-SW01(config-if)# no shutdown Cafe01-SW01(config-if)# exit Cafe01-SW01(config)# interface Vlan20 Cafe01-SW01(config-if)# description GATEWAY_PATRON_SERVICES Cafe01-SW01(config-if)# ip address 10.0.18.65 255.255.255.192 Cafe01-SW01(config-if)# no shutdown Cafe01-SW01(config-if)# exit Cafe01-SW01(config)# ip routing Cafe01-SW01(config)# end Cafe01-SW01# copy running-config startup-config

## Step 2: Verify SVI Status and Connected Routes

Cafe01-SW01# show ip interface brief Cafe01-SW01# show interfaces Vlan10 Cafe01-SW01# show interfaces Vlan20 Cafe01-SW01# show ip route Cafe01-SW01# show running-config ¦ section interface Vlan

Expected: Vlan10 is 10.0.18.1/up/up and Vlan20 is 10.0.18.65/up/up. The routing table contains connected routes for 10.0.18.0/26 through Vlan10 and 10.0.18.64/26 through Vlan20.

SVI line-protocol rule: An SVI may remain down/down until the VLAN exists and at least one access port or trunk carrying that VLAN is operational. Verify show vlan brief, show interfaces trunk, and connected endpoint ports.

## Step 3: Preliminary Inter-VLAN Test

! From Cafe01-SW01 Cafe01-SW01# ping 10.0.18.1 Cafe01-SW01# ping 10.0.18.65 ! After endpoints are moved and addressed PC1> ping 10.0.18.65 Laptop1> ping 10.0.18.1

# Part 6 - Create the Separate 10.0.19.0/30 Transit Network

Before entering the commands Verify that SW01 Gi0/1 is physically connected to Cafe01-RT01 Gi0/1. This link is not a trunk and is not an access port. The no switchport command changes it into a Layer 3 routed interface.

## Step 1: Convert SW01 Gi0/1 to a Routed Port and Assign 10.0.19.2/30

Cafe01-SW01# configure terminal Cafe01-SW01(config)# interface GigabitEthernet0/1 Cafe01-SW01(config-if)# description ROUTED_LINK_TO_CAFE01-RT01_G0/1 Cafe01-SW01(config-if)# no switchport Cafe01-SW01(config-if)# ip address 10.0.19.2 255.255.255.252 Cafe01-SW01(config-if)# no shutdown Cafe01-SW01(config-if)# exit Cafe01-SW01(config)# ip route 0.0.0.0 0.0.0.0 10.0.19.1 Cafe01-SW01(config)# end Cafe01-SW01# copy running-config startup-config

Required change from Part 1 Part 1 placed 10.0.18.1/26 on Cafe01-RT01 Gi0/1 because the router was the LAN gateway. In Part 2, SW01 interface Vlan10 becomes the gateway at 10.0.18.1/26. Therefore, remove that address from RT01 before assigning 10.0.19.1/30.

## Step 2: Remove the Old 10.0.18.1/26 Address from RT01 and Assign 10.0.19.1/30

Cafe01-RT01> enable Cafe01-RT01# configure terminal Cafe01-RT01(config)# interface GigabitEthernet0/1 Cafe01-RT01(config-if)# description ROUTED_LINK_TO_CAFE01-SW01_G0/1 Cafe01-RT01(config-if)# no ip address Cafe01-RT01(config-if)# ip address 10.0.19.1 255.255.255.252 Cafe01-RT01(config-if)# no shutdown Cafe01-RT01(config-if)# exit Cafe01-RT01(config)# ip route 10.0.18.0 255.255.255.192 10.0.19.2 Cafe01-RT01(config)# ip route 10.0.18.64 255.255.255.192 10.0.19.2 Cafe01-RT01(config)# ip route 0.0.0.0 0.0.0.0 216.0.5.1 Cafe01-RT01(config)# end Cafe01-RT01# copy running-config startup-config

## Step 3: Verify the Transit Link, Default Route, and Return Routes

Cafe01-SW01# show interfaces GigabitEthernet0/1 switchport Cafe01-SW01# show ip interface brief Cafe01-SW01# show ip route Cafe01-SW01# ping 10.0.19.1 Cafe01-RT01# show ip interface brief Cafe01-RT01# show ip route Cafe01-RT01# ping 10.0.19.2 Cafe01-RT01# ping 10.0.18.1 Cafe01-RT01# ping 10.0.18.65

Expected routed-port result: show interfaces Gi0/1 switchport should report Switchport: Disabled. The multilayer switch should have a default route via 10.0.19.1, and RT01 should have two static routes via 10.0.19.2.

## Expected Routing Logic After the Transit Network Is Configured

| Device | Required Route | Meaning |
| --- | --- | --- |
| Cafe01-SW01 | C 10.0.18.0/26 via Vlan10 C 10.0.18.64/26 via Vlan20 | Both user VLANs are directly connected to the multilayer switch. |
| Cafe01-SW01 | C 10.0.19.0/30 via Gi0/1 S* 0.0.0.0/0 via 10.0.19.1 | The transit network is directly connected, and all unknown destinations go to RT01. |
| Cafe01-RT01 | S 10.0.18.0/26 via 10.0.19.2 S 10.0.18.64/26 via 10.0.19.2 | RT01 sends return traffic for both VLANs back to the multilayer switch. |
| Cafe01-RT01 | S* 0.0.0.0/0 via 216.0.5.1 | Internet-bound traffic continues toward the simulated Internet router. |

# Part 7 - Verify Internet Return Routes and Update Endpoints

## Step 1: Verify the Internet Router Return Routes

Internet> enable Internet# configure terminal Internet(config)# ip route 10.0.18.0 255.255.255.192 216.0.5.2 Internet(config)# ip route 10.0.18.64 255.255.255.192 216.0.5.2 Internet(config)# end Internet# copy running-config startup-config Internet# show ip route

No NAT required: The Internet router owns the simulated destinations 1.1.1.1 and 8.8.8.8 and has explicit routes back to both VLAN networks. NAT and PAT are outside the scope of this lab.

## Step 2: Move WAP1 and WAP2 to VLAN 20 Ports

1. Disconnect WAP1 from SW01 Fa0/3 and reconnect it to SW01 Fa0/5.

2. Disconnect WAP2 from SW02 Fa0/3 and reconnect it to SW02 Fa0/5.

3. Wait for the access links and the Vlan20 SVI to reach the up/up state.

## Step 3: Configure Endpoint Addresses

| Device | IPv4 Address | Mask | Default Gateway | Connection |
| --- | --- | --- | --- | --- |
| Server | 10.0.18.6 | 255.255.255.192 | 10.0.18.1 | SW02 Fa0/1 VLAN 10 |
| PC1 | 10.0.18.21 | 255.255.255.192 | 10.0.18.1 | SW01 Fa0/2 VLAN 10 |
| PC2 | 10.0.18.22 | 255.255.255.192 | 10.0.18.1 | SW02 Fa0/2 VLAN 10 |
| WAP1 | 10.0.18.66 | 255.255.255.192 | 10.0.18.65 | SW01 Fa0/5 VLAN 20 |
| WAP2 | 10.0.18.67 | 255.255.255.192 | 10.0.18.65 | SW02 Fa0/5 VLAN 20 |
| Laptop1 | 10.0.18.84 | 255.255.255.192 | 10.0.18.65 | Wireless via WAP1 |
| Laptop2 | 10.0.18.85 | 255.255.255.192 | 10.0.18.65 | Wireless via WAP2 |

Addressing rule: A host connected to VLAN 20 must use an address from 10.0.18.64/26 and default gateway 10.0.18.65. A VLAN 10 address will not work on a VLAN 20 access port.

# Part 8 - Verification Tests

## Test 1: Verify VLAN 10

! From PC1 C:\> ipconfig C:\> ping 10.0.18.1 C:\> ping 10.0.18.6 C:\> ping 10.0.18.22

## Test 2: Verify VLAN 20

! From Laptop1 C:\> ipconfig C:\> ping 10.0.18.65 C:\> ping 10.0.18.67 C:\> ping 10.0.18.85

## Test 3: Verify SVI Inter-VLAN Routing

! From PC1 in VLAN 10 C:\> ping 10.0.18.84 C:\> tracert 10.0.18.84 ! From Laptop1 in VLAN 20 C:\> ping 10.0.18.21

Expected: Inter-VLAN pings succeed. The first hop from PC1 is 10.0.18.1, and the packet is routed by Cafe01-SW01 rather than Cafe01-RT01.

## Test 4: Verify WAN and Simulated Internet Reachability

! From PC1 and Laptop1 C:\> ping 10.0.19.1 C:\> ping 216.0.5.1 C:\> ping 1.1.1.1 C:\> ping 8.8.8.8 C:\> tracert 8.8.8.8

Expected traceroute path: the host SVI gateway, Cafe01-RT01 at 10.0.19.1, the Internet router at 216.0.5.1, and the simulated destination.

## Infrastructure Verification Commands

Use show vlan brief and show interfaces trunk on both switches. On Cafe01-SW01, also use show ip interface brief, show ip route, and show interfaces GigabitEthernet0/1 switchport. Use show ip route on both routers.

# Part 9 - Troubleshooting Guide

| Symptom | Likely Cause | Corrective Action |
| --- | --- | --- |
| ip routing command is rejected | Cafe01-SW01 is a Layer 2 switch such as a Catalyst 2960 | Replace it with a multilayer switch such as a Catalyst 3560. |
| Vlan10 or Vlan20 is down/down | VLAN missing or no active access/trunk port carries the VLAN | Verify show vlan brief, connected access ports, and the Gi0/2 trunk. |
| Hosts in the same VLAN work, but inter-VLAN ping fails | ip routing is missing or host default gateway is incorrect | Enable ip routing and verify 10.0.18.1 or 10.0.18.65 on the host. |
| Hosts work on SW01 but not SW02 | Gi0/2 trunk is down or VLAN 10/20 is not allowed | Verify show interfaces trunk and allowed VLANs on both switches. |
| SW01 cannot ping 10.0.19.1 | Gi0/1 is still a switchport, wrong /30 address, or cable is down | Apply no switchport, assign 10.0.19.2/30, and verify RT01 10.0.19.1/30. |
| RT01 cannot ping an SVI | Static routes to the VLAN networks are missing | Add both routes through 10.0.19.2. |
| VLAN hosts reach RT01 but not 8.8.8.8 | SW01 default route, RT01 default route, or Internet return route is missing | Verify the complete routing chain in all three Layer 3 devices. |
| VLAN 20 host cannot reach 10.0.18.65 | WAP is still on Fa0/3 or host retains a VLAN 10 address | Move WAP to Fa0/5 and assign a valid VLAN 20 address and gateway. |
| show interfaces trunk does not list Gi0/1 | Gi0/1 is correctly configured as a routed port | This is expected. Gi0/1 must not appear in the trunk table. |
| First ping loses one packet | ARP resolution or spanning-tree convergence | Wait briefly and repeat the ping. |
| PC1 can ping 10.0.18.1 and 10.0.18.65, but ping 8.8.8.8 returns “Reply from 10.0.18.1: Destination host unreachable.” | SW01 has no default route, Gi0/1 is still a switchport, or the 10.0.19.0/30 transit link is missing. | Configure SW01 Gi0/1 as 10.0.19.2/30 with no switchport, configure RT01 Gi0/1 as 10.0.19.1/30, and add the required routes. |

## Resolved Example - PC1 Can Reach Both SVIs but Cannot Reach 8.8.8.8

C:\> ping 8.8.8.8 Reply from 10.0.18.1: Destination host unreachable.

Interpretation: PC1 reached its default gateway at 10.0.18.1, so the PC address, VLAN 10 access port, SVI, and inter-VLAN routing were working. The gateway generated the unreachable message because Cafe01-SW01 had no default route and Gi0/1 was still a Layer 2 switchport. The correction was to configure SW01 Gi0/1 as 10.0.19.2/30 with no switchport, configure RT01 Gi0/1 as 10.0.19.1/30, add the SW01 default route, and add RT01 return routes to both VLANs.

## Useful Diagnostic Commands

! Multilayer switch show ip interface brief show ip route show vlan brief show interfaces trunk show interfaces <port> switchport show running-config ¦ section interface Vlan ! Layer 2 switch show vlan brief show interfaces trunk show mac address-table ! Routers and hosts show ip route ping <destination> traceroute <destination> ipconfig tracert <destination>

# Lab Completion Checklist

| Completion Item | Complete |
| --- | --- |
| Cafe01-SW01 is a multilayer switch and accepts ip routing. | [ ] |
| VLAN 10 ADMIN_SERVICES and VLAN 20 PATRON_SERVICES exist on both switches. | [ ] |
| Fa0/1-Fa0/4 are VLAN 10 access ports on both switches. | [ ] |
| Fa0/5-Fa0/8 are VLAN 20 access ports on both switches. | [ ] |
| SW01 Gi0/2 and SW02 Gi0/2 form an 802.1Q trunk carrying VLANs 10 and 20. | [ ] |
| SW01 Vlan10 is 10.0.18.1/26 and up/up. | [ ] |
| SW01 Vlan20 is 10.0.18.65/26 and up/up. | [ ] |
| ip routing is enabled on Cafe01-SW01. | [ ] |
| SW01 Gi0/1 is a routed port with 10.0.19.2/30. | [ ] |
| RT01 Gi0/1 is 10.0.19.1/30. | [ ] |
| SW01 has a default route via 10.0.19.1. | [ ] |
| RT01 has routes to both VLAN networks via 10.0.19.2. | [ ] |
| Internet has return routes to both VLAN networks via 216.0.5.2. | [ ] |
| VLAN 10 and VLAN 20 hosts can ping each other. | [ ] |
| Both VLANs can ping 8.8.8.8. | [ ] |
| All configurations are saved. | [ ] |
| 10.0.18.1/26 appears only on SW01 interface Vlan10 and is not configured on RT01 Gi0/1. | [ ] |
| The 10.0.19.0/30 transit network is used only between SW01 Gi0/1 and RT01 Gi0/1. | [ ] |

# Student Knowledge Check

1. What is an SVI, and why does each VLAN require one for inter-VLAN routing?

_________________________________________________________________________________________________________

_________________________________________________________________________________________________________

2. What command enables Layer 3 routing on a multilayer switch?

_________________________________________________________________________________________________________

_________________________________________________________________________________________________________

3. Why is SW01 Gi0/1 configured with no switchport, and why must it use 10.0.19.2/30 instead of 10.0.18.1/26?

_________________________________________________________________________________________________________

_________________________________________________________________________________________________________

4. What is the difference between the Gi0/2 trunk and the Gi0/1 routed link?

_________________________________________________________________________________________________________

_________________________________________________________________________________________________________

5. Why must VLAN 10 and VLAN 20 use different IPv4 subnets?

_________________________________________________________________________________________________________

_________________________________________________________________________________________________________

6. What conditions must be met for an SVI to reach the up/up state?

_________________________________________________________________________________________________________

_________________________________________________________________________________________________________

# Student Knowledge Check - Continued

7. Why does Cafe01-SW01 use an ip route 0.0.0.0 0.0.0.0 10.0.19.1 command instead of ip default-gateway?

_________________________________________________________________________________________________________

_________________________________________________________________________________________________________

8. Which routes must Cafe01-RT01 have to return traffic to both VLANs?

_________________________________________________________________________________________________________

_________________________________________________________________________________________________________

9. Why does inter-VLAN traffic not need to pass through Cafe01-RT01 in this design?

_________________________________________________________________________________________________________

_________________________________________________________________________________________________________

10. Which commands verify the SVIs, the inter-switch trunk, and the routed uplink?

_________________________________________________________________________________________________________

_________________________________________________________________________________________________________

11. Why is NAT not required in this Packet Tracer topology?

_________________________________________________________________________________________________________

_________________________________________________________________________________________________________

12. What control could prevent VLAN 20 patron devices from reaching VLAN 10 administrative devices?

_________________________________________________________________________________________________________

_________________________________________________________________________________________________________

Final design summary VLAN 10 gateway = SW01 Vlan10 10.0.18.1/26. VLAN 20 gateway = SW01 Vlan20 10.0.18.65/26. Routed transit = RT01 Gi0/1 10.0.19.1/30 and SW01 Gi0/1 10.0.19.2/30. Never assign the VLAN 10 gateway address to the routed transit link.

# Student Reflection

Describe one configuration error you encountered during the SVI conversion. Identify the command or connectivity test that revealed the error and explain how you corrected it.

_________________________________________________________________________________________________________

_________________________________________________________________________________________________________

_________________________________________________________________________________________________________

_________________________________________________________________________________________________________

_________________________________________________________________________________________________________

_________________________________________________________________________________________________________

_________________________________________________________________________________________________________

_________________________________________________________________________________________________________