CCNA LAB MANUAL

Part 2 - Converting the Static LAN to VLAN 10 and VLAN 20

External Router with Multiple Physical Interfaces

One dedicated router interface per VLAN; no router-on-a-stick and no Layer 3 switch SVIs

| Student | Date | Lab Time |
| --- | --- | --- |
| ________________________ | ________________ | __________ |

Figure 1. District Shop #1 converted to two VLANs using two separate physical LAN interfaces on Cafe01-RT01. The only 802.1Q trunk is the inter-switch link between Cafe01-SW01 and Cafe01-SW02.

# Lab Overview

Purpose. Begin with the verified Part 1 static IPv4 network, divide the LAN into VLAN 10 ADMIN_SERVICES and VLAN 20 PATRON_SERVICES, extend both VLANs between the switches with an 802.1Q trunk, and use a different physical Cafe01-RT01 interface as the default gateway for each VLAN.

Traditional design This method assigns one router port to each VLAN. It is simple to understand and provides clear physical separation, but it consumes one router interface and one switch link for every VLAN. It does not scale well as the number of VLANs grows.

## Learning Objectives

Verify the Part 1 static network before making VLAN changes.

Create and name VLAN 10 ADMIN_SERVICES and VLAN 20 PATRON_SERVICES on both switches.

Assign Fa0/1-Fa0/4 to VLAN 10 and Fa0/5-Fa0/8 to VLAN 20.

Configure the SW01-to-SW02 link as an 802.1Q trunk carrying VLANs 10 and 20.

Connect Cafe01-RT01 Gi0/1 to an SW01 access port in VLAN 10 and Gi0/2 to a separate SW01 access port in VLAN 20.

Configure router interfaces 10.0.18.1/26 and 10.0.18.65/26 as the two VLAN default gateways.

Verify inter-VLAN routing and simulated Internet access without NAT.

## Equipment and Assumptions

Cafe01-RT01 must have at least three usable physical Layer 3 interfaces: one WAN interface, one VLAN 10 interface, and one VLAN 20 interface. A Cisco 2911 in Packet Tracer provides Gi0/0, Gi0/1, and Gi0/2.

Cafe01-SW01 and Cafe01-SW02 may both be Catalyst 2960 Layer 2 switches. A multilayer switch is not required for this method.

The Internet router uses Loopback0 1.1.1.1/32 and Loopback1 8.8.8.8/32 as simulated Internet destinations.

NAT is not required because the Internet router receives static return routes to both private VLAN subnets.

Starting condition Before the conversion, PC1 should successfully ping 10.0.18.1, 216.0.5.1, and 8.8.8.8 in the Part 1 flat-LAN design. Record a known-good baseline before recabling or changing IP addresses.

# VLAN and Addressing Plan

| VLAN | Name | IPv4 Network | Router Gateway Interface | Gateway | Access Ports |
| --- | --- | --- | --- | --- | --- |
| 10 | ADMIN_SERVICES | 10.0.18.0/26 | Cafe01-RT01 Gi0/1 | 10.0.18.1 | Fa0/1-Fa0/4 |
| 20 | PATRON_SERVICES | 10.0.18.64/26 | Cafe01-RT01 Gi0/2 | 10.0.18.65 | Fa0/5-Fa0/8 |

Subnet reminder VLAN 10 uses 10.0.18.0/26 with usable hosts 10.0.18.1-10.0.18.62 and broadcast 10.0.18.63. VLAN 20 uses 10.0.18.64/26 with usable hosts 10.0.18.65-10.0.18.126 and broadcast 10.0.18.127.

## Post-Conversion Static Addressing

| Device | Interface / Port | VLAN | IPv4 Address | Mask | Default Gateway |
| --- | --- | --- | --- | --- | --- |
| Cafe01-RT01 | Gi0/1 | 10 | 10.0.18.1 | 255.255.255.192 | - |
| Cafe01-RT01 | Gi0/2 | 20 | 10.0.18.65 | 255.255.255.192 | - |
| Server | SW02 Fa0/1 | 10 | 10.0.18.6 | 255.255.255.192 | 10.0.18.1 |
| PC1 | SW01 Fa0/2 | 10 | 10.0.18.21 | 255.255.255.192 | 10.0.18.1 |
| PC2 | SW02 Fa0/2 | 10 | 10.0.18.22 | 255.255.255.192 | 10.0.18.1 |
| WAP1 | SW01 Fa0/5 | 20 | 10.0.18.66 | 255.255.255.192 | 10.0.18.65 |
| WAP2 | SW02 Fa0/5 | 20 | 10.0.18.67 | 255.255.255.192 | 10.0.18.65 |
| Laptop1 | Wireless via WAP1 | 20 | 10.0.18.84 | 255.255.255.192 | 10.0.18.65 |
| Laptop2 | Wireless via WAP2 | 20 | 10.0.18.85 | 255.255.255.192 | 10.0.18.65 |
| Cafe01-RT01 | Gi0/0 | WAN | 216.0.5.2 | 255.255.255.252 | - |
| Internet | Gi0/0 | WAN | 216.0.5.1 | 255.255.255.252 | - |

# External Router with Multiple Physical Interfaces - Concept

Each VLAN is a separate Layer 2 broadcast domain and a separate IPv4 subnet. A router must have a Layer 3 presence in each VLAN to route traffic between those subnets. In this method, a separate physical router interface supplies that Layer 3 presence for each VLAN.

| Physical Link | Switch-Port Mode | Router Address | Function |
| --- | --- | --- | --- |
| SW01 Gi0/1 ↔ RT01 Gi0/1 | Access VLAN 10 | 10.0.18.1/26 | Default gateway and routed exit for VLAN 10 |
| SW01 Fa0/24 ↔ RT01 Gi0/2 | Access VLAN 20 | 10.0.18.65/26 | Default gateway and routed exit for VLAN 20 |
| SW01 Gi0/2 ↔ SW02 Gi0/2 | 802.1Q trunk | No IP address | Extends VLANs 10 and 20 between the switches |

## Why Two Router Interfaces Are Required

Gi0/1 is physically attached to VLAN 10 and must use an address from 10.0.18.0/26.

Gi0/2 is physically attached to VLAN 20 and must use an address from 10.0.18.64/26.

The router sees both subnets as directly connected and routes between them without subinterfaces or SVIs.

The switch ports facing the router are access ports, not trunks. Frames sent to the router are untagged.

The two physical links do not create a Layer 2 loop because each link belongs to a different VLAN and terminates on a router Layer 3 interface.

Do not mix methods Do not configure router subinterfaces or encapsulation dot1Q commands. Do not configure 10.0.18.1 or 10.0.18.65 on switch SVIs as host gateways. In this lab, the physical router interfaces are the gateways.

## Scalability Limitation

This method works well for a small demonstration with two VLANs. However, every additional VLAN requires another router interface, another cable, and another switch access port. A router-on-a-stick or multilayer-switch SVI design is normally preferred when many VLANs are required.

# Part 1 - Verify the Static Baseline and Prepare the Conversion

## Step 1: Verify the Existing Part 1 Network

C:\> ipconfig C:\> ping 10.0.18.1 C:\> ping 216.0.5.1 C:\> ping 8.8.8.8

Expected: All pings succeed before the VLAN conversion begins. If the baseline fails, correct it before changing VLANs, cabling, or addressing.

## Step 2: Record Existing Device State

! Both switches show vlan brief show interfaces status show cdp neighbors ! Cafe01-RT01 show ip interface brief show ip route show running-config

## Step 3: Recable for Two Physical Router Interfaces

1.  Disconnect the existing SW01-to-RT01 LAN cable if it uses SW01 Fa0/1.

2.  Connect Cafe01-SW01 Gi0/1 to Cafe01-RT01 Gi0/1. This becomes the VLAN 10 physical gateway link.

3.  Connect Cafe01-SW01 Fa0/24 to Cafe01-RT01 Gi0/2. This becomes the VLAN 20 physical gateway link.

4.  Keep Cafe01-SW01 Gi0/2 connected to Cafe01-SW02 Gi0/2 for the inter-switch trunk.

5.  Move WAP1 from SW01 Fa0/3 to SW01 Fa0/5 and WAP2 from SW02 Fa0/3 to SW02 Fa0/5.

Port reservation SW01 Gi0/1 and SW01 Fa0/24 are dedicated router-facing access ports. Do not connect endpoint devices to these ports.

# Part 2 - Configure VLANs and Access Ports on Cafe01-SW01

## Step 1: Create and Name the VLANs

Cafe01-SW01> enable Cafe01-SW01# configure terminal Cafe01-SW01(config)# vlan 10 Cafe01-SW01(config-vlan)# name ADMIN_SERVICES Cafe01-SW01(config-vlan)# exit Cafe01-SW01(config)# vlan 20 Cafe01-SW01(config-vlan)# name PATRON_SERVICES Cafe01-SW01(config-vlan)# exit

## Step 2: Assign Endpoint Access Ports

Cafe01-SW01(config)# interface range FastEthernet0/1 - 4 Cafe01-SW01(config-if-range)# switchport mode access Cafe01-SW01(config-if-range)# switchport access vlan 10 Cafe01-SW01(config-if-range)# spanning-tree portfast Cafe01-SW01(config-if-range)# exit Cafe01-SW01(config)# interface range FastEthernet0/5 - 8 Cafe01-SW01(config-if-range)# switchport mode access Cafe01-SW01(config-if-range)# switchport access vlan 20 Cafe01-SW01(config-if-range)# spanning-tree portfast Cafe01-SW01(config-if-range)# exit

## Step 3: Configure the Two Router-Facing Access Ports

Cafe01-SW01(config)# interface GigabitEthernet0/1 Cafe01-SW01(config-if)# description VLAN10_TO_CAFE01-RT01_G0/1 Cafe01-SW01(config-if)# switchport mode access Cafe01-SW01(config-if)# switchport access vlan 10 Cafe01-SW01(config-if)# no shutdown Cafe01-SW01(config-if)# exit Cafe01-SW01(config)# interface FastEthernet0/24 Cafe01-SW01(config-if)# description VLAN20_TO_CAFE01-RT01_G0/2 Cafe01-SW01(config-if)# switchport mode access Cafe01-SW01(config-if)# switchport access vlan 20 Cafe01-SW01(config-if)# no shutdown Cafe01-SW01(config-if)# end Cafe01-SW01# copy running-config startup-config

Important The router-facing ports use switchport mode access. Do not configure either link as a trunk.

# Part 3 - Configure Cafe01-SW02 and the Inter-Switch Trunk

## Step 1: Create VLANs and Assign Access Ports on SW02

Cafe01-SW02> enable Cafe01-SW02# configure terminal Cafe01-SW02(config)# vlan 10 Cafe01-SW02(config-vlan)# name ADMIN_SERVICES Cafe01-SW02(config-vlan)# exit Cafe01-SW02(config)# vlan 20 Cafe01-SW02(config-vlan)# name PATRON_SERVICES Cafe01-SW02(config-vlan)# exit Cafe01-SW02(config)# interface range FastEthernet0/1 - 4 Cafe01-SW02(config-if-range)# switchport mode access Cafe01-SW02(config-if-range)# switchport access vlan 10 Cafe01-SW02(config-if-range)# spanning-tree portfast Cafe01-SW02(config-if-range)# exit Cafe01-SW02(config)# interface range FastEthernet0/5 - 8 Cafe01-SW02(config-if-range)# switchport mode access Cafe01-SW02(config-if-range)# switchport access vlan 20 Cafe01-SW02(config-if-range)# spanning-tree portfast Cafe01-SW02(config-if-range)# exit

## Step 2: Configure Gi0/2 as a Trunk on Both Switches

! Cafe01-SW01 Cafe01-SW01(config)# interface GigabitEthernet0/2 Cafe01-SW01(config-if)# description TRUNK_TO_CAFE01-SW02_G0/2 Cafe01-SW01(config-if)# switchport mode trunk Cafe01-SW01(config-if)# switchport trunk allowed vlan 10,20 Cafe01-SW01(config-if)# no shutdown ! Cafe01-SW02 Cafe01-SW02(config)# interface GigabitEthernet0/2 Cafe01-SW02(config-if)# description TRUNK_TO_CAFE01-SW01_G0/2 Cafe01-SW02(config-if)# switchport mode trunk Cafe01-SW02(config-if)# switchport trunk allowed vlan 10,20 Cafe01-SW02(config-if)# no shutdown Cafe01-SW02(config-if)# end Cafe01-SW02# copy running-config startup-config

Catalyst 2960 note Do not enter switchport trunk encapsulation dot1q on a Catalyst 2960. It uses IEEE 802.1Q for trunking.

## Step 3: Verify VLANs and Trunking

Cafe01-SW01# show vlan brief Cafe01-SW01# show interfaces trunk Cafe01-SW02# show vlan brief Cafe01-SW02# show interfaces trunk

Expected: SW01 Gi0/2 and SW02 Gi0/2 are trunking with VLANs 10 and 20 allowed and active. SW01 Gi0/1 appears in VLAN 10, and SW01 Fa0/24 appears in VLAN 20.

# Part 4 - Configure Cafe01-RT01 with One Physical Interface per VLAN

## Step 1: Configure VLAN 10 on Gi0/1

Cafe01-RT01> enable Cafe01-RT01# configure terminal Cafe01-RT01(config)# interface GigabitEthernet0/1 Cafe01-RT01(config-if)# description VLAN10_ADMIN_SERVICES_GATEWAY Cafe01-RT01(config-if)# ip address 10.0.18.1 255.255.255.192 Cafe01-RT01(config-if)# no shutdown Cafe01-RT01(config-if)# exit

## Step 2: Configure VLAN 20 on Gi0/2

Cafe01-RT01(config)# interface GigabitEthernet0/2 Cafe01-RT01(config-if)# description VLAN20_PATRON_SERVICES_GATEWAY Cafe01-RT01(config-if)# ip address 10.0.18.65 255.255.255.192 Cafe01-RT01(config-if)# no shutdown Cafe01-RT01(config-if)# exit

## Step 3: Verify the WAN and Default Route

Cafe01-RT01(config)# interface GigabitEthernet0/0 Cafe01-RT01(config-if)# description WAN_TO_INTERNET Cafe01-RT01(config-if)# ip address 216.0.5.2 255.255.255.252 Cafe01-RT01(config-if)# no shutdown Cafe01-RT01(config-if)# exit Cafe01-RT01(config)# ip route 0.0.0.0 0.0.0.0 216.0.5.1 Cafe01-RT01(config)# end Cafe01-RT01# copy running-config startup-config

## Step 4: Verify the Router

Cafe01-RT01# show ip interface brief Cafe01-RT01# show ip route Cafe01-RT01# ping 216.0.5.1 Cafe01-RT01# ping 8.8.8.8

Expected connected routes Cafe01-RT01 should show 10.0.18.0/26 directly connected through Gi0/1 and 10.0.18.64/26 directly connected through Gi0/2. No static routes are needed between the two VLANs because both are directly connected to the router.

# Part 5 - Update the Internet Router Return Routes

The Internet router must know how to return traffic to both VLAN subnets. The forward path from Cafe01-RT01 is not sufficient by itself.

Internet> enable Internet# configure terminal Internet(config)# ip route 10.0.18.0 255.255.255.192 216.0.5.2 Internet(config)# ip route 10.0.18.64 255.255.255.192 216.0.5.2 Internet(config)# end Internet# copy running-config startup-config

## Verify the Return Routes

Internet# show ip route Internet# ping 216.0.5.2 Internet# ping 10.0.18.1 Internet# ping 10.0.18.65

| Route | Next Hop | Purpose |
| --- | --- | --- |
| 10.0.18.0/26 | 216.0.5.2 | Returns traffic to VLAN 10 through Cafe01-RT01 |
| 10.0.18.64/26 | 216.0.5.2 | Returns traffic to VLAN 20 through Cafe01-RT01 |

Why NAT is not required The Internet router directly hosts 1.1.1.1 and 8.8.8.8 as loopbacks and has explicit return routes to the private VLANs. Therefore, this Packet Tracer lab demonstrates routing without address translation.

## Routing Table Check on Cafe01-RT01

C 10.0.18.0/26 is directly connected, GigabitEthernet0/1 C 10.0.18.64/26 is directly connected, GigabitEthernet0/2 C 216.0.5.0/30 is directly connected, GigabitEthernet0/0 S* 0.0.0.0/0 via 216.0.5.1

# Part 6 - Reconnect Devices and Change Static Addresses

## Step 1: Keep Administrative Devices in VLAN 10

| Device | Switch Port | IP Address | Mask | Gateway |
| --- | --- | --- | --- | --- |
| Server | SW02 Fa0/1 | 10.0.18.6 | 255.255.255.192 | 10.0.18.1 |
| PC1 | SW01 Fa0/2 | 10.0.18.21 | 255.255.255.192 | 10.0.18.1 |
| PC2 | SW02 Fa0/2 | 10.0.18.22 | 255.255.255.192 | 10.0.18.1 |

## Step 2: Move Patron Devices to VLAN 20 Ports

1.  Move WAP1 to Cafe01-SW01 Fa0/5.

2.  Move WAP2 to Cafe01-SW02 Fa0/5.

3.  Wait for the access ports to enter the forwarding state.

4.  Update the WAP and laptop static addresses to the VLAN 20 subnet.

| Device | Switch / Connection | New IP Address | Mask | Gateway |
| --- | --- | --- | --- | --- |
| WAP1 | SW01 Fa0/5 | 10.0.18.66 | 255.255.255.192 | 10.0.18.65 |
| WAP2 | SW02 Fa0/5 | 10.0.18.67 | 255.255.255.192 | 10.0.18.65 |
| Laptop1 | Wireless via WAP1 | 10.0.18.84 | 255.255.255.192 | 10.0.18.65 |
| Laptop2 | Wireless via WAP2 | 10.0.18.85 | 255.255.255.192 | 10.0.18.65 |

Addressing rule A device connected to a VLAN 20 access port cannot keep a VLAN 10 address. The host address, subnet mask, and default gateway must all match 10.0.18.64/26.

## Step 3: Confirm Host Configuration

C:\> ipconfig VLAN 10 host example: IP address 10.0.18.21 Subnet mask 255.255.255.192 Default gateway 10.0.18.1 VLAN 20 host example: IP address 10.0.18.84 Subnet mask 255.255.255.192 Default gateway 10.0.18.65

# Part 7 - Verification Tests

## Test 1: Verify VLAN 10 from PC1

C:\> ping 10.0.18.1 C:\> ping 10.0.18.6 C:\> ping 10.0.18.22 C:\> ping 216.0.5.1 C:\> ping 8.8.8.8

Expected: All pings succeed.

## Test 2: Verify VLAN 20 from Laptop1

C:\> ping 10.0.18.65 C:\> ping 10.0.18.67 C:\> ping 216.0.5.1 C:\> ping 8.8.8.8

Expected: All pings succeed.

## Test 3: Verify Inter-VLAN Routing

! From PC1 in VLAN 10 C:\> ping 10.0.18.84 ! From Laptop1 in VLAN 20 C:\> ping 10.0.18.21

Expected: Both pings succeed because Cafe01-RT01 routes between its directly connected Gi0/1 and Gi0/2 networks.

## Test 4: Verify Trunks and Physical Router Links

! Both switches show vlan brief show interfaces trunk show interfaces status show mac address-table ! Cafe01-RT01 show ip interface brief show ip route

First-ping behavior The first ICMP echo may time out while ARP resolves a next-hop MAC address. Repeat the ping before treating one initial timeout as a fault.

# Packet Flow Through the External Router

## Example: PC1 in VLAN 10 Pings Laptop1 in VLAN 20

1.  PC1 determines that 10.0.18.84 is outside its local 10.0.18.0/26 subnet.

2.  PC1 sends the packet to its default gateway, 10.0.18.1, using the MAC address of RT01 Gi0/1.

3.  SW01 forwards the untagged VLAN 10 frame through the Gi0/1 access link to RT01 Gi0/1.

4.  RT01 checks its routing table and finds 10.0.18.64/26 directly connected through Gi0/2.

5.  RT01 sends a new Ethernet frame from Gi0/2 into VLAN 20 through SW01 Fa0/24.

6.  SW01 forwards the VLAN 20 traffic locally or over the Gi0/2 trunk to SW02 as required.

7.  The reply follows the reverse path through RT01.

## Example: PC1 Pings 8.8.8.8

| Hop | Device / Interface | Routing Decision |
| --- | --- | --- |
| 1 | Cafe01-RT01 Gi0/1 — 10.0.18.1 | Receives VLAN 10 traffic and uses the default route toward 216.0.5.1 |
| 2 | Internet Gi0/0 — 216.0.5.1 | Receives the packet and delivers it to local Loopback1 |
| 3 | Internet Loopback1 — 8.8.8.8 | Responds; static return route sends the reply through 216.0.5.2 |

C:\> tracert 8.8.8.8 Expected logical path: 1 10.0.18.1 2 216.0.5.1 3 8.8.8.8

Tagging boundary Frames are tagged only on the SW01-to-SW02 trunk. The two switch-to-router links are access links, so the router receives ordinary untagged Ethernet frames on separate physical interfaces.

# Part 8 - Troubleshooting Guide

| Symptom | Likely Cause | Corrective Action |
| --- | --- | --- |
| VLAN 10 host cannot ping 10.0.18.1 | Host port is not in VLAN 10, SW01 Gi0/1 is not an access port in VLAN 10, or RT01 Gi0/1 is down. | Verify VLAN membership, cabling, Gi0/1 IP address, and no shutdown. |
| VLAN 20 host cannot ping 10.0.18.65 | Host is on the wrong access port, SW01 Fa0/24 is not in VLAN 20, or RT01 Gi0/2 is down. | Move the host to Fa0/5-Fa0/8, verify Fa0/24 access VLAN 20, and verify RT01 Gi0/2. |
| Hosts work on SW01 but not SW02 | Gi0/2 trunk is down or VLAN 10/20 is not allowed on one side. | Verify cable, trunk mode, and switchport trunk allowed vlan 10,20 on both switches. |
| VLAN 10 can reach Internet but VLAN 20 cannot | Internet router lacks the 10.0.18.64/26 return route, or RT01 Gi0/2 is misconfigured. | Add the VLAN 20 return route and verify Gi0/2 = 10.0.18.65/26. |
| Inter-VLAN ping fails but each host reaches its own gateway | One router LAN interface has the wrong subnet mask or IP address. | Verify Gi0/1 = 10.0.18.1/26 and Gi0/2 = 10.0.18.65/26. |
| show interfaces trunk lists a router-facing link | The router-facing switch port was incorrectly configured as a trunk. | Change the port to switchport mode access and assign the correct VLAN. |
| First ping loses one packet | ARP resolution or spanning-tree convergence. | Repeat the ping after a few seconds. |
| Router has only Gi0/0 and Gi0/1 | The selected router lacks enough physical interfaces for WAN plus two VLANs. | Use a router with at least three interfaces or choose a different inter-VLAN routing method. |

## Useful Diagnostic Commands

! Router show ip interface brief show ip route show running-config show arp ping <destination> traceroute <destination> ! Switch show vlan brief show interfaces trunk show interfaces status show interfaces switchport show mac address-table show cdp neighbors ! PC / Laptop ipconfig ping <destination> tracert <destination>

# Lab Completion Checklist

| Completion Item | Complete |
| --- | --- |
| VLAN 10 ADMIN_SERVICES exists on both switches. | [ ] |
| VLAN 20 PATRON_SERVICES exists on both switches. | [ ] |
| Fa0/1-Fa0/4 are access ports in VLAN 10. | [ ] |
| Fa0/5-Fa0/8 are access ports in VLAN 20. | [ ] |
| SW01 Gi0/2 and SW02 Gi0/2 form an 802.1Q trunk carrying VLANs 10 and 20. | [ ] |
| SW01 Gi0/1 is an access port in VLAN 10 connected to RT01 Gi0/1. | [ ] |
| SW01 Fa0/24 is an access port in VLAN 20 connected to RT01 Gi0/2. | [ ] |
| RT01 Gi0/1 is 10.0.18.1/26 and up/up. | [ ] |
| RT01 Gi0/2 is 10.0.18.65/26 and up/up. | [ ] |
| RT01 Gi0/0 is 216.0.5.2/30 and the default route points to 216.0.5.1. | [ ] |
| Internet has return routes to both /26 VLAN networks through 216.0.5.2. | [ ] |
| A VLAN 10 host can ping a VLAN 20 host. | [ ] |
| VLAN 10 and VLAN 20 hosts can ping 8.8.8.8. | [ ] |
| All configurations are saved. | [ ] |

## Configuration Evidence to Capture

Cafe01-SW01: show vlan brief and show interfaces trunk.

Cafe01-SW02: show vlan brief and show interfaces trunk.

Cafe01-RT01: show ip interface brief and show ip route.

Internet: show ip route.

PC1: ipconfig, ping 10.0.18.65, ping 8.8.8.8, and tracert 8.8.8.8.

# Student Knowledge Check

1. Why must VLAN 10 and VLAN 20 use different IPv4 subnets?

________________________________________________________________________________

________________________________________________________________________________

2. Why is Cafe01-RT01 Gi0/1 assigned 10.0.18.1/26 while Gi0/2 is assigned 10.0.18.65/26?

________________________________________________________________________________

________________________________________________________________________________

3. Why are the two SW01-to-router ports configured as access ports rather than trunks?

________________________________________________________________________________

________________________________________________________________________________

4. What does the SW01-to-SW02 trunk accomplish in this topology?

________________________________________________________________________________

________________________________________________________________________________

5. Why does Cafe01-RT01 not require static routes between VLAN 10 and VLAN 20?

________________________________________________________________________________

________________________________________________________________________________

6. What route must the Internet router use to return traffic to VLAN 20?

________________________________________________________________________________

________________________________________________________________________________

7. Why is this method less scalable than router-on-a-stick or multilayer-switch SVIs?

________________________________________________________________________________

________________________________________________________________________________

8. What happens if RT01 Gi0/2 is mistakenly configured with 10.0.18.1/26?

________________________________________________________________________________

________________________________________________________________________________

9. Which commands verify that the router-facing switch ports are access ports in the correct VLANs?

________________________________________________________________________________

________________________________________________________________________________

10. What additional control would prevent VLAN 20 patron devices from reaching VLAN 10 administrative devices?

________________________________________________________________________________

________________________________________________________________________________

# Student Reflection

Describe one configuration error you encountered while converting the flat LAN to the external-router, multiple-physical-interface design. Identify the command or connectivity test that revealed the error, explain why the error prevented traffic from being forwarded, and describe how you corrected it.

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

Final design summary VLAN 10 uses RT01 Gi0/1 at 10.0.18.1/26. VLAN 20 uses RT01 Gi0/2 at 10.0.18.65/26. The switch ports facing these router interfaces are access ports in their respective VLANs. Only the SW01-to-SW02 link is an 802.1Q trunk.